IEEE P802.11
Wireless LANs

| D0.1 Comment Resolution – PHY Service Specification |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: May 5, 2011 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Youhan Kim | Atheros |  |  | youhan.kim@atheros.com |

Abstract

This document provides resolution for the following CIDs on TGac D0.1 on PHY Service Specification clause:

CID: 1208, 249, 253, 188, 189, 190, 1375

Comments (page and line numbers) are based on D0.1.

Proposed edits are based on D0.4.

| CID | Subclause | Page | Line | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- |
| 1208 | 12.3.5.10.2 | 64 | 19 | TR | "Potential elements of this set are listed" - what other members of the set are you hiding from me? | Reword so it doesn't imply that the specification here is incomplete. ie. "Table 12-1 defines the members of this set.". |
| 249 | 12.3.5.10.2 | 64 | 36 | TR | "at least one of the 20 MHz channels in the secondary40 is busy" yet the CCA would also report busy on 40MHz ED or a 40MHz PPDU. Ditto secondary80 | a) report all cases (ugh) or b) refer to the CCA clause (easier!) |

Response:

AGREE.

Proposed Resolution Text:

- Semantics of the service primitive

When STATE is IDLE or when, for the type of PHY in operation, CCA is determined by a single channel, the channel-list parameter is absent. Otherwise, it carries a set indicating which channels are busy, represented by the values {primary}, {primary, secondary}, and {secondary}.  Table 7-ac1 defines the members of this set.(#1208)Channel-list parameter elements.

| - Channel-list parameter elements |  |
| --- | --- |
| channel-list elements | Meaning |
| primary | Indicates that the primary 20 MHz channel is busy |
| secondary | Indicates that the secondary 20 MHz channel is busy. The secondary 20 MHz channel is the 20 MHz channel adjacent to the primary 20 MHz channel, which together form a 40 MHz channel. |
| secondary40 | Indicates that the secondary 40 MHz channel is busy according to the rules specified in 22.3.20.5.2(#249). The secondary 40 MHz channel is the 40 MHz channel adjacent to the primary 40 MHz channel, which together form the primary 80 MHz channel. |
| secondary80 | Indicates that the secondary 80 MHz channel is busy according to the rules specified in 22.3.20.5.2(#249). The secondary 80 MHz channel is the 80 MHz channel that does not include the primary 20 MHz channel and which together with the primary 80 MHz channel forms the 160 MHz channel or the 80+80 MHz channel. |

| CID | Subclause | Page | Line | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- |
| 253 | 12.3.5.10.3 | 65 | 26 | TR | CCA for VHT is more complicated than described in this clause. Under some circumstances aCCATime is replaced by aCCAMidTime, there is energy detect, at P65L50 a constraint is missing that the SIG field be on the primary 20, there is no language for non-primary channels etc. Also, none of this is normative language. | a) report all cases (ugh) or b) for VHT STAs, refer to the CCA clause (easier!) |
| 188 | 12.3.5.10.3 | 65 | 35 | TR | SIG should be L-SIG | As in comment |
| 189 | 12.3.5.10.3 | 65 | 45 | TR | SIG should be L-SIG | As in comment |
| 190 | 12.3.5.10.3 | 65 | 50 | TR | SIG should be L-SIG | As in comment |
| 1375 | 12.3.5.10.3 | 65 | 50 | TR | "SIG field" --> "L-SIG" | Correct it |

Response - CID 253:

AGREE.

Furthermore, CCA operation is highly PHY dependent and is described in detail in the respective PHY clauses.  It is highly challenging to maintain the CCA operation description in multiple places.  Hence, it would be better to refer the readers to the respective PHY clauses on when the PHY-CCA.indication primitive is generated.

Response – CID 188, 189:

COUNTER.

The relevant text is now deleted.

Response – CID 190, 1375:

Duplicate of #747.

Proposed Resolution Text:

- When generated

This primitive is generated when the status of the channel(s) changes from channel idle to channel busy or from channel busy to channel idle, or when the channel-list parameter changes. Refer to specific PHY clauses for details about CCA behavior for a given PHY.

Straw Poll

Do you agree with comment resolution in 11-0641/r2?