IEEE P802.11
Wireless LANs

| Comment Resolution for MLME SAP interface |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-05-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Yongho Seok | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea | +82-31-450-1497 | yongho.seok@lge.com |

Abstract

Submission for Draft P802.11ac_D0.1 comment resolution. The document addresses various comments related to MLME SAP interface.

Submission addresses Draft P802.11ac_D0.1 comments 1307 and 1308.

Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGac Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGac Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGac Editor:  Editing instructions preceded by “TGac Editor” are instructions to the TGac editor to modify existing material in the TGac draft.   As a result of adopting the changes, the TGac editor will execute the instructions rather than copy them to the TGac Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

| CID | Commenter Name | Subclause | Page | Line | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1307 | Stephens, Adrian | 10.4 | 60 |  | TR | Where are the PLME interfaces to configure group membership? | Add PLME interfaces. |
| 1308 | Stephens, Adrian | 10.4 | 60 |  | TR | Where are the PLME interfaces to configure filtering by partial AID? | Add PLME interfaces. |

Discussion:

In order to configure group membership and Partial AID, the proposed resolution is to utilize PHY-CONFIG.request primitive with the additional PHYCONFIG_VECTOR. Also, we should need to define other MLME SAP interfaces, such as the group ID management protocol.

Proposed Resolution:

AGREE IN PRINCIPLE. Editing instructions below.

## Editing instructions:

6.3 MLME SAP Interface

6.3.7 Associate

6.3.7.2 MLME-ASSOCIATE.request

6.3.7.2.2 Semantics of the service primitive

TGac editor: Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-ASSOCIATE.request (

PeerSTAAddress,

AssociateFailureTimeout,

CapabilityInformation,

ListenInterval,

Supported Channels,

RSN,

QoSCapability,

Content of FT Authentication elements,

SupportedOperatingClasses,

HT Capabilities,

Extended Capabilities,

20/40 BSS Coexistence,

QoSTrafficCapability,

TIMBroadcastRequest,

EmergencyServices,

VHT Capabilities,

VendorSpecificInfo

)

TGac editor: Insert the following rows before VendorSpeificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| VHT Capabilities | As defined in VHT Capabilities element | As defined in 8.4.2.94 (VHT Capabilities element) | The VHT capabilities to be advertised for the BSS. The parameter is present if dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |

6.3.7.3 MLME-ASSOCIATE.confirm

6.3.7.3.2 Semantics of the service primitive

TGac editor: Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-ASSOCIATE.confirm(
				ResultCode,
				CapabilityInformation,
				AssociationID,
				SupportedRates,

EDCAParameterSet,
				RCPI.request,
				RSNI.request,
				RCPI.response,
				RSNI.response,
				RMEnabledCapabilities,
				Content of FT Authentication elements, 
				SupportedOperatingClasses,
				HT Capabilities,
				Extended Capabilities, 
				20/40 BSS Coexistence,

TimeoutInterval,
				BSSMaxIdlePeriod,
				TIMBroadcastResponse,

QosMapSet,

MFQ Policy,

VHT Capabilities,

VendorSpecificInfo

)

TGac editor: Insert the following rows before VendorSpeificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| VHT Capabilities | As defined in VHT Capabilities element | As defined in 8.4.2.94 (VHT Capabilities element) | The VHT capabilities to be advertised for the BSS. The parameter is present if dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |

6.3.7.4 MLME-ASSOCIATE.indication

6.3.7.4.2 Semantics of the service primitive

TGac editor: Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-ASSOCIATE.indication (

PeerSTAAddress,

CapabilityInformation,

ListenInterval,

SSID,

SupportedRates,

RSN,

QoSCapability,

RCPI,

RSNI,

RMEnabledCapabilities,

Content of FT Authentication elements,

SupportedOperatingClasses,

DSERegisteredLocation,

HT Capabilities,

Extended Capabilities,

20/40 BSS Coexistence,

QoSTrafficCapability,

TIMBroadcastRequest,

VHT Capabilities,

VendorSpecificInfo

)

TGac editor: Insert the following rows before VendorSpeificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| VHT Capabilities | As defined in VHT Capabilities element | As defined in 8.4.2.94 (VHT Capabilities element) | The VHT capabilities to be advertised for the BSS. The parameter is present if dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |

6.3.7.5 MLME-ASSOCIATE.response

6.3.7.5.2 Semantics of the service primitive

TGac editor: Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-ASSOCIATE.response(
				PeerSTAAddress,
				ResultCode,
				CapabilityInformation,
				AssociationID,
				EDCA Parameter Set,
				RCPI,
				RSNI,
				RMEnabledCapabilities,
				Content of FT Authentication Elements,
				SupportedOperatingClasses,
				DSERegisteredLocation,
				HTCapabilities,
				Extended Capabilities, 
				20/40 BSS Coexistence,

TimeoutInterval,
				BSSMaxIdlePeriod,
				TIMBroadcastResponse,
				QosMapSet,

MFQ Policy,
				VHT Capabilities,

VendorSpecificInfo

)

TGac editor: Insert the following rows before VendorSpeificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| VHT Capabilities | As defined in VHT Capabilities element | As defined in 8.4.2.94 (VHT Capabilities element) | The VHT capabilities to be advertised for the BSS. The parameter is present if dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |

6.3.8 Reassociate

6.3.8.2 MLME-REASSOCIATE.request

6.3.8.2.2 Semantics of the service primitive

TGac editor: Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-REASSOCIATE.request (

NewAPAddress,

ReassociateFailureTimeout,

CapabilityInformation,

ListenInterval,

Supported Channels,

RSN,

QoSCapability,

Content of FT Authentication elements,

SupportedOperatingClasses,

HT Capabilities,

Extended Capabilities,

20/40 BSS Coexistence,

QoSTrafficCapability,

TIMBroadcastRequest,

FMSRequest,

DMSRequest,

EmergencyServices,

VHT Capabilities,

VendorSpecificInfo

)

TGac editor: Insert the following rows before VendorSpeificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| VHT Capabilities | As defined in VHT Capabilities element | As defined in 8.4.2.94 (VHT Capabilities element) | The VHT capabilities to be advertised for the BSS. The parameter is present if dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |

6.3.8.3 MLME-REASSOCIATE.confirm

6.3.8.3.2 Semantics of the service primitive

TGac editor: Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-REASSOCIATE.confirm(

ResultCode,

CapabilityInformation,

AssoicationID,

SupportedRates,

EDCAParameterSet,

RCPI.request,

RSNI.request,

RCPI.response,

RSNI.response,

RMEnabledCapabilities,

Content of FT Authentication Elements,

SupportedOperatingClasses,

HT Capabilities,

Extended Capabilities,

20/40 BSS Coexistence,

TimeoutInterval,

BSSMaxIdlePeriod,

TIMBroadcastResponse,

FMSResponse,

DMSResponse,

QoSMapSet,

MFQ Policy,

VHT Capabilities,

VendorSpecificInfo

)

TGac editor: Insert the following rows before VendorSpeificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| VHT Capabilities | As defined in VHT Capabilities element | As defined in 8.4.2.94 (VHT Capabilities element) | The VHT capabilities to be advertised for the BSS. The parameter is present if dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |

6.3.8.4 MLME-REASSOCIATE.indication

6.3.8.4.2 Semantics of the service primitive

TGac editor: Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-REASSOCIATE.indication (

PeerSTAAddress,

CurrentAPAddress,

CapabilityInformation,

ListenInterval,

SSID,

SupportedRates,

RSN,

QoSCapability,

RCPI,

RSNI,

RMEnabledCapabilities,

Content of FT Authentication elements,

SupportedOperatingClasses,

DSERegisteredLocation,

HT Capabilities,

Extended Capabilities,

20/40 BSS Coexistence,

QoSTrafficCapability,

TIMBroadcastRequest,

FMSRequest,

DMSRequest,

VHT Capabilities,

VendorSpecificInfo

)

TGac editor: Insert the following rows before VendorSpeificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| VHT Capabilities | As defined in VHT Capabilities element | As defined in 8.4.2.94 (VHT Capabilities element) | The VHT capabilities to be advertised for the BSS. The parameter is present if dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |

6.3.8.5 MLME-REASSOCIATE.response

6.3.8.5.2 Semantics of the service primitive

TGac editor: Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-REASSOCIATE.response(

PeerSTAAddress,

ResultCode,

CapabilityInformation,

AssociationID,

EDCAParameterSet,

RCPI,

RSNI,

RMEnabledCapabilities,

Content of FT Authentication Information elements,

SupportedOperatingClasses,

DSERegisteredLocation,

HT Capabilities,

Extended Capabilities,

20/40 BSS Coexistence,

TimeoutInterval,

BSSMaxIdlePeriod,

TIMBroadcastResponse,

FMSResponse,

DMSResponse,

QoSMapSet,

MFQ Policy,

VHT Capabilities,

VendorSpecificInfo

)

TGac editor: Insert the following rows before VendorSpeificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| VHT Capabilities | As defined in VHT Capabilities element | As defined in 8.4.2.94 (VHT Capabilities element) | The VHT capabilities to be advertised for the BSS. The parameter is present if dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |

6.3.11 Start

6.3.11.2 MLME-START.request

6.3.11.2.2 Semantics of the service primitive

TGac editor: Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-START.request(

SSID,

BSSType,

BeaconPeriod,

DTIMPeriod,

CF parameter set,

PHY parameter set,

IBSS parameter set,

ProbeDelay,

CapabilityInformation,

BSSBasicRateSet,

OperationalRateSet,

Country,

IBSS DFS Recovery Interval,

EDCAParameterSet,

DSERegisteredLocation,

HT Capabilities,

HT Operation,

BSSMembershipSelectorSet,

BSSBasicMCSSet,

HTOperationalMCSSet,

Extended Capabilities,

20/40 BSS Coexistence,

Overlapping BSS Scan Parameters,

MultipleBSSID,

InterworkingInfo,

AdvertisementProtocolInfo,

RoamingConsortiumInfo,

Mesh ID,

Mesh Configuration,

MFQ Policy,

VHT Capabilities,

VHT Operation,

VHT Transmit Power Envelope,

Extended Power Constraint,

Extended BSS Load,

VendorSpecificInfo

)

TGac editor: Insert the following rows before VendorSpeificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| VHT Capabilities | As defined in VHT Capabilities element | As defined in 8.4.2.94 (VHT Capabilities element) | The VHT capabilities to be advertised for the BSS. The parameter is present if dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |
| VHT Operation | As defined in VHT Operation element | As defined in 8.4.2.95 (VHT Operation element) | The additional VHT capabilities to be advertised for the BSS. The parameter is present if BSSType = INFRASTRUCTURE and dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |
| VHT Transmit Power Envelope | As defined in VHT Transmit Power Envelope element | As defined in 8.4.2.98 (VHT Transmit Power Envelop element) | Specifies the parameters within the VHT Transmit Power Envelope element that are indicated by the MAC entity. The VHT Transmit Power Envelope element is present if dot11SpectrumManagementRequired is true or dot11RadioMeasurementActivated is true. |
| Extended Power Constraint | As defined in Extended Power Constraint element | As defined in 8.4.2.99 (Extended Power Constraint element) | Specifies the parameters within the Extended Power Constraint that are indicated by the MAC entity. The Extended Power Constraint element is present if dot11VHTOptionImplemented attribute is true, the STA Channel Width subfield of the VHT Operation element is set to indicate a channel width of 80 MHz or wider, and dot11SpectrumManagementRequired is true. |
| Extended BSS Load | As defined in Extended BSS Load element | As defined in 8.4.2.96 (Extended BSS Load element) | Specifies the parameters within the Extended BSS Load element that are indicated by the MAC entity. The Extended BSS Load element is present if dot11QosOptionImplemented and dot11QBSSLoadImplemented and dot11VHTOptionImplemented are true. |

TGac editor: Insert a new subclause at the end of clause 6.3 as follows:

6.3.ac1 Group ID Management

The following MLME primitives support the signaling of the group ID management. Figure 6-ac1 depicts the group ID management process. The figure illustrates the basic protocol, which is only an example and is not meant to be exhaustive of all possible protocol uses.

Figure 6-ac1—Group ID Management Protocol Exchange

6.3.ac1.1 MLME-GIDMGMT.request

6.3.ac1.1.1 Function

This primitive requests that a Group ID Management frame be sent to a peer entity.

6.3.ac1.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-GIDMGMT.request(

PeerSTAAddress,
MembershipStatusArrary,
STAPositionArrary

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the Group ID Management process. |
| MembershipStatusArrary | Integer | As defined in 8.5.16.3 (Group ID Management frame) | Specifies Membership Status for each of the 64 Group IDs. |
| STAPositionArrary | Integer | As defined in 8.5.16.3 (Group ID Management frame) | Specifies STA position for each of the 64 Group IDs. |

6.3.ac1.1.3 When Generated

This primitive is generated by the SME to request that a Group ID Management frame be sent to the peer entity.

6.3.ac1.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Group ID Management frame. The STA then attempts to transmit this to the peer entity.

6.3.ac1.2 MLME-GIDMGMT.confirm

6.3.ac1.2.1 Function

This primitive reports the result of the Group ID Management.

6.3.ac1.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-GIDMGMT.confirm(

ResultCode

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, TRANSMISSION FAILURE, INVALID PARAMETERS, OR UNSPECIFIED FAILURE | Report the outcome of a request to send a Group ID Management frame. |

6.3.ac1.2.3 When Generated

This primitive is generated by the MLME when transmission of the Group ID Management frame is acknowledged, (re)transmission of the Group ID Management frame fails, the Group ID Management frame contains invalid parameters, or for unspecified failure reasons.

6.3.ac1.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the result code.

6.3.ac1.3 MLME-GIDMGMT.indication

6.3.ac1.3.1 Function

This primitive indicates that a Group ID Management frame was received from a peer entity.

6.3.ac1.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-GIDMGMT.indication(

PeerSTAAddress,
MembershipStatusArrary,
STAPositionArrary

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity from which a Group ID Management frame was received. |
| MembershipStatusArrary | Integer | As defined in 8.5.16.3 (Group ID Management frame) | Specifies Membership Status for each of the 64 Group IDs. |
| STAPositionArrary | Integer | As defined in 8.5.16.3 (Group ID Management frame) | Specifies STA position for each of the 64 Group IDs. |

6.3.ac1.3.3 When Generated

This primitive is generated by the MLME when a valid Group ID Management frame is received.

6.3.ac1.3.4 Effect of Receipt

On receipt of this primitive, the SME should operate according to the procedure in 22.3.12.3.

6.3.ac2 VHT Operating Mode Notification

The following MLME primitives support the signaling of the VHT Operating Mode Notification. Figure 6-ac2 depicts the VHT Operating Mode Notification process. The figure illustrates the basic protocol, which is only an example and is not meant to be exhaustive of all possible protocol uses.

Figure 6-ac2—VHT Operating Mode Notification Protocol Exchange

6.3.ac2.1 MLME-VHTOPERATINGMODE.request

6.3.ac2.1.1 Function

This primitive requests that a VHT Operating Mode Notification frame be sent to a peer entity.

6.3.ac2.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-VHTOPERATINGMODE.request(

PeerSTAAddress,
VHTOPERATINGMODE

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the VHT Operating Mode Notification process. |
| VHTOPERATINGMODE | VHT Operating Mode field | As defined in 8.4.1.36 (VHT Operating Mode field) | Specifies the proposed service parameters for the VHT Operaing Mode Nofication. |

6.3.ac2.1.3 When Generated

This primitive is generated by the SME to request that a VHT Operating Mode Notification frame be sent to the peer entity.

6.3.ac2.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a VHT Operating Mode Notification frame. The STA then attempts to transmit this to the peer entity.

6.3.ac2.2 MLME-VHTOPERATINGMODE.confirm

6.3.ac2.2.1 Function

This primitive reports the result of the VHT Operating Mode Notification.

6.3.ac2.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME- VHTOPERATINGMODE.confirm(

ResultCode

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, TRANSMISSION FAILURE, INVALID PARAMETERS, OR UNSPECIFIED FAILURE | Report the outcome of a request to send a VHT Operating Mode Notification frame. |

6.3.ac2.2.3 When Generated

This primitive is generated by the MLME when transmission of the VHT Operating Mode Notification frame is acknowledged, (re)transmission of the VHT Operating Mode Notification frame fails, the VHT Operating Mode Notification frame contains invalid parameters, or for unspecified failure reasons.

6.3.ac2.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the result code.

6.3.ac2.3 MLME-VHTOPERATINGMODE.indication

6.3.ac2.3.1 Function

This primitive indicates that a VHT Operating Mode Notification frame was received from a peer entity.

6.3.ac2.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME- VHTOPERATINGMODE.indication(

PeerSTAAddress,
VHTOPERATINGMODE

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity from which a Group ID Management frame was received. |
| VHTOPERATINGMODE | VHT Operating Mode field | As defined in 8.4.1.36 (VHT Operating Mode field) | Specifies the proposed service parameters for the VHT Operaing Mode Nofication. |

6.3.ac2.3.3 When Generated

This primitive is generated by the MLME when a valid VHT Operating Mode Notification frame is received.

6.3.ac2.3.4 Effect of Receipt

On receipt of this primitive, the SME should operate according to the procedure in 10.24.1.

7.3.4.5 Vector descriptions

TGac editor: Insert the following rows in Table 7-4:

Table 7-4 —Vector description

| Parameter | Associate vector | Value |
| --- | --- | --- |
| GROUP_ID | PHYCONFIG_VECTOR | The GROUP_ID parameter specifies membership status and STA position for each of the 64 Group IDs. The length of the field is 24 octets. (As defined in 8.5.16.3) |
| PARTIAL_AID | PHYCONFIG_VECTOR | The PARTIAL_AID specifies an abbreviated indication as the intended recipient of the frame (As defined in 9.17a). |

22.3.12.3 Group ID

TGac editor: Insert the following paragraph after the last paragraph in section 22.3.12.3:

The STA assign or change STA positions corresponding to one or more Group IDs through the PHY-CONFIG.request primitive specifying a PHYCONFIG_VECTOR parameter GROUP_ID that indicates    membership status and STA position.