IEEE P802.11
Wireless LANs

| Comment Resolution for MLME SAP interface |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-05-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Yongho Seok | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea | +82-31-450-1497 | yongho.seok@lge.com |

Abstract

Submission for Draft P802.11ac_D0.1 comment resolution. The document addresses various comments related to MLME SAP interface.

Submission addresses Draft P802.11ac_D0.1 comments 1307 and 1308.

Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGac Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGac Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGac Editor:  Editing instructions preceded by “TGac Editor” are instructions to the TGac editor to modify existing material in the TGac draft.   As a result of adopting the changes, the TGac editor will execute the instructions rather than copy them to the TGac Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

| CID | Commenter Name | Subclause | Page | Line | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1307 | Stephens, Adrian | 10.4 | 60 |  | TR | Where are the PLME interfaces to configure group membership? | Add PLME interfaces. |
| 1308 | Stephens, Adrian | 10.4 | 60 |  | TR | Where are the PLME interfaces to configure filtering by partial AID? | Add PLME interfaces. |

Discussion:

In order to configure group membership and Partial AID, the proposed resolution is to utilize PHY-CONFIG.request primitive with the additional PHYCONFIG_VECTOR. Also, we should need to define other MLME SAP interfaces, such as the group ID management protocol.

Proposed Resolution:

AGREE IN PRINCIPLE. Editing instructions below.

## Editing instructions:

6.3 MLME SAP Interface

6.3.7 Associate

6.3.7.2 MLME-ASSOCIATE.request

6.3.7.2.2 Semantics of the service primitive

TGac editor: Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-ASSOCIATE.request (

PeerSTAAddress,

AssociateFailureTimeout,

CapabilityInformation,

ListenInterval,

Supported Channels,

RSN,

QoSCapability,

Content of FT Authentication elements,

SupportedOperatingClasses,

HT Capabilities,

Extended Capabilities,

20/40 BSS Coexistence,

QoSTrafficCapability,

TIMBroadcastRequest,

EmergencyServices,

VHT Capabilities,

VendorSpecificInfo

)

TGac editor: Insert the following rows before VendorSpeificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| VHT Capabilities | As defined in VHT Capabilities element | As defined in 8.4.2.94 (VHT Capabilities element) | The VHT capabilities to be advertised for the BSS. The parameter is present if dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |

6.3.7.3 MLME-ASSOCIATE.confirm

6.3.7.3.2 Semantics of the service primitive

TGac editor: Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-ASSOCIATE.confirm(
				ResultCode,
				CapabilityInformation,
				AssociationID,
				SupportedRates,

EDCAParameterSet,
				RCPI.request,
				RSNI.request,
				RCPI.response,
				RSNI.response,
				RMEnabledCapabilities,
				Content of FT Authentication elements, 
				SupportedOperatingClasses,
				HT Capabilities,
				Extended Capabilities, 
				20/40 BSS Coexistence,

TimeoutInterval,
				BSSMaxIdlePeriod,
				TIMBroadcastResponse,

QosMapSet,

MFQ Policy,

VHT Capabilities,

VendorSpecificInfo

)

TGac editor: Insert the following rows before VendorSpeificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| VHT Capabilities | As defined in VHT Capabilities element | As defined in 8.4.2.94 (VHT Capabilities element) | The VHT capabilities to be advertised for the BSS. The parameter is present if dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |

6.3.7.4 MLME-ASSOCIATE.indication

6.3.7.4.2 Semantics of the service primitive

TGac editor: Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-ASSOCIATE.indication (

PeerSTAAddress,

CapabilityInformation,

ListenInterval,

SSID,

SupportedRates,

RSN,

QoSCapability,

RCPI,

RSNI,

RMEnabledCapabilities,

Content of FT Authentication elements,

SupportedOperatingClasses,

DSERegisteredLocation,

HT Capabilities,

Extended Capabilities,

20/40 BSS Coexistence,

QoSTrafficCapability,

TIMBroadcastRequest,

VHT Capabilities,

VendorSpecificInfo

)

TGac editor: Insert the following rows before VendorSpeificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| VHT Capabilities | As defined in VHT Capabilities element | As defined in 8.4.2.94 (VHT Capabilities element) | The VHT capabilities to be advertised for the BSS. The parameter is present if dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |

6.3.7.5 MLME-ASSOCIATE.response

6.3.7.5.2 Semantics of the service primitive

TGac editor: Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-ASSOCIATE.response(
				PeerSTAAddress,
				ResultCode,
				CapabilityInformation,
				AssociationID,
				EDCA Parameter Set,
				RCPI,
				RSNI,
				RMEnabledCapabilities,
				Content of FT Authentication Elements,
				SupportedOperatingClasses,
				DSERegisteredLocation,
				HTCapabilities,
				Extended Capabilities, 
				20/40 BSS Coexistence,

TimeoutInterval,
				BSSMaxIdlePeriod,
				TIMBroadcastResponse,
				QosMapSet,

MFQ Policy,
				VHT Capabilities,

VendorSpecificInfo

)

TGac editor: Insert the following rows before VendorSpeificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| VHT Capabilities | As defined in VHT Capabilities element | As defined in 8.4.2.94 (VHT Capabilities element) | The VHT capabilities to be advertised for the BSS. The parameter is present if dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |

6.3.8 Reassociate

6.3.8.2 MLME-REASSOCIATE.request

6.3.8.2.2 Semantics of the service primitive

TGac editor: Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-REASSOCIATE.request (

NewAPAddress,

ReassociateFailureTimeout,

CapabilityInformation,

ListenInterval,

Supported Channels,

RSN,

QoSCapability,

Content of FT Authentication elements,

SupportedOperatingClasses,

HT Capabilities,

Extended Capabilities,

20/40 BSS Coexistence,

QoSTrafficCapability,

TIMBroadcastRequest,

FMSRequest,

DMSRequest,

EmergencyServices,

VHT Capabilities,

VendorSpecificInfo

)

TGac editor: Insert the following rows before VendorSpeificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| VHT Capabilities | As defined in VHT Capabilities element | As defined in 8.4.2.94 (VHT Capabilities element) | The VHT capabilities to be advertised for the BSS. The parameter is present if dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |

6.3.8.3 MLME-REASSOCIATE.confirm

6.3.8.3.2 Semantics of the service primitive

TGac editor: Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-REASSOCIATE.confirm(

ResultCode,

CapabilityInformation,

AssoicationID,

SupportedRates,

EDCAParameterSet,

RCPI.request,

RSNI.request,

RCPI.response,

RSNI.response,

RMEnabledCapabilities,

Content of FT Authentication Elements,

SupportedOperatingClasses,

HT Capabilities,

Extended Capabilities,

20/40 BSS Coexistence,

TimeoutInterval,

BSSMaxIdlePeriod,

TIMBroadcastResponse,

FMSResponse,

DMSResponse,

QoSMapSet,

MFQ Policy,

VHT Capabilities,

VendorSpecificInfo

)

TGac editor: Insert the following rows before VendorSpeificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| VHT Capabilities | As defined in VHT Capabilities element | As defined in 8.4.2.94 (VHT Capabilities element) | The VHT capabilities to be advertised for the BSS. The parameter is present if dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |

6.3.8.4 MLME-REASSOCIATE.indication

6.3.8.4.2 Semantics of the service primitive

TGac editor: Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-REASSOCIATE.indication (

PeerSTAAddress,

CurrentAPAddress,

CapabilityInformation,

ListenInterval,

SSID,

SupportedRates,

RSN,

QoSCapability,

RCPI,

RSNI,

RMEnabledCapabilities,

Content of FT Authentication elements,

SupportedOperatingClasses,

DSERegisteredLocation,

HT Capabilities,

Extended Capabilities,

20/40 BSS Coexistence,

QoSTrafficCapability,

TIMBroadcastRequest,

FMSRequest,

DMSRequest,

VHT Capabilities,

VendorSpecificInfo

)

TGac editor: Insert the following rows before VendorSpeificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| VHT Capabilities | As defined in VHT Capabilities element | As defined in 8.4.2.94 (VHT Capabilities element) | The VHT capabilities to be advertised for the BSS. The parameter is present if dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |

6.3.8.5 MLME-REASSOCIATE.response

6.3.8.5.2 Semantics of the service primitive

TGac editor: Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-REASSOCIATE.response(

PeerSTAAddress,

ResultCode,

CapabilityInformation,

AssociationID,

EDCAParameterSet,

RCPI,

RSNI,

RMEnabledCapabilities,

Content of FT Authentication Information elements,

SupportedOperatingClasses,

DSERegisteredLocation,

HT Capabilities,

Extended Capabilities,

20/40 BSS Coexistence,

TimeoutInterval,

BSSMaxIdlePeriod,

TIMBroadcastResponse,

FMSResponse,

DMSResponse,

QoSMapSet,

MFQ Policy,

VHT Capabilities,

VendorSpecificInfo

)

TGac editor: Insert the following rows before VendorSpeificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| VHT Capabilities | As defined in VHT Capabilities element | As defined in 8.4.2.94 (VHT Capabilities element) | The VHT capabilities to be advertised for the BSS. The parameter is present if dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |

6.3.11 Start

6.3.11.2 MLME-START.request

6.3.11.2.2 Semantics of the service primitive

TGac editor: Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-START.request(

SSID,

BSSType,

BeaconPeriod,

DTIMPeriod,

CF parameter set,

PHY parameter set,

IBSS parameter set,

ProbeDelay,

CapabilityInformation,

BSSBasicRateSet,

OperationalRateSet,

Country,

IBSS DFS Recovery Interval,

EDCAParameterSet,

DSERegisteredLocation,

HT Capabilities,

HT Operation,

BSSMembershipSelectorSet,

BSSBasicMCSSet,

HTOperationalMCSSet,

Extended Capabilities,

20/40 BSS Coexistence,

Overlapping BSS Scan Parameters,

MultipleBSSID,

InterworkingInfo,

AdvertisementProtocolInfo,

RoamingConsortiumInfo,

Mesh ID,

Mesh Configuration,

MFQ Policy,

VHT Capabilities,

VHT Operation,

VHT Transmit Power Envelope,

Extended Power Constraint,

Extended BSS Load,

VendorSpecificInfo

)

TGac editor: Insert the following rows before VendorSpeificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| VHT Capabilities | As defined in VHT Capabilities element | As defined in 8.4.2.94 (VHT Capabilities element) | The VHT capabilities to be advertised for the BSS. The parameter is present if dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |
| VHT Operation | As defined in VHT Operation element | As defined in 8.4.2.95 (VHT Operation element) | The additional VHT capabilities to be advertised for the BSS. The parameter is present if BSSType = INFRASTRUCTURE and dot11VHTOptionImplemented is true; otherwise, this parameter is not present. |
| VHT Transmit Power Envelope | As defined in VHT Transmit Power Envelope element | As defined in 8.4.2.98 (VHT Transmit Power Envelop element) | Specifies the parameters within the VHT Transmit Power Envelope element that are indicated by the MAC entity. The VHT Transmit Power Envelope element is present if dot11SpectrumManagementRequired is true or dot11RadioMeasurementActivated is true. |
| Extended Power Constraint | As defined in Extended Power Constraint element | As defined in 8.4.2.99 (Extended Power Constraint element) | Specifies the parameters within the Extended Power Constraint that are indicated by the MAC entity. The Extended Power Constraint element is present if dot11VHTOptionImplemented attribute is true, the STA Channel Width subfield of the VHT Operation element is set to indicate a channel width of 80 MHz or wider, and dot11SpectrumManagementRequired is true. |
| Extended BSS Load | As defined in Extended BSS Load element | As defined in 8.4.2.96 (Extended BSS Load element) | Specifies the parameters within the Extended BSS Load element that are indicated by the MAC entity. The Extended BSS Load element is present if dot11QosOptionImplemented and dot11QBSSLoadImplemented and dot11VHTOptionImplemented are true. |

7.3.4.5 Vector descriptions

TGac editor: Insert the following rows in Table 7-4:

Table 7-4 —Vector description

| Parameter | Associate vector | Value |
| --- | --- | --- |
| GROUP_ID | PHYCONFIG_VECTOR | The GROUP_ID parameter specifies membership status and STA position for each of the 64 Group IDs. The length of the field is 24 octets. (As defined in 8.5.16.3) |
| PARTIAL_AID | PHYCONFIG_VECTOR | The PARTIAL_AID specifies an abbreviated indication as the intended recipient of the frame (As defined in 9.17a). |

22.3.12.3 Group ID

TGac editor: Insert the following paragraph after the last paragraph in section 22.3.12.3:

The STA assign or change STA positions corresponding to one or more Group IDs through the PHY-CONFIG.request primitive specifying a PHYCONFIG_VECTOR parameter GROUP_ID that indicates    membership status and STA position.