IEEE P802.11
Wireless LANs

| D0.1 Comment Resolution – Stream Parser |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: May 5, 2011 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Youhan Kim | Atheros |  |  | youhan.kim@atheros.com |

Abstract

This document provides resolution for the following CIDs on TGac D0.1 on Stream Parser:

CID: 426, 427, 428

Comments (page and line numbers) are based on D0.1.

Proposed edits are based on D0.4.

| CID | Subclause | Page | Line | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- |
| 426 | 22.3.11.5 | 120 | 50 | TR | This paragraph is a good introduction but is not as precise as math | Define these important special cases via math. And normative language. As well, in the same way that the math for a deinterleaver is defined, define the math for a stream deparser |
| 427 | 22.3.11.5 | 120 | 52 | TR | "until the last interger block (floor()) of S bits" | for floor() integer blocks of S bits |
| 428 | 22.3.11.5 | 120 | 53 | TR | Sentence starting "Assuming that" needs a rewrite for clarity | Start with "At this stage, for the current OFDM symbol, each BCC encoder has Ms (where M = [insert equation] and M<s) residue bits …" and keep improving |

Response:

AGREE IN PRINCIPLE.

Proposed Resolution Text:

#### 22.3.11.6 Stream Parser

After coding and puncturing, the data bit streams at the output of the FEC encoders are re-arranged into  blocks of  bits. This operation is referred to as “stream parsing” and is described in this section.

The number of bits assigned to a single axis (real or imaginary) in a constellation point in a spatial stream is denoted by Equation (22-55).

(22-55)

The sum of these over all streams is:

Consecutive blocks of  bits are assigned to different spatial streams in a round robin fashion.

Let

(22-56)

and

(22-57)

For the first  bits of each OFDM symbol,  bits from the output of first encoder are fed into all spatial streams, and then  bits from the output of next encoder are used, and so on.  If  is greater than , then for the last  bits of each OFDM symbol,  bits from the output of first encoder are fed into spatial streams 1 through  ( bits per stream), and then  bits from the output of next encoder are used for spatial stream  through , and so on, where  is the remainder resulting from the division of interger  by integer .

The following equations are an equivalent description of the above procedure.  Bit  at(Ed) the output of encoder  is assigned to input bit  of spatial stream  where(#1654)

(22-58)

and

(#1660)

(22-59)

where

is equal to

is equal to

is the largest integer less than or equal to

is the remainder resulting from the division of integer  by integer