IEEE P802.11
Wireless LANs

| D0.1 Comment Resolution – Stream Parser |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: May 9, 2011 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Youhan Kim | Atheros |  |  | youhan.kim@atheros.com |

Abstract

This document provides resolution for the following CIDs on TGac D0.1:

CID: 350

Comments (page and line numbers) are based on D0.1.

Proposed edits are based on D0.4.

| CID | Subclause | Page | Line | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- |
| 350 | 22.3.7 | 90 | 61 | TR | To refer to (22-2), should be r_{PPDU}. From fig 22-7, shouldn't sum over segments - delete summation, and keep i_{seg} as a superscript of r_RF. Also, r_PPDU depends on iTx, so add iTx as a supersript to r_RF and r_{PPDU} | As in comment |

Response:

AGREE.

NOTE – This comments is on P91L20 (Equation (22-1)) of D0.1.

Proposed Resolution Text:

#### 22.3.7 Mathematical description of signals

The transmitted signal is described in complex baseband signal notation.  The actual transmitted signal is related to the complex baseband signal by the relation shown in Equation .

where

Re	represents the real part of a complex variable;

NSeg	represents the number of frequency segments the transmit signal(#900) as defined in Table 22-4 (Timing-related constants)(#351).  NSeg=1 for contiguous transmission and NSeg=2for non-contiguous transmission(Ed)

riSegt	represents the complex baseband signal of frequency segment iSeg in transmit antenna ;

fciSeg	represents the carrier center frequency of frequency segment iSeg.

NOTE – Transmitted signals may have different phase and/or phase noise between the two frequency segments, which is not shown in Equation (22-1) for simplicity.