IEEE P802.11
Wireless LANs

| Management frames exempt from QMF service |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-09-22 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Harish Ramamurthy | Marvell Semiconductor, Inc. | 5488 Marvell Lane, Santa Clara, CA-95054 |  | harishr@marvell.com |
| Yong Liu | Marvell |  |  | yongliu@marvell.com |

### Overview

## REV HISTORY:

R0 – initial revision

R1 – Use SNE management frame table

R2 – Align with 11mb terminology and 11ae text

R3 – Add background discussion

Discussion

802.11mb (D10.1) defined time priority management frame as following:

Time priority management frame: a management(#14137) frame that is transmitted outside of the normal MAC queuing process.(#13022)

Please modify Section 8.2.4.4.2 as follows:

8.2.4.4.2 Sequence Number field

A management frame is an IQMF frame when all fourfive of the following conditions are met:

1. The RA of the management frame corresponds to an individual MAC address.

2. The frame is transmitted by a QMF STA.

3. The sending STA has previously received an Extended Capabilities element from the STA corresponding to the RA of the frame to be transmitted.

4. The most recently received such Extended Capabilities element indicated that the STA is a QoS STA and has the value of one in the Q MFActivated subfield.

5. The frame is not a time priority management frame.

A management frame is a G QMF frame when all fourfive of the following conditions are met:

1.The RA of the management frame corresponds to a group MAC address.

2.The frame is transmitted by a QMF STA.

3.The transmitting STA has received an Extended Capabilities element that has the QMFActivated subfield equal to one from every member of the group address that is also a member of the BSS corresponding to the BSSID of the management frame.

4.The BSSID of the management frame is not the wildcard BSSID.

5. The frame is not a time priority management frame.

Please add following text to the end of 10.25.1

10.25.1 General

…

Time-priority management frames, when not sent as an immediate response, shall be transmitted by using AC_VO.