IEEE P802.11
Wireless LANs

| TXTIME and PSDU_LENGTH Comment Resolution for LB 178 D1.0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 20 July 2011 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Eldad Perahia | Intel Corporation |  |  | eldad.perahia@intel.com |

Abstract

This document provides resolutions for CIDs 3273, 2712, 2976, 3692, 3724, 3693, 2714, 3694

| CID | Page | Clause | Comment | Proposed Change | Resn Status | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 3273 | 199.05 | 22.4.3 | The equation uses the term "LENGTH", but the description of each of the terms refers to "LENGTHu". | Either add the subscript "u" to the term in the equation 22-96 or else drop the subscript in the explanation of that term below the equation. | P | Agree in Principle. See resolution in 11/YYYY |
| 2712 | 199.11 | 22.4.3 | User index is not needed for SU packets. | Change to 'LENGTH is given by the TXVECTOR'. Also, is it clear that LENGTH in TXVECTOR, which has type MU, is also present for SU packets as well? | P | Agree in Principle. See resolution in 11/YYYY. In addition, see resolution to CID 2691 in 11/0954. |
| 2976 | 199.11 | 22.4.3 | Since LENGTHu does not appear in (22-96), it is unnessary to explain it. | Remove this sentence. | P | Agree in Principle. See resolution in 11/YYYY |
| 3692 | 199.11 | 22.4.3 | For SU, the user index isn't needed in "LENGTH_u is passed as the LENGTH parameter for user u in the TXVECTOR." | Change to "LENGTH is passed as the LENGTH parameter in the TXVECTOR" and update the TXVECTOR to include LENGTH also for the SU case. | P | Agree in Principle. See resolution in 11/YYYY |
| 3724 | 199.11 | 22.4.3 | Should Lengthu be length? | Equation 22-96 is SU packet only | P | Agree in Principle. See resolution in 11/YYYY |

Discussion:

LENGTHu does not apply to SU.  However later at P199L27, we reference Eq 22-54 for MU, which is just a summation for N_SYM,u.  But there is no equation for N_SYM,u.  Furthermore, in the TXVECTOR it is not clear whether the parameters with “MU” are valid for SU.  In reality, the only parameter that is not valid for SU is USER_POSITION.  To resolve that, we’ll modify the notation in the TXVECTOR to include “MU-ONLY” and change “MU” to cover both MU and SU, as given by resolution to CID 2691 in 11/0954.

TGac editor: modify D1.0 P199L11, as follows

LENGTHu is passed as the LENGTH parameter for user u in the TXVECTOR.

TGac editor: modify D1.0 P199L27, as follows

For a MU packet, the total number of data symbols in the Data field, NSYM , is given by Equation (22-54) and NSYM,u given by Equation (22-YY).

(22-YY)

where

is passed as the LENGTH parameter for user u in the TXVECTOR.

is equal to 2 when STBC is used, and 1 otherwise

is defined in Table 22-5

is defined in Table 22-5

and are defined in Table 22-4

TGac editor: modify D1.0 Table 22-5, as follows

| Symbol | Explanation |
| --- | --- |
| … |  |
|  | Number of data bits per symbol for user u, u=0, 1, 2, 3. For SU packets, For MU packets, is undefined |

TGac editor: modify D1.0 Table 22-1, as follows

| Parameter | Condition | Value | TXVECTOR | RXVECTOR |
| --- | --- | --- | --- | --- |
| … |  |  |  |  |
| USER_POSITION | FORMAT is VHT | Index for user in MU transmission. Integer: range 0-3 | MU-ONLY | O |
|  | Otherwise | Not present | N | N |
| … |  |  |  |  |
| NOTE 1—In the “TXVECTOR” and “RXVECTOR” columns, the following apply: Y = Present; N = Not present; O = Optional; MU indicates that the parameter is present per user. Parameters specified to be present per user are conceptually supplied as an array of values indexed by u, where u takes values 1 through NUM_USERS-1. MU-ONLY indicates that the parameter is present per user. Parameters specified to be present per user are conceptually supplied as an array of values indexed by u, where u takes values 1 through NUM_USERS-1. For SU, the parameter is not present . NOTE 2—On reception, where valid, the CH_BANDWIDTH_IN_NON_HT parameter is likely to be a more reliable indication of format and channel width than the NON_HT_DUP_OFDM and CH_BANDWIDTH parameters. |  |  |  |  |

| CID | Page | Clause | Comment | Proposed Change | Resn Status | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 3693 | 199.52 | 22.4.3 | It is a bit vague to refer to TX-side computations for the RXVECTOR. | Instead, use a similar approach as described in Section 22.3.21 PLCP receive procedure, equations 22-92 and 22-93 and corresponding text | P | Agree in Principle. See resolution in 11/YYYY |
| 2714 | 199.60 | 22.4.3 | Equation (22-49) is for TX. For PSDU_LENGTH computation for RXVECTOR, N_sym,init should be based on Equation (22-93). Same comment for P200L20. | Change to 'N_sym,init is givne by Equation (22-49) when computing PSDU_LENGTH for PLME-TIME.confirm primitive, and Equation (22-93) when computing PSDU_LENGTH for RXVECTOR'. Similar change for P200L20. | P | Agree in Principle. See resolution in 11/YYYY |
| 3694 | 200.02 | 22.4.3 | It is a bit vague to refer to TX-side computations for the RXVECTOR. | Instead, use a similar approach as described in Section 22.3.21 PLCP receive procedure, equations 22-92 and 22-93 and corresponding text | P | Agree in Principle. See resolution in 11/YYYY |

Discussion:

PSDU_LENGTH is computed for PLME-TXTIME and RXVECTOR.  That’s problematic since we have different equations for N_SYM depending on Tx or Rx.  The solution for this is to create a separate PSDU_LENGTH_RX and move computation of PSDU_LENGTH_RX into Rx Procedure.  On further examination of PSDU_LENGTH, it is used several times in conjunction with TXVECTOR.  This is an error since PSDU_LENGTH is not in the TXVECTOR.  Resolve this by deleting linkage to TXVECTOR, since “new” PSDU_LENGTH is calculated by the PHY using the equations in section 22.4.3 using other parameters provided in the TXVECTOR.

TGac editor: modify D1.0 P199L30, as follows

The value of the PSDU_LENGTH parameter returned in the PLME-TXTIME.confirm primitive

for a SU packet using BCC encoding is calculated using Equation (22-97).

TGac editor: modify D1.0 P199L51, as follows

The value of the PSDU_LENGTH parameter returned in the PLME-TXTIME.confirm primitive

for a SU packet using LDPC encoding is calculated using Equation (22-98).

TGac editor: modify D1.0 P200L31, as follows

The value of the PSDU_LENGTH parameter for user u returned in the PLME-TXTIME.confirm primitive

for a MU packet is calculated using Equation (22-99).

TGac editor: modify D1.0 P200L31, as follows

The value of the PSDU_LENGTH parameter returned in the PLME-TXTIME.confirm primitive

for an NDP is 0.

TGac editor: insert the following text after Eq 22-92 in D1.0

The value of the PSDU_LENGTH_RX parameter returned in the RXVECTOR using BCC encoding is calculated using Equation (22-YY).

where

TGac editor: modify above reference to 22-96 to 22-92

TGac editor: insert the following text after Eq 22-93 in D1.0

The value of the PSDU_LENGTH_RX parameter returned in the RXVECTOR using LDPC encoding is calculated using Equation (22-ZZ).

TGac editor: modify above reference to 22-49 to 22-93

TGac editor: modify D1.0 Table 22-5, as follows

| Parameter | Condition | Value | TXVECTOR | RXVECTOR |
| --- | --- | --- | --- | --- |
| … |  |  |  |  |
| PSDU_LENGTH_RX | FORMAT is VHT | Indicates the number of octets in the VHT PSDU in the range of 0 to 1 048 575 octets. | N | Y |
|  | Otherwise | Not present | N | N |
| … |  |  |  |  |

TGac editor: modify D1.0 P156L12, as follows

PSDU_LENGTHu is defined in 22.4.3

TGac editor: modify D1.0 P156L31, as follows

PSDU_LENGTHu is defined in 22.4.3

TGac editor: modify D1.0 P112L42, as follows

NOTE—The rounding up of the LENGTH parameter to a 4-octet

word boundary may result in a LENGTH parameter that is larger

than the PSDU_LENGTH calculated using the equations in section 22.4.3."