IEEE P802.11
Wireless LANs

| Proposed Resolution to Channel Power Management Element Length and Capabilities |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-09-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Eunsun Kim | LG Electronics | Mobile Comm. Lab, LG R&D Complex 533, Hogye1, Dongan, Anyang, Korea | +82-31-450-1860 | Esun.kim@lge.com |
| Yongho Seok | LG Electronics | Mobile Comm. Lab, LG R&D Complex 533, Hogye1, Dongan, Anyang, Korea | +82-31-450-1947 | Yongho.seok@lge.com |
| Jihyun Lee | LG Electronics | Mobile Comm. Lab, LG R&D Complex 533, Hogye1, Dongan, Anyang, Korea | +82-31-450-1860 | jihyun1220.lee@lge.com |

Abstract

This document provides proposed resolutions for technical comments 173, 824, 1227, 32, 246, 402 and 1010.

- Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGaf Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGaf Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGaf Editor: Editing instructions preceded by “TGaf Editor” are instructions to the TGaf editor to modify existing material in the TGaf draft.  As a result of adopting the changes, the TGaf editor will execute the instructions rather than copy them to the TGaf Draft.

| CID | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 173 | 32.36 | 8.4.2.1 | Element id and length fiel not considered when computing the minimal length of the channel power management annoucnement. | Change minimum value of length from "3" to "5" |
| 824 | 27.36 | 8.4.2.1 | Length of Channel Power Management Announcement doesn't agree with description in 8.4.2.af3. | Change length to “5-257”. |
| 1227 | 27.36 | 8.4.2.1 | Inconsistent parameter | change 257 to 255 |

Discussion

CIDs 173, 824 and 1227 ask to fix the length of Channel Power Management IE. CID 173 and 824 request to change the minimum value of the length field so that the element ID and length field can be considered. CID 1227 requests to change the maximum value of the length field. Looking at the Channel Power Management IE, it always contains Element ID, Length, Channel Power Management Mode, Channel Power Switch Count fields and a triplet comprised of Operating Class, Channel Number and Constrained Maximum Transmit Power, which sets the value of minimum length of Channel Power Management to be 7. The maximum length of Channel Power Management is defined by the maximum possible number of the triplets.

Propose Revised for CIDs 173 and 824 per discussion and editing instructions in 11-11/1103r1.

Propose Rejected for 1227 per discussion and editing instructions in 11-11/1103r1.

| CID | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 32 | 28.30 | 8.4.2.29 | Ok, I've tried to parse the essay note in the "Channel Power Management" row several times and failed. I suspect it's the numerous "to/from", "and" and "or" combos. | Make a "dot11ChannelPowerManagementImplemented" MIB attribute, refer to that MIB attribute in table 8-88 and put the rest of the text from the note in clause 10, to describes when dot11ChannelPowerManagementImplemented can be set to true. |
| 246 | 28.30 | 8.4.2.29 | This very long sentence is not clear. Also say "..field is set to 1.." instead of "..field is 1..". Also say "..field is set to 0.." instead of "..field is 0.." | As in comment. |
| 402 | 28.32 | 8.4.2.29 | What is "power limit switching"? | If this only means a change of power limits, then replace "switching" with "changes". |
| 1010 | 28.31 | 8.4.2.29 | This whole sentence (in the Notes column) is non-intelligable | Rephrase. Perhaps "...set to 1 to indicate a change in <something>…" or "…set to 1 to indicate one or more of the following conditions: <enumerated list>" |

Discussion:

CIDs 32,246, 402 and 1010 point out that Notes for channel power management capability field in Table 8-88 is not clearly written. CID 246 and 402 request to choose more appropriate words or terms making the sentence and CID 32 requests to refer to a MIB attribute for clarification. dot11ChannelPowerManagementActivated is defined in annex C. When to set dot11ChannelPowerManagementActivated to true is also defined in E.2.4.

Propose Accepted for CID 32 per discussion and editing instructions in 11-11/1103r1.

Propose Revised for CIDs 246, 402 and 1010 per discussion and editing instructions in 11-11/1103r1.

## Editing instructions:

8.4 Management frame body components

8.4.2 Information elements

TGaf editor: modify the following row in Table 8-54:

Table 8-54 – Element IDs

| Information Element | Element ID | Length (in octets) | Extensible |
| --- | --- | --- | --- |
| Channel Power Management Announcement (see 8.4.2.131 (Channel Power Management Announcement element)) | <ANA> | 3 7 to 257 256 |  |

8.4.2.29 Extended Capabilities element

TGaf editor: Modify Table 8-102 as the following:

Table 8-102 — Capabilities field

| Bit | Information | Notes |
| --- | --- | --- |
| <ANA> | Channel Power Management | The Channel Power Management field is 1 to indicate support for the communication of insertion/deletion of a channel to/from the available channel list, of power limit switching on the operating channel or of channel switching associated with the allowed power limit on the new channel through the transmission and reception of the Channel Power Management Announcement element, RLQP Channel Power Management element, and management frame as described in 8.5.8.27 (Channel Power Management Announcement frame format). The Channel Power Management field is 0 to indicate a lack of support for channel power management. The STA sets the Channel Power Management field to 1 when dot11ChannelPowerManagementActivated is true, and sets it to 0 otherwise. See 10.10.4 Channel power management (CPM). |