IEEE P802.11
Wireless LANs

| WSM beaconing comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-09-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Eunsun Kim | LG Electronics | Mobile Comm. Lab, LG R&D Complex 533, Hogye1, Dongan, Anyang, Korea | +82-31-450-1860 | Esun.kim@lge.com |
| Yongho Seok | LG Electronics | Mobile Comm. Lab, LG R&D Complex 533, Hogye1, Dongan, Anyang, Korea | +82-31-450-1947 | Yongho.seok@lge.com |

Abstract

This document provides proposed resolutions for technical comments 9, 317, 484, 310, 311, 313, 325 and 509.

- Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGaf Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGaf Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGaf Editor: Editing instructions preceded by “TGaf Editor” are instructions to the TGaf editor to modify existing material in the TGaf draft.  As a result of adopting the changes, the TGaf editor will execute the instructions rather than copy them to the TGaf Draft.

| 9 | 25.42 | 8.3.3.2 | White space map should not be sent in the beacon or probe response since it needs to be secured. | Remove White space element from beacon |
| --- | --- | --- | --- | --- |
| 317 | 25.43 | 8.3.3.2 | Transmission of white space map in beacon frame may waste the medium time (actually not required by regulations). | Remove the WSM IE from beacon frame, and the respective rules in clause 10.af2.2. |
| 325 | 106.18 | C.3 | DEFVAL of 1 means that the WSM IE is transmitted every beacon interval as a default value? This will cause large overhead to the network. | If there is the need of WSM in beacons, use a reasonable default value. |
| 509 | 106.18 | C.3 | DEFVAL of 1 means that the WSM IE is transmitted every beacon interval as a default value? This will cause large overhead to the network. | If there is the need of WSM in beacons, use a reasonable default value. |
| 484 | 25.43 | 8.3.3.2 | Transmission of white space map in beacon frame can unnecessarily waste the medium time. I do not think this is required by existing regulations. All that is required is to request the channel map by the dependent STA at the beginning, which should be provided with a secure exchange between the AP and the requesting STA. | Remove the WSM IE from beacon frame, and the respective rules in clause 10.af2.2 (page 64-65). |

Discussion

CID 9, 317 and 484 ask for removal of WSM IE in beacon frame due to a security requirement in some regulatory domains and overhead of medium time. CID 325 and 509 indicate that default interval of WSM transmission in beacon causes a lot of overhead and propose using a reasonable value. We agree with the comments that it causes a lot overhead if a full map of WSM is transmitted in every beacon, however if a partial channel map of WSM is transmitted it does not cause a lot of overhead. Therefore, we propose that a partial WSM should be transmitted in a beacon frame including the beacon and Direct link channels that are in use. APs generate the channel information to be broadcasted in a beacon frame with the order of priorities based on the use of the channels, WSMs transmitted in beacon frames are not the exactly same list as received from the GDB and it does not need to be secure.

Propose Revised for CID 9, 317, 325, 484 and 509 based on the discussion and editing instructions in 11/11-1288r1. We proposed to add descriptions that partial WSMs are included in beacon frames.

| 310 | 65.03 | 10.af2.2 | wording and normative verb question - why is this "shall"? Shouldn't it be "should"? Why is there a normative requirement to operate? | Change "shall transmit the updated WSM element in the next beacon transmission instance when the WSM element is supposed to be transmitted within the beacon frame if the enabling STA is an AP-STA" to "should transmit a WSM element that has been updated with the recently acquired available channel information in the next beacon that is scheduled to include a WSM element if the enabling STA is an AP-STA" |
| --- | --- | --- | --- | --- |
| 311 | 64.65 | 10.af2.2 | "Whenever the information from the regulatory domain database is updated" - can we be more specific about what this information is? | Provide a more formal description of the information that is being referenced. See also the next several paragraphs. |
| 313 | 65.25 | 10.af2.2 | Again, doesn't the AP have a choice here? | Change "shall transmit the updated WSM element" to "should transmit the updated WSM element" |

Discussion :

CID 310, 311 and 313 are commenting on the description of normative behaviors of WSM. The paragraph that CID 310 and 311 are commenting on is proposed to be removed. The sentence that CID 313 is commenting on is proposed to be removed.

Propose Revised to CID 310, 311 and 313 per discussion and editing instructions in 11-11/1288r1.

10.38 Operation under the control of a Geolocation Database

10.38.4 White space map (WSM)

TGaf editor: Change the first paragraph in 10.38.4 as follows:

If dot11GDBAccessActivateddot11WhiteSpaceMapActivated is set to true, a GDC enabling STA transmits a WSM and a GDC dependent STA shall not operate unless it has a valid WSM. GDBs can be different according to each regulatory domain, and its regulatory requirements. If the WSM Type is 1, it indicates that the WSM is generated from the TV band database and the WSM Information field of the WSM element specifies the available information for TVWS, which is country-specific and is described in 8.2.6.1.5 (WSM Information Values).

TGaf editor: Change the 5th paragraph in 10.38.4 as follows:

A GDC enabling STA that has dot11WhiteSpaceMapActivated set to true shall includes a WSM element within at least one transmitted beacon frame every dot11WhiteSpaceMapPeriod beacon intervals if the GDC enabling STA is an AP STA. WSMs in beacon frames are partial WSMs and include the frequency information that are in use of the AP or the STA in an IBSS and direct link channels.  Whenever the information from the regulatory domain database is updated and When the GDC enabling STA retrieves the updated available channel information from a GDB the database, then the GDC enabling STA shall operate only within the available channel list. If the GDC enabling STA is an AP-STA, it shall transmits the updated WSM element in the next beacon transmission instance where the WSM element is schedule to be included within the beacon frame.

TGaf editor: Change the text after 9th paragraph in 10.38.4 as follows:

A dependent AP STA that has dot11WhiteSpaceMapActivated set to true should include a WSM element within at least one transmitted a beacon frame every dot11WhiteSpaceMapPeriod beacon interval. Whenever the information from the regulatory domain database is updated and the dependent AP received the updated available channel information from the GDC enabling STA, then the dependent AP shall operate only within the available channel list and the dependent AP shall transmit the updated WSM element in the next beacon transmission instance when the WSM element is supposed to be transmitted within the beacon frame.

GDC dependent STAs receiving a WSM should scan for existing BSSs on the available channels identified within received WSM element. A GDC dependent STA receiving a WSM element shall operate on the channels indicated in the WSM. A GDC dependent STA that has previously received a WSM and that receives an updated WSM from its AP or GDC enabling STA shall move its channel of operation if it is operating on a channel that has become unavailable in the updated WSM.

If dot11WhiteSpaceMapActivated is true, then the enabled GDC dependent STA can send a probe request frame on any channel identified in the received WSM element.

An AP that has dot11WhiteSpaceMapActivated set to true and that receives a probe request frame should respond with a probe response frame containing a WSM element.

Receipt of a WSM in beacon or probe response frame does not affect the GDC Enablement status of a STA.

Annex C(normative)

ASN.1 encoding of the MAC and PHY MIB

C.3 MIB Detail

TGaf editor: Change the following elements at the end of the dot11StationConfigTable element definitions:

dot11WhiteSpaceMapPeriod OBJECT-TYPE

SYNTAX Unsigned32(1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive.

This attribute specifies the minimum number of beacon intervals that

elapse between transmission of Beacon frames containing

a White Space Map element.”

"

DEFVAL {1}

::= { dot11StationConfigEntry <ANA> }