IEEE P802.11
Wireless LANs

| Comment resolution for CIDs: 2964, 3270, 2061, 2418, 2424, 2231, 2423, 3143, 2320, 2321, 2965, 2417, 3124 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-10-26 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Yi Luo | Huawei Technologies | F1-17, Bantian, Shenzhen, China | +86-18665891036 | Roy.Luoyi@huawei.com |

Abstract

The baseline is 11ac D 1.2. This document provides the comment resolution for the CIDs: 2964, 3270, 2061, 2418, 2424, 2231, 2423, 3143, 2320, 2321, 2965, 2417, 3124

| CID | Page | Comment | Proposed Change | Resn Status |
| --- | --- | --- | --- | --- |
| 2964 | 144.12 | In the description of B2-B3 of VHT-SIG-A2, the first "B3" should be "B2". | change "B3" to "B2" | Agree in principle. See CID2060 |
| 3270 | 144.12 | The bit identification "B3:" is not correct. | Change "B3:" to "B2:". | Agree in principle. See CID2060 |
| 2061 | 144.25 | In an MU-MIMO PPDU case, B4-B7 in VHT-SIG-A2 do not carry MCS index, so the name of this field should be reconsidered. | Instead of "MCS", "MCS/Coding" should be used. | Agree |
| 2418 | 144.09 | Since these fields are really composed of subfields and the subfields need to be explicitly referred to elsewhere, the section talks of B0, B1 etc. Better is to give each atomic component its own distinct name "Partial AID", "NSTS0", "Short GI" "Short GI Nsym Disambiguation" etcetc | Give atomic components their own name | Agree |

Discussion:

CID 2964 and 3270 overlapped with CID 2060, which had passed in the motion. For the clarification, parts of subfields in VHT-SIG-A are renamed as below.

- Editing Instructions:

- Page 183, line 26, part of Table 22-10:

| VHT-SIG-A1 | B10-B21 | NSTS/Partial AID | 12 | For MU: 3 bits/user with maximum of 4 users (user u uses bits B(10+3u )-B(12+3u ),u=0,1,2,3 ) Set to 0 for 0 space time streams Set to 1 for 1 space time stream Set to 2 for 2 space time streams Set to 3 for 3 space time streams Set to 4 for 4 space time streams Values 5-7 are reserved For SU: B10-B12 Set to 0 for 1 space time stream Set to 1 for 2 space time streams Set to 2 for 3 space time streams Set to 3 for 4 space time streams Set to 4 for 5 space time streams Set to 5 for 6 space time streams Set to 6 for 7 space time streams Set to 7 for 8 space time streams B13-B21 Partial AID: Set to the value of the TXVECTOR parameter PARTIAL_AID. Partial AID provides an abbreviated indication of the intended recipient(s) of the frame (see 9.17a (Group ID and Partial AID in VHT PPDUs)). |
| --- | --- | --- | --- | --- |

- Page 184, line 9, part of Table 22-10:

| VHT-SIG-A2 | B0-B1 | Short GI | 21 | B0: Set to 0 if short guard interval is not used in the Data field. Set to 1 if short guard interval is used in the Data field. |
| --- | --- | --- | --- | --- |
|  | B1 | Short GI Nsym disambiguation | 1 | B1: Set to 1 if short guard interval is used and NSYM mod 10 = 9, otherwise set to 0. NSYM is defined in 22.4.3 (TXTIME and PSDU_LENGTH calculation). |
|  | B2-B3 | SU/MU[0] Coding | 21 | B3: For SU, B2 is set to 0 for BCC, 1 for LDPC For MU, if the NSTS field for user 0 MU[0] NSTS is non-zero, then B2 indicates the coding used for user 0; set to 0 for BCC and 1 for LDPC. If the NSTS field for user 0 MU[0] NSTS is set to 0, then this field is reserved and set to 1. |
|  | B3 | Extra OFDM symbol | 1 | B3: Set to 1 if the LDPC PPDU encoding process, or at least one LPDC user’s PPDU encoding process, results in an extra OFDM symbol (or symbols) as described in 22.3.10.5.2 (LDPC coding) and 22.3.10.5.3 (Encoding process for MU transmissions). Set to 0 otherwise. |
|  | B4-B7 | SU MCS/MU[1-3] Coding | 4 | For SU: MCS index For MU: If the NSTS field for user 1 MU[1] NSTS is non-zero, then B4 indicates coding for user 1: set to 0 for BCC, 1 for LDPC. If NSTS for user 1 MU[1] NSTS is set to 0, then B4 is reserved and set to 1. If the NSTS field for user 2 MU[2] NSTS is non-zero, then B5 indicates coding for user 2: set to 0 for BCC, 1 for LDPC. If NSTS for user 2 MU[2] NSTS is set to 0, then B5 is reserved and set to 1. If the NSTS field for user 3 MU[3] NSTS is non-zero, then B6 indicates coding for user 3: set to 0 for BCC, 1 for LDPC. If NSTS for user 3 MU[3] NSTS is set to 0, then B6 is reserved and set to 1. B7 is reserved and set to 1 |

| CID | Page | Comment | Proposed Change | Resn Status |
| --- | --- | --- | --- | --- |
| 2417 | 142.15 | Traditionally this information has been presented in a diagram | Add diagram | Agree in principle |

Discussion:

Insert two figures for VHT-SIG-A1 and VHT-SIG-S2 structure respectively. The changing of fields name in other paragraphs in the draft can be seen in annex.

- Editing Instructions:

Insert in after Table 22-10:

|  |  |  |  | SU NSTS | Partial AID |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BW | R | STBC | Group ID | MU[0] NSTS | MU[1] NSTS | MU[2] NSTS | MU[3] NSTS | TXOP_PS_NOT_ALLOWED | R |
| 0-1 | 2 | 3 | 4-9 | 10-12 | 13-15 | 16-18 | 19-21 | 22 | 23 |

Figure 22-9a VHT-SIG-A1 structure

| Short GI | Short GI Nsym Disambiguation | SU/MU[0] Coding | Extra OFDM Symbol | SU MCS |  |  |  | Beam-formed | R | CRC | Tail |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MU[1] Coding | MU[2] Coding | MU[3] Coding | R |  |  |  |  |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10-17 | 18-23 |

Figure 22-9b VHT-SIG-A2 structure

| CID | Page | Comment | Proposed Change | Resn Status |
| --- | --- | --- | --- | --- |
| 2424 | 145.28 | Dk,n,BW -> Dk,n,20; dM'BW(k),n -> dM'20(k),n; M'BW(k) -> M'20(k) (these are 20MHz mappings) |  | Agree. |

Discussion:

This comment is overlapped with CID 2416, the resolution can be found in document 11/1042r1.

| CID | Page | Comment | Proposed Change | Resn Status |
| --- | --- | --- | --- | --- |
| 2231 | 145.15 | The formula for VHT-SIG-A contains a multiplication with "j" for symbol n=1 (i.e. VHT-SIG-A2). The preceding text that describes the construction of the complex numbers d_k,n seems to imply that the rotation of VHT-SIG-A2 is already contained in the numbers D_k,n. In that case, no explicit rotation is needed anymore. | Make text and formula consistent. | Agree in principle. |

Discussion:

Agree that there is ambiguity about the rotation of VHT-SIG A2 between text and equations. Some modification is made to eliminate this ambiguity.

- Editing Instructions:

- Page 185, line 2:

The VHT-SIG-A symbols shall be BCC encoded at rate, R = 1/2, interleaved, mapped to a BPSK constellation, and have pilots inserted following the steps described in 18.3.5.6 (Convolutional encoder), 18.3.5.7 (Data interleaving), 18.3.5.8 (Subcarrier modulation mapping), and 18.3.5.9 (Pilot subcarriers), respectively. The first and second half of the stream of 96 complex numbers generated by these steps is divided into two groups of 48 complex numbers dk,n, k=0…47, where n=0,1 respectively. The BPSK constellation for VHT-SIG-A2 subfield is rotated by 90° counter-clockwise relative to VHT-SIG-A1 subfield in order to accommodate differentiation of the VHT PPDU from a non-HT and HT PPDU. The first and second half of the stream of 96 complex numbers generated by these steps is divided into two groups of 48 complex numbers dk,n, k=0…47, where n=0,1 respectively. The time domain waveform for the VHT-SIG-A field in a VHT format packet shall as specified in Equation (22-24).

| CID | Page | Comment | Proposed Change | Resn Status |
| --- | --- | --- | --- | --- |
| 2423 | 144.48 | Reference to clause 19 is incomplete given that the bits to be protected only appear in this subclause | "CRC calculated as in 19.xxx with Bits 0-23 of HT-SIG1 and bits 0-9 of HT-SIG2 replaced by bits 0-23 of VHTSIGA1 and bits 0-9 of VHT_SIGA2 respectively" | Agree |

Discussion:

Agree

- Editing Instructions:

- Page 184, line 54, part of Table 22-10:

| VHT-SIG-A2 | B10-B17 | CRC | 8 | CRC calculated as in 19.3.9.4.4(CRC calculation for HT-SIG) with C7c7 in B10, etc. Bits 0-23 of HT-SIG1 and bits 0-9 of HT-SIG2 replaced by bits 0-23 of VHTSIGA1 and bits 0-9 of VHT_SIGA2 respectively. |
| --- | --- | --- | --- | --- |

| CID | Page | Comment | Proposed Change | Resn Status |
| --- | --- | --- | --- | --- |
| 3143 | 143.09 | Clarify that STBC must be used for all users in MU case | As in comment | Agree in principle |

Discussion:

Agree in principle.

- Editing Instructions:

- Page 143, line 9, part of Table 22-10:

| VHT-SIG-A1 | B3 | STBC | 1 | Set to 1 if all streams of all users have space time block coding and set to 0 otherwise if none of streams of any user has space time block coding. Parts of users use space time block coding is not allowed. |
| --- | --- | --- | --- | --- |

| CID | Page | Comment | Proposed Change | Resn Status |
| --- | --- | --- | --- | --- |
| 2320 | 143.21 | Table 22-10: range for user u is u = 0,1,2,3 | Change to " u = 1, 2, 3, 4 " and accordingly B(7+3u)-B(9+3u) | Disagree See CID2040. |
| 2321 | 143.22 | Definition includes u=1, which means a single user | My understanding is, that at least 2 users are required for MU operation. Explain difference between Multi-User operation with 1 user and Single User operation, if supported. | Disagree |

Discussion:

CID 2320 overlapped with CID 2040, the resolution can be found in the document 11-11-1369-00-00ac-D1_comment_resolution_brianh_part7.docx.

For CID2321: the difference between Multi-user operation with 1 user and Single user operation is only at VHT-SIG-A setting. But it doesn’t necessary to forbid the Multi-User operation with 1 user. How to choose the transmit operation should up to the AP.

| CID | Page | Comment | Proposed Change | Resn Status |
| --- | --- | --- | --- | --- |
| 2965 | 144.39 | 19.3.11.11.2 (Spatial mapping) is not originally for 802.11ac. When citing the content of this subclause, it should be pointed out that the maximum spatial stream is extended to 8. | Add a corresponding explanation to describe the maximum spatial stream is extended to 8. | Disagree |

Discussion:

Disagree. When the beamforming steering matrix is introduced in clause 19.3.11.11.2, the maximum NSTS doesn’t restricted to 4, so it doesn’t need to make an extension here.

| CID | Page | Comment | Proposed Change | Resn Status |
| --- | --- | --- | --- | --- |
| 3124 | 144.46 | It would simplify implementation, and potentially reduce power consumption if a "sounding bit" were included in the VHT-SIGA. Currently, the beamformee must wait until VHT-SIGB field to compute the steering matrices. | Replace reserve bit B9 in SIGA with a VHT sounding bit. | Disagree |

Discussion:

Beamformer will send an announcement before it sends a NDP frame. No matter the announcement is VHT NDPA frame or an NDP announcement bit in HT Control field, beamformee will definitely know when the NDP frame will be received, so it doesn’t need to add “sounding bit” in VHT-SIG A.

Pre-Motion 1:

Do you accept the resolutions provided to the CIDs and the changes to the spec text as presented in editing instructions sections of this document?

Yes:

No:

Abstain:

- References:

- IEEE Draft P802.11ac_D1.2

Annex:

paragraph in page 175, line 41 makes the following changes:

An example definition of the windowing function, , is given in 18.3.2.5 (Mathematical conventions in the signal descriptions).  is 8 μs for L-STF and L-LTF, 4 μs for L-SIG, VHT-SIG-A, VHT-STF, VHT-LTF and VHT-SIG-B.  is also 4 μs for VHTData when not using the short guard interval (B0 Short GI field of VHT-SIG-A2 is equals to 0).  is 3.6 μs for VHTData when using the short guard interval (B0 Short GI field of VHT-SIG-A2 is equals to 1).

paragraph in page 176, line 28 makes the following changes:

TGI,Field is the guard interval duration used for each OFDM symbol in the field. For L-STF and VHT-STF, TGI,Field is 0 μs. For L-SIG, VHT-SIG-A, VHT-LTF and VHT-SIG-B fields, TGI,Field = TGI . For the L-LTF field, TGI,Field = TGI2 . For the Data field, TGI,Field = TGI when not using the short guard interval (B0 Short GI field of VHT-SIG-A2 is equals to 0) and TGI,Field = TGIS when using the short guard interval (B0 Short GI field of VHT-SIG-A2 is equals to 1). TGI, TGI2 and TGIS are given in Table 22-5(Timing-related constants).

paragraph in page 199, line 26 makes the following changes:

The Data field shall be encoded using either the binary convolutional code (BCC) defined in 22.3.10.5.2 (BCC encoder parsing operation) and 22.3.10.5.3 (Binary convolutional coding and puncturing), or the low density parity check (LDPC) code defined in 22.3.10.5.4 (LDPC coding). The encoder is selected by the SU/MU[0] Coding, MU[1] Coding, MU[2] Coding, or MU[3] Coding field in VHT-SIG-A, as defined in 22.3.8.2.3 (VHT-SIG-A definition).

paragraph in page 200, line 56 makes the following changes:

In addition, if NSYM computed in Equation (20-41) in step (d) of 20.3.11.7.5 (LDPC PPDU encoding process) is greater than NSYM,init, then B3 Extra OFDM Symbol field of VHT-SIG-A2 shall be set to 1. Otherwise, B3 Extra OFDM Symbol field of VHT-SIG-A2 shall be set to 0.

paragraph in page 202, line 37 makes the following changes:

In addition, if NSYM computed in Equation (22-63) is greater than NSYM_max_init computed in Equation (22-61), then B3 Extra OFDM Symbol field of VHT-SIG-A2 shall be set to 1. Otherwise, B3 Extra OFDM Symbol field of VHT-SIG-A2 shall be set to 0.

paragraph in page 214, line 1 makes the following changes:

TGI,Datais the guard interval duration. TGI,Data = TGI when not using the short guard interval (B0 Short GI field of VHT-SIG-A2 is equals to 0) and TGI,Data = TGIS when using the short guard interval (B0 Short GI field of VHT-SIG-A2 is equals to 1). TGI and TGIS are given in Table 22-5 (Timing-related constants).

paragraphs in page 217, line 35 makes the following changes:

A value in the Group ID field in VHT-SIG-A (see 22.3.8.2.3 (VHT-SIG-A definition)) in the range 1 to 62 indicates an MU-MIMO packet. Prior to transmitting a MU-MIMO packet, group definitions have been established by the AP for MU-MIMO capable STAs using the Group ID Management frame as defined in 8.5.18.3 (Group ID Management frame format). For Group IDs Group ID[i] (i=0, 1, …, 63) whose corresponding Membership Status in Group ID[i] subfield is set to 1 in the Group ID Management frame, the User Position in Group ID[i] subfield determines which of the four sets of 3 bits in the NSTS MU[User Position in Group ID[i]] NSTS field in VHT-SIG-A1 corresponds to the user in an MU transmission. When an MU-MIMO data packet is received, each STA identifies whether it is a member of the group for this packet by detecting the Group ID field in VHT-SIG-A. If a STA finds that it is a member of the group for the MU-MIMO data packet, the STA reads the number of space-time streams from its corresponding 3 bits in the NSTS field in VHT-SIG-A as determined by the group definition of the corresponding Group ID. At this point, a STA is also able to identify which space-time streams correspond to its own signal and which streams correspond to interference. For an MU-MIMO transmission, VHT-LTF symbols are used to measure not only the channel for a beamformee’s designated signals but also to measure the channel for the interfering signals at the beamformee. While receiving an MU-MIMO transmission, it is recommended that the receiver use its channel knowledge to all spatial streams (including those that are interference) to do receive processing, in order to reduce potential interference from other users' space-time streams.

If a STA finds that it is not a member of the group, or the STA is a member of the group but the corresponding MU[u] NSTS (u=0, 1, 2, or 3) in VHT-SIG-A indicates that there are zero space-time streams for the STA in the packet, then the STA may elect to not process the remainder of the packet.

paragraph in page 218, line 18 makes the following changes:

NOTE—The number of VHT-LTF symbols in the NDP is determined by the SU NSTS field in VHT-SIG-A1.

Sentences in page 235, line 30 makes the following changes:

Reserved VHT-SIG-A Indication is defined as a VHT-SIG-A with Reserved bits equal to 0 or NSTS per user for MU MU[u] NSTS fields (u=0, 1, 2, 3) set to 5-7 or Short GI with VHT-SIG-A2 B0 field set to 0 and VHT-SIG-A2 B1 Short GI Nsym Disambiguation filed set to 1, or a combination of MCS and NSTS not included in 22.5 (Parameters for VHT MCSs) or any other VHT-SIG-A field bit combinations that do not correspond to modes of PHY operation defined in Clause 22.

Equation (22-100) in page 236, line 1 makes the following changes:

Original:

Revised:

Paragraph in page 236, line 40 makes the following changes:

For an SU transmission, B2 SU/MU[0] Coding field of VHT-SIG-A2 indicates the type of coding. The PHY entity shall use an LDPC decoder to decode the C-PSDU if this bit is 1, otherwise a BCC decoder shall be used. For an MU transmission, B2, B4, B5 and B6 SU/MU[0] Coding, MU[1] Coding, MU[2] Coding and MU[3] Coding fields of VHT-SIG-A2 indicate the type of coding for user 0, 1, 2 and 3, respectively. The PHY entity shall use an LDPC decoder to decode the C-PSDU for the respective user if its bit for its C-PSDU is 1. A BCC decoder shall be used otherwise. When an LDPC decoder is to be used, Npld can be computed by Equation (22-57) using NSYM,init obtained from Equation (22-102).

Equation (22-102) in page 236, line 50 makes the following changes:

Original:

Revised:

Part of Table 22-59 in page 263, line 1 makes the following changes:

| Parameter | Associate | primitive |
| --- | --- | --- |
| GI_TYPE | PMD_TX_PARAMETERS.request | Set to 0 indicates short GI is not used in the packet Set to 1 indicates short GI is used in the packet and Short GI Nsym Disambiguation field equals to 0 in VHT-SIG-A2 B1 is 0 Set to 2 indicates short GI is used in the packet and Short GI Nsym Disambiguation field equals to 1 in VHT-SIG-A2 B1 is 1 |
| FEC_CODING | PMD_TX_PARAMETERS.request | Indicates which FEC encoding is used. Enumerated type: BCC_CODING indicates binary convolutional code. LDPC_CODING_0 indicates low-density parity check code and Extra OFDM Symbol field equals to 0 in VHT-SIG-A2 B3 is 0. LDPC_CODING_1 indicates low-density parity check code and Extra OFDM Symbol field equals to 1 in VHT-SIG-A2 B3 is 1. |