IEEE P802.11
Wireless LANs

| Remaining WSM comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-11-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Eunsun Kim | LG Electronics | Mobile Comm. Lab, LG R&D Complex 533, Hogye1, Dongan, Anyang, Korea | +82-31-450-1860 | Esun.kim@lge.com |

Abstract

Proposed resolutions to remaining comments in MAC WSM:

724, 584, 585, 930, 931, 312, 544, 613, 904, 944, 977, 838, 457, 1038 and 1039

Editing instructions are based on P802.11af Draft 1.04.

- Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGaf Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGaf Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGaf Editor: Editing instructions preceded by “TGaf Editor” are instructions to the TGaf editor to modify existing material in the TGaf draft.  As a result of adopting the changes, the TGaf editor will execute the instructions rather than copy them to the TGaf Draft.

| CID | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 724 |  | General | The draft constantly switches between using "WSM" and "White Space Map". This is not consistent | Use "WSM" wherever possible. |

Discussion

CID 724 ask to use of “WSM” instead of “White Space Map”. As the TGaf D1.04 has a definition of white space map and acronym of “WSM”, we agree to use of WSM and propose to replace White Space Map with WSM.

Propose Accepted for CDI 724 per discussion and editing instruction in 11-11/1466r1.

| 584 /585 /930 /931 | 137.00 | E.3.1.5 | MAP ID field is not necessarily to be country-specific or band-specific. | Move the description on the MAP ID field into the main body section in White Space Map. See forthcoming document 11-11/xxxxr0. |
| --- | --- | --- | --- | --- |

Discussion

CIDs 584, 585, 930 and 931 ask to move MAP ID field to main body section. As the text was moved to the TLV section in clause 8 in D1.01, not only description about MAP ID field but all of description of  other fields were moved to clause 8.

Propose Revised for CIDs 584, 585, 930 and 931 per discussion in 11-11/1466r1. No change is required in the current working draft D1.04.

| 312 | 65.09 | 10.af2.2 | Isn't there a timeout value for how long a dependent STA can operate without hearing from the enabler? Shouldn't that timeout be referenced here when discussing when the new information needs to be transmitted? | Make reference to a timeout MIB that will be used to determine a deadline for transmitting the new information. |
| --- | --- | --- | --- | --- |

Discussion

CID 312 requests to make reference to a timeout value for how long hearing from the enabler is valid. As the approval of 11-11/1035r2 and 11-11/414r7 in July meeting, a GDC dependent STA maintains its GDC enablement state during the time of dot11GDCEnablementValidTimer and when it receives a CVS signal, the value of the timer is set to dot11ContactVerificationSignalInterval. If the timer has expired, the GDC dependent STA becomes unenabled. Related behaviours are described in 11af D1.04 and no change is required.

Propose Rejected for CID 312 based on the discussion in 11-11/1466r1.

| 544 /613 /904 | 64.00 | 10.af.2 | "If an enabled dependent STA does not receive a WSM within dot11WhiteSpaceMapValidTime, it becomes unenabled." This sentence is related with the behavior of CVS signal. Provide a clear behavor of the case that STAs became unenabled. | See forthcoming document 11-11/xxxxr0. |
| --- | --- | --- | --- | --- |

Discussion

CID 544, 613 and 904 ask for clarifying the behaviour of the case that STAs becomes unenabled. CVS signals determine the validity of a WSM and we propose to remove dot11WhiteSpaceMapValidTime and corresponding rules. As the approval of 11-11/1035r2 and 11-11/414r7 in July meeting, the method of maintaining “enabled” state and conditions that a GDC dependent STA becomes “unenabled” are now clear. See 11-11/1035r2 and 11-11/414r7.

Propose Accepted for CID 544, 613 and 904 based on discussion in 11-11/1466r1. See 11-11/1035r2 and 11-11/414r7.

| 977 | 109.48 | C.3 | The description dot11WhiteSpaceMapValidTime is recursive. WSM element is also present in other frames as well such as Probe response. | replace "This attribute specifies the amount of time in units of seconds that a White Space Map element is valid for a dot11WhiteSpaceMapValidTime from the transmission of the beacon frame containing WSM element.” with "This attribute specifies the amount of time in units of seconds that a White Space Map element is valid from the transmission of the frame containing WSM element.” |
| --- | --- | --- | --- | --- |

Discussion

CID 977 indicates that the description of dot11WhiteSpaceMapValidTime is recursive and asks to change the sentence. However, we propose to remove this dot11WhiteSpaceMapValidTime from the TGaf draft as the approval of 11-11/1035r2 and 11-11/414r7 in July meeting, which are describing the ways of keeping the enablement state and how long the WSM is valid, the corresponding sentence and descriptions need to be removed.

Propose Revised for CID 977 based on the discussion and editing instruction in 11-11/1466rx. See 11-11/1035r2 and 11-11/414r7.

| 838 | 65.18 | 10.af.2.2 | If the complete channel list is spread over several beacon frames and takes longer than the timeout, then some dependent STAs operating on some channels may be forced to stop operations. | Unless this was covered elsewhere, add a sentence requiring the enabling STA to provide the complete channel map within the timeout period, preferably more than once to accommodate the occasional lost packet. |
| --- | --- | --- | --- | --- |

Discussion

CID 838 asks to add a description of timeout value for transmission of complete channel list. Even though the complete channel list is spread over several frames over timeout, the validity of WSM is controlled by receptions of CVS frames. Therefore, no change is required.

Propose Rejected for CID 838 per discussion in 11-11/1466r1.

| 457 | 4.10 | 6.3.af1.1.4 | The usual 11mb statement for Effect of Receipt of request to transmits a frame is a bit more general, but also clearer. | Replace the sentence: "The STA then attempts…another STA." with: "This frame is then scheduled for transmission." |
| --- | --- | --- | --- | --- |

Discussion:

CID 457 asks to replace the sentence as proposed, “This frame is then scheduled for transmission.” and we agree.

Propose Accepted for CID 457 per discussion and editing instruction in 11-11/1466r1.

| 1038 | 4.11 | 6.3.af1.1.4 | "attempts to transmit this to another STA." - presumably it flips a coin to decide which. | Replace "another" with "the specified destination" |
| --- | --- | --- | --- | --- |
| 1039 | 4.18 | 6.3.af1.2.1 | "received from another STA." - well, it's not going to receive it from itself is it? | Remove meaningless fluff: "from another STA" or replace with something informative. |

Discussion

CID 1038 and 1039 ask to clarify the meaning of another STA and specify the destination. We agree that WSM are transmitted from GDC enabling STAs.

Propose Accepted for CIDs 1038 and 1039 per discussion and editing instructions in 11-11/1466r1.

| 944 |  | General | According to the FCC rule, a fixed device provides a supplemental list of available channels above 512 MHz (above TV channel 20) that are adjacent to occupied TV channels on which the Mode I device, but not the fixed device, may operate." | Describe how White Space Map provides a supplemental list of available channels for the Mode I device. |
| --- | --- | --- | --- | --- |

Discussion

CID 944 asks to describe a way that fixed device can provide supplemental list of available channels for personal /portable devices. In the approved document 11-11/774r3, it is said that “The value of a Device Class field of a WSM in a GDC Enablement Response frame and CAQ response frame shall be set to a value of a Device Class in a GDC Enablement Request frame and CAQ Request frame, respectively.”

Propose Accepted for CID 977 per discussion in 11-11/1466r1. See 11-11/774r3.

Editing Instructions

TGaf Editor: Replace all occurrences of “White Space Map” and “white space map” with WSM in all clauses after clause 4.

TGaf editor: Change the first sentence in sub-clause 6.3.93.1.4as follows:

6.3.93.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a (Protected) GDC Enablement Request frame. The STA then attempts to transmit this to the peer entity. This frame is then scheduled for transmission.

TGaf editor: Change the first sentence in sub-clause 6.3.93.4.4as follows:

6.3.93.4.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a GDC Enablement Response frame. The STA then attempts to transmit this to the peer entity. This frame is then scheduled for transmission.

TGaf editor: Change the first sentence in sub-clause 6.3.94 as follows:

6.3.94 White space map

The following MLME primitives support the signaling of the white space map. Figure 6-29 (White Space Map Protocol Exchange) depicts the white space map process. The figure illustrates the basic protocol, which is only an example and is not meant to be exhaustive of all possible protocol uses.

6.3.94.1 MLME-WSM.request

6.3.94.1.1 Function

This primitive requests that a White Space Map Announcement frame be sent to another STA. by a GDC enabling STA in order to provide a WSM to a GDC dependent STA.

6.3.94.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-WSM.request(

PeerSTAAddress,

WhiteSpaceMap

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entitiy with which to perform the white space map process. |
| WhiteSpaceMap | White Space Map element | As defined in 8.4.2.160 (White Space Map element) | Specifies the service parameters for the white space map. |

6.3.94.1.3 When Generated

This primitive is generated by the SME to request that a White Space Map Announcement frame be sent to another STA. by a GDC enabling STA to a specified peer MAC entity.

6.3.94.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a White Space Map Announcement frame. The STA then attempts to transmit this to another STA. This frame is then scheduled for transmission.

6.3.94.2 MLME-WSM.confirm

6.3.94.2.1 Function

This primitive reports the result of an MLME-WSM.request primitive.

6.3.94.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-WSM.confirm (

ResultCode

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, REFUSED, TIMEOUT | Indicates the result response to the white space map process. |

6.3.94.2.3 When Generated

This primitive is generated by the MLME as a result of an MLME-WSM.request and indicates the results of the request.

6.3.94.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the results of the MLME-WSM.request primitive and may use the reported data.

6.3.94.3 MLME-WSM.indication

6.3.94.3.1 Function

This primitive indicates that a White Space Map Announcement frame was received from another STA. a GDC enabling STA.

6.3.94.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-WSM.indication(

PeerSTAAddress,

WhiteSpaceMap

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity from which a white space map was received. |
| WhiteSpaceMap | White Space Map element | As defined in 8.4.2.160 (White Space Map element) | Specifies the service parameters for the white space map. |

6.3.94.3.3 When Generated

This primitive is generated by the MLME when a valid White Space Map Announcement frame is received.

6.3.94.3.4 Effect of Receipt

On receipt of this primitive, the SME operates according to the procedure in 10.38.4 (White space map (WSM)).