IEEE P802.11
Wireless LANs

| CID 3351 Comment Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-11-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Yong Liu | Marvell | 5488 Marvell Ln, Santa Clara, CA 95054 | 4082228412 | yongliu@marvell.com |

Abstract

This document provides resolution to CID 3351 and 2861 based on 11ac D1.2.

| 3351 | 9.5 | Section 9.5 of the 802.11 spec says that "A STA shall be capable of receiving fragments of arbitrary length that is less than the maximum allowed MSDU size, plus any encapsulation headers." This implies a STA, including by omission a VHT STA, is not required to be capable of receiving a fragment of a size greater than 2304 (plus encapsulation). This conflicts with the desire to support MMPDUs of size greater than 2304 octets | Maybe changing "MSDU" to "MSDU (or, for VHT STAs, MMPDU if appropriate)" would fix the problem -- but you still need to be clear on what the maximum MMPDU size is here (it's not the theoretical maximum, but the maximum as derived from the STA's maximum MPDU size capability). You probably also need to tweak "The length of a fragment shall never be larger than dot11FragmentationThreshold" and/or change the maximum value of dot11FragmentationThreshold (in annex C) | AGREE IN PRINCIPLE See text changes in 11/xxxxr0 |
| --- | --- | --- | --- | --- |

Editor: Please make the following changes:

9.5 Fragmentation

A fragment is an MPDU, the payload of which carries all or a portion of an MSDU or MMPDU. When data are to be transmitted, the number of octets in the fragment (before processing by the security mechanism) shall be determined by dot11FragmentationThreshold and the number of octets in the MPDU that have yet to be assigned to a fragment at the instant the fragment is constructed for the first time. Once a fragment is transmitted for the first time, its frame body content and length shall be fixed until it is successfully delivered to the immediate receiving STA. A STA shall be capable of receiving fragments of arbitrary length that is less than the maximum allowed MSDU, plus any encapsulation headers, for a fragmented data frame, or the maximum allowed MMPDU size for a fragmented management frame.