IEEE P802.11
Wireless LANs

| Comment Resolution for Multi-rate Support |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-11-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Yongho Seok | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea | +82-31-450-1497 | yongho.seok@lge.com |

Abstract

Submission for Draft P802.11ac_D1.0 comment resolution. The document addresses various comments related to multi-rate support.

Submission addresses Draft P802.11ac_D1.0 comments 2265, 2266, 2943 and 3444.

Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGac Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGac Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGac Editor:  Editing instructions preceded by “TGac Editor” are instructions to the TGac editor to modify existing material in the TGac draft.   As a result of adopting the changes, the TGac editor will execute the instructions rather than copy them to the TGac Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

| CID | Commenter Name | Subclause | Page | Line | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2265 | Edgar, Richard | 9.7.5.6 | 74 | 26-36 | T | This wording is not logical. The first bullet point '- Should set …' makes the second bullet point '- Shall not set …' completely redundant: If a value is the same as another value, it cannot be greater than it. | Reword as: — Shall set the TXVECTOR parameter CH_BANDWIDTH to the same value as the RXVECTOR parameter CH_BANDWIDTH for the frame eliciting the response. — Shall not set the TXVECTOR parameter CH_BANDWIDTH to a value greater than the TXVECTOR parameter CH_BANDWIDTH for the frame eliciting the response. Explanation: The RXVECTOR parameter CH_BANDWIDTH is always known to the responder. When receiving non-HT duplicate frames with I/G bit equal to zero, there is no data in the signal fields or payload of these frames indicating the TXVECTOR parameter CH_BANDWIDTH that the frame was transmitted with. The receiver may estimate the TXVECTOR parameter CH_BANDWIDTH for these frames by other means [and assign this estimate to the RXVECTOR parameter CH_BANDWIDTH]. This rule [with the new wording] constrains the implementation of the responder's RXVECTOR CH_BANDWIDTH estimate to prevent the responder transmitting a response frame with a wider bandwidth than the frame responded to. The original wording is inadequate [as well as illogical] because it allows the responder to transmit a response with a wider bandwidth than the frame eliciting the response, if the receiver's estimated RXVECTOR parameter CH_BANDWIDTH is incorrect. The 'Note' on line 34 becomes redundant. |

Discussion:

The following two sentences are not redundant. Because first bullet is “should” sentence and second bullet is “shall sentence. Both are meaningful.

A STA that sends a control frame in response to a non-HT or non-HT duplicate frame with a non-signaling

TA:

— Should set the TXVECTOR parameter CH_BANDWIDTH to the same value as the RXVECTOR

parameter CH_BANDWIDTH for the frame eliciting the response.

— Shall not set the TXVECTOR parameter CH_BANDWIDTH to a value greater than the RXVECTOR

parameter CH_BANDWIDTH for the frame eliciting the response.

Proposed Resolution:

REJECT

| CID | Commenter Name | Subclause | Page | Line | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2266 | Edgar, Richard | 9.7.9 | 74 | 51 | T | This regulates against implementations [non-VHT] outside the scope of this draft, including implementations pre-dating it. | Remove this statement. |

Discussion:

In Draft 1.0, sub-clause 9.7.9 was a channel width selection for non-VHT STAs.

But, in 11/926r1, for resolving CID 2200, sub-clause 9.7.9 has been changed to a channel width selection in non-HT and non-HT duplicate PPDUs.

Because the updated text is specifying the channel width selection for VHT STA. It is in the scope of TGac draft.

Proposed Resolution:

REJECT.

| CID | Commenter Name | Subclause | Page | Line | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2943 | LU, KAIYING | 9.7.5.6 | 74 | 41 | T | A VHT STA that sends a control response frame that is not a CTS in response to a non-HT or non-HT duplicate format frame with the Individual/Group bit in the TA field equal to 1, shall set the I/G bit inthe RA field of the response frame to "0" | add a general description about responding to a control frame iin p74/l41 "....TXVECTOR parameter CH_BANDWIDTH_IN_NON_HT for the frame eliciting the response.,and shall set the RA field to the MAC address obtained from the TA field of the frame eliciting the response with the Individual/Group bit set to 0." |

Discussion:

Signaling TA is defined as the following.

“signaling TA: the IEEE MAC individual address of the STA that has transmitted, onto the WM, the MPDU contained in the frame body field, but with the Individual/Group bit changed to 1.”

Commenter asks to add when the signaling RA is used.

However, since a signaling RA is not defined and does not exist in TGac draft, it is not necessary to add any description related to signaling RA usage.

Proposed Resolution:

REJECT

| CID | Commenter Name | Subclause | Page | Line | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3444 | Shapira, Nir | 9.7.5.5 | 73 | 61 | T | This section should probably apply to VHT single MPDU, so a restriction similar to the one in 9.7.5.1 should be applied here. Also, every occurrence of HT PPDU in this section should probably apply to VHT PPDU as well | Add in the end of 9.7.5.5.1: "that does not contain a VHT single PPDU". Replace each occurrence of "HT PPDU" by "HT or VHT PPDU" |

Discussion:

Comment is correct. This section should applies to  VHT single PPDU.

Add in the end of 9.7.5.5.1 the following:  “that does not contain a VHT single PPDU”.

But, 11/1439r1 already replaced “HT PPDU” by HT or VHT PPDU”

Proposed Resolution:

AGREE

## Editing instructions:

TGac editor: Modify clause 9.7.6.5.1 as the following:

9.7.6.5 Rate selection for control response frames

9.7.6.5.1 Introduction

Subclauses 9.7.6.5.2 through 9.7.6.5.5 describe the rate selection rules for control response frames that are not carried in an A-MPDU that does not contain a VHT single MPDU.