IEEE P802.11
Wireless LANs

| Text Proposal for FILS IP configuration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-01-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Jae-Hyung Song | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-10-5778-0915 | jaehyung.song@lge.com |
| Jeongki Kim | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-7912 | jeongki.kim@lge.com |
| Jihyun Lee | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-1860 | Jihyun1220.lee@lge.com |
| Yongho Seok | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-1947 | Yongho.seok@lge.com |

Abstract

Submission for P802.11ai draft text.  This document contains a proposal to allow 802.11 WLAN STAs to reduce the initial link set up time through an enhanced IP configuration procedure, the procedure that was presented in 11-11/0027r0.

The baseline of this text is P802.11REVmb_D12.0.

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are auctioned in the TGai Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGai Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGai Editor: Editing instructions preceded by “TGai Editor” are instructions to the TGai editor to modify existing material in the TGai draft.  As a result of adopting the changes, the TGai editor will execute the instructions rather than copy them to the TGai Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

Proposal:

Accept the following editing instructions and include the material in the initial TGai Draft.

TGai Editor: Insert the following text in the TGai initial baseline draft, as shown:

8.3.3.2 Beacon frame format

Insert the following row (ignoring the header row) in Table 8-20 Beacon frame body in the correct position to preserve ordering by the “Order” column.

Table 8-26 – Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| <ANA> | Subnet Prefix | The Subnet Prefix information element may be present if dot11FILSActivated is true. |

8.3.3.10 Probe Response frame format

Insert the following row (ignoring the header row) in Table 8-27 Probe Response frame body in the correct position to preserve ordering by the “Order” column.

Table 8-27 – Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| <ANA> | Subnet Prefix | The Subnet Prefix information element may be present if dot11FILSActivated is true. |

8.4.2.1 General

Insert the following row (ignoring the header row) in Table 8-54 Element IDs in the correct position to preserve ordering by the “Element ID” column.

Table 8-54 – Element IDs

| Element | Element ID | Length of indicated element (in octets) | Extensible |
| --- | --- | --- | --- |
| Subnet Prefix (See 8.4.2.ai1) | <ANA> | 5 to 132 |  |

Insert the following new subclauses after the last subclause of 8.4.2:

8.4.2.ai1 Subnet Prefix element

The Subnet Prefix element contains the network prefix of the subnet to which AP belongs. The format of the Subnet Prefix element is shown in Figure 8-ai1. See 10.aii.1 for details.

|  | Element ID | Length | Prefix Type | Prefix Length | Subnet Prefix |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | Variable |

Figure 8-ai1 – Subnet Prefix element format

The Length field indicates the length of the remaining fields in octets, and the value is variable.

The Prefix Type field is set to 1 when the subnet prefix is taken from IPv6, and is set to 0 when the subnet prefix is taken from IPv4.

The Prefix Length is set to the number of bits set in the Subnet Prefix field.

The Subnet Prefix is set to the network prefix of the subnet to which the AP belongs.

Insert the following new subclauses after the last subclause of section 10:

10.ai1 Fast initial link setup procedure

10.ai1.1 IP configuration in ESS

If dot11FILSActivated is true, the STA shall include the Subnet Prefix element in the Beacon frame or Probe Response frame. Otherwise, the STA shall not include the Subnet Prefix element in the Beacon frame or Probe Response frame.

A STA has to perform the entire IP configuration process, when it attempts to create a connection with an AP. If the STA knows the subnet, to which the AP belongs, ahead of the association, the process of IP configuration can be simplified. If the subnet has not been changed, the STA may reuse the old IP configuration, if and only if that specific IP configuration is not assigned to another STA in the subnet.

While scanning, the STA may find several candidate APs with different subnet prefix. The STA is likely to choose the one in the same subnet in which its old AP resides. By doing so, the STA may not perform DHCP discovery and DHCP offer process.