IEEE P802.11
Wireless LANs

| 802.11 TGac WG Letter Ballot LB187 Proposed resolutions to CIDs 4147, 4686, 4267, 4268 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-03-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Youhan Kim | Qualcomm | 1700 Technology Drive San Jose, CA 95110 |  | youhank@qca.qualcomm.com |

##### Comments are based on 11ac D2.0.  Proposed resolutions are based on 11ac D2.0. Changes indicated by a mixture of Word track-changes and instructions. For equation changes, Latex notation is sometimes used. E.g. a_{xyz}^b denotes axyzb

Following CID is covered in this document:

MAC: 4147, 4686, 4267, 4268

R1: Updated based upon discussion during ad hoc.

| CID | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 4147 | 3.24 | 3.2 | The PPDU is transmitted using a 40 MHz transmit spectral mask defined in Clause 20 or Clause 22. Right? | Add the reference to Clause 22 ... |
| 4686 | 3.24 | 3.2 | Will a VHT 40MHz PPDU be sent by using HT spectral mask? | Clarify it. |
| 4267 | 3.25 | 3.2 | clause 20 works for 1)-3) but not 4)-5) | refer to VHT clause for 4) and 5) |

Discussion:

Context:

Figure 22-1 (Interaction of Clause 22 primitives with Clause 18 and 20 primitives) shows that when transmitted from a VHT STA, VHT PPDUs use the Clause 22 masks, HT PPDUs use the Clause 20 masks, 11a PPDUs use Clause 18 mask, and non-HT duplicate PPDUs use Clause 22 masks.  Hence, the commenters are correct that 4) and 5) should be subject to the Clause 22 masks.

Proposed Resolution:

CID 4147, 4686, 4267:

REVISE.  The mask to be used is a function of both the STA type and PPDU format.  Make changes as specified at the end of 12/0318r0.

Proposed Text Change:

See consolidated text change at the end of this document.

| CID | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 4268 | 3.24 | 3.2 | "The PPDU is transmitted ..." implies it is extra spurious info, when in fact it is core | Replace "The PPDU is txed" by "that is transmitted". Do this 4x for 40/80/160/80+80 definitions |

Proposed Resolution:

REVISE.  For 40 MHz, CID 4147, 4686 and 4267 moves the sentence under question to a different location, making the comment obsolete.  For 80/160/80+80 MHz, we can make the condition more obviously “core” by rewording.  Make changes as specified at the end of 12/0318r0.

Proposed Text Change:

Change P3L5 as follows:

40 MHz mask physical layer convergence procedure (PLCP) protocol data unit (PPDU): One of the following

PPDUs:

- a 40 MHz high-throughput (HT) PPDU (TXVECTOR parameter CH_BANDWIDTH equal to HT_CBW40) transmitted using the 40 MHz transmit spectral mask defined in Clause 20 (High Throughput (HT) PHY specification);

- a 40 MHz non-HT duplicate PPDU (TXVECTOR parameter CH_BANDWIDTH equal to NON_HT_CBW40) transmitted using the 40 MHz transmit spectral mask defined in

  - Clause 20 (High Throughput (HT) PHY specification) when transmitted by a non-VHT STA

  - Clause 22 (Very High Throughput (VHT) PHY specification) when transmitted by a VHT STA; or

- a Clause 20 (High Throughput (HT) PHY specification) 20 MHz HT PPDU with the TXVECTOR parameter CH_BANDWIDTH equal to HT_CBW20 and the CH_OFFSET parameter equal to either CH_OFF_20U or CH_OFF_20L. transmitted using the 40 MHz transmit spectral mask defined in Clause 20 (High Throughput (HT) PHY specification);

- a Clause 22 (Very High Throughput (VHT) PHY specification) 20MHz VHT PPDU with the TXVECTOR parameter CH_BANDWIDTH equal to CBW20 transmitted using the 40 MHz transmit spectral mask defined in Clause 22 (Very High Throughput (VHT) PHY specification); or

- a 40 MHz very high throughput (VHT) PPDU with the TXVECTOR parameter CH_BANDWIDTH equal to CBW40 transmitted using the 40 MHz transmit spectral mask defined in Clause 22 (Very High Throughput (VHT) PHY specification).

Change P4L9 as follows:

80 MHz mask physical layer convergence procedure (PLCP) protocol data unit (PPDU): A PPDU that is transmitted using the 80 MHz transmit spectral mask defined in Clause 22 and that is one of the following:

Delete P4L21:

Change P4L25 as follows:

160 MHz mask physical layer convergence procedure (PLCP) protocol data unit (PPDU): A PPDU that is transmitted using the 160 MHz transmit spectral mask defined in Clause 22 and that is one of the following:

Delete P4L40:

Change P4L45 as follows:

80+80 MHz mask physical layer convergence procedure (PLCP) protocol data unit (PPDU): A PPDU that is transmitted using the 80+80 MHz transmit spectral mask defined in Clause 22 and that is one of the following:

Delete P4L53: