IEEE P802.11
Wireless LANs

| Comment Resolution for Sub-clause 8.4.2.163 and 8.4.2.165 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-03-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Yongho Seok | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea | +82-31-450-1497 | yongho.seok@lge.com |
| Seunghee Han | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea |  |  |

Abstract

Submission for Draft P802.11ac_D2.0 comment resolution. The document addresses various comments related to sub-clause 8.4.2.163 and 8.4.2.165.

Submission addresses Draft P802.11ac_D2.0 comments 4331, 5259, 4333, 4332, 4334, 4338, 4340, 4341 and 4805.

Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGac Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGac Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGac Editor:  Editing instructions preceded by “TGac Editor” are instructions to the TGac editor to modify existing material in the TGac draft.   As a result of adopting the changes, the TGac editor will execute the instructions rather than copy them to the TGac Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

| CID | Clause | Page | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 4331 | 8.4.2.163 | 74.43 | Element ID and Length fields not defined | Define |

Discussion:

Wide Bandwidth Channel Switch element is missing the description about the element ID and length fields.

Proposed Resolution:

Revised.

Proposed Text Changes:

Add the following texts in P74L60,

“The Element ID field is set to the value for the Wide Bandwidth Channel Switch element in Table 8-54.

The value of the Length field is set to 3.”

| CID | Clause | Page | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 5259 | 8.4.2.163 | 74.44 | "The Wide Bandwidth Channel Switch element is included in Channel Switch Announcement frames, as described in 8.5.2.6" And also in other frames, as defined in 10.38.1 | Indicate in which other frames it is present |

Discussion:

Wide Bandwidth Channel Switch element is also included in TDLS Channel Switch Request frame.

Proposed Resolution:

Revised.

Proposed Text Changes:

Modify P74L44 as the following:

“The Wide Bandwidth Channel Switch element is included in Channel Switch Announcement frames, as described in 8.5.2.6 (Channel Switch Announcement frame format), and TDLS Channel Switch Request frames, as described in 8.5.13.7 (Channel Switch Announcement frame format).”

| CID | Clause | Page | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 4333 | 8.4.2.163 | 74.51 | Why "New *STA* Ch Width" - all the BSS is changing | Rename: e.g. New Channel Width |

Discussion:

Wide Bandwidth Channel Switch element is used to change the channel width of the BSS.

Proposed Resolution:

Accepted. Replace the “STA Channel Width” with “Channel Width” in sub-clause 8.4.2.163, as the following. .

“The subfields New STA Channel Width, New Channel Center Frequency Segment 1 and New Channel Center Frequency Segment 2 have the same definition, respectively, as STA Channel Width, Channel Center Frequency Segment 1 and Channel Center Frequency Segment 2 in the VHT Operation Information element, described in Table 8-183w (VHT Operation Information subfields).”

| CID | Clause | Page | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 4332 | 8.4.2.163 | 74.56 | No need to specify where something is used - and that kind of reference gets stale fast. And to be properly useful this needs to be allowed in ECSA frames, beacons and probe responses. | Remove where used |

Discussion:

Although it may be no harmful removing the relevances, keeping them would be also helpful for consistency.

Proposed Resolution:

Rejected.

| CID | Clause | Page | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 4334 | 8.4.2.163 | 74.62 | Given 8.5.2.6, the 20/40 MHz value should be reserved is this element - i.e. same definition but with refinements | Indicate same definition but with another reserved/unused value |

Discussion:

In 8.5.2.6, the Wide Bandwidth Channel Switch element is present when switching to a channel width wider than 40 MHz. It means that Channel Width field in Wide Bandwidth Channel Switch element is not able to set to 0. The value 0 is defined for 20MHz or 40MHz operating channel.

Proposed Resolution:

Revised. For indicating that the Channel Width field in Wide Bandwidth Channel Switch element is used for switching to a channel width wider than 40 MHz, change the value 0 in Channel Width field to the reserved value.

Proposed Text Changes:

Modify P75L62 as the following:

“The subfields New STA Channel Width, New Channel Center Frequency Segment 1 and New Channel Center Frequency Segment 2 have the same definition, respectively, as STA Channel Width, Channel Center Frequency Segment 1 and Channel Center Frequency Segment 2 in the VHT Operation Information element, described in Table 8-183w (VHT Operation Information subfields). But, the New Channel Width value for 20MHz or 40MHz operating channel width is reserved, because the Wide Bandwidth Channel Switch element is used for switching to a channel width wider than 40 MHz.”

| CID | Clause | Page | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 4338 | 8.4.2.165 | 75.62 | Element ID not defined | Define |

Discussion:

Extended Power Constraint element is missing the description about the element ID.

Proposed Resolution:

Revised.

Proposed Text Changes:

Add the following sentence in P75L62:

“The Element ID field is set to the value for the Extended Power Constraint element in Table 8-54.”

| CID | Clause | Page | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 4340 | 8.4.2.165 | 76.02 | "The encoding for 80 MHz channel width is always present ... VHT BSS" | Ummm ... VHT APs have to be 80 Mz capable but are perfectly allowed to start 20 or 40 MHz BSSs if circumstances dictate. E.g. ch169! Delete this sentence! |

Discussion:

The operation can be done within the capability.

Proposed Resolution:

Accepted.

Proposed Text Changes:

Delete the following sentence in P76L02:

“The encoding for 80 MHz channel width is always present when operating in a VHT BSS.”

| CID | Clause | Page | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 4341 | 8.4.2.165 | 76.13 | No need to specify where something is used - and that kind of reference gets stale fast. And any inclusion is subject to more detailed rules as specified in the beacon section | Remove where used |

Discussion:

Although it may be no harmful removing the relevances, keeping them would be also helpful for consistency.

Proposed Resolution:

Rejected.

| CID | Clause | Page | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 4805 | 8.4.2.165 | 76.01 | If Extended Power Constraint is allowed to carry information for 20 and 40 MHz channel width, there is an apparent conflict with the (non-Extended) Power Constraint element | Either add "except that the encoding 0 is reserved", or otherwise clarify |

Discussion:

As a matter of fact, there was a similar comment in IEEE 802.11-11/1543r1. The PRE-MOTION#7 as in IEEE 802.11-11/1590r0 was approved in November 2011. Below is captured from doc1543.

Proposed Resolution:

Revised. Reflect PRE-MOTION#7 in IEEE 802.11-11/1590r0.

Proposed Text Changes:

Replace P76L01 as follows:

The Channel Width field uses the same encoding as the STA Channel Width field of the VHT Operation element (see 8.4.2.161 (VHT Operation element)). The encoding for 20 MHz or 40 MHz operating channel width is not used. The power constraint for these operating channel widths is already specified in the Power Constraint element.