IEEE P802.11
Wireless LANs

| White Space Map for UK |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-03-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Yongho Seok | LG Electronics | Mobile Comm. Lab, LG R&D Complex 533, Hogye1, Dongan, Anyang, Korea | +82-31-450-1947 | yongho.seok@lge.com |
| Wookbong Lee | LG Electronics | Mobile Comm. Lab, LG R&D Complex 533, Hogye1, Dongan, Anyang, Korea | +82-31-450-1883 | wookbong.lee@lge.com |

Abstract

Proposed resolutions for CID 948. Editing instructions are based on P802.11af Draft 1.06.

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGaf Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGaf Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGaf Editor: Editing instructions preceded by “TGaf Editor” are instructions to the TGaf editor to modify existing material in the TGaf draft.  As a result of adopting the changes, the TGaf editor will execute the instructions rather than copy them to the TGaf Draft.

| CID | Clause Number(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 948 | E.3.1.5 | "If the Device Type value is set to 0, the Validity field is not present." Why is the validity information not provided to Personal/Portable non-AP STA? Provide the validity information to Personal/Portable non-AP STA. | as per comment | Rejected for CID 948 based on the discussion in 11-11/1405r3. The validity of WSM is controlled by the reception of CVS frames in Personal/Portable non-AP STAs. |

Discussion:

In “Draft regulatory requirements for white space devices in the UHF TV band” updated on 16, Feb, 2012, the OFCOM asks to provide a single time validity for the parameters to the a master WSD and a slave WSD.

Also, the OFCOM provides the definition of the channel number and the maximum power level for UK.

Propose Revised for CID 948, adding Channel Number and Maximum Power Level for UK discussion and editing instructions in 11-12/0383r0.

TGaf Editor: Change Table 8-14b Device Class definition value fields as follows:

| Name | Type | Length (octets) | Value | Scope |
| --- | --- | --- | --- | --- |
| Device Class | <ANA> | 1 | The Device Class field is set to an integer that specifies the device’s TVWS band mode of operation as follows: 0: Personal/portable non-AP STA, GDCnonAPSTA, 1: Personal/portable AP STA, GDCAP, 2: Fixed STA, GDCFixedSTA, 3-255: Reserved. | CAQ |

8.2.6.1.2 Device Identification Information

TGaf Editor: Change Table 8-14d Device Identification Information Value field as follows:

| Name | Length (octets) | Value | Scope |
| --- | --- | --- | --- |
| Device Serial Number | 4 | The Device Serial Number field is set to the manufacturer's serial number. This field is present only if the Device Class field (Table 8-14b (Device Class definition)) is equal to 1 (Personal/portable AP STAGDCAP) or 2 (Fixed STAGDCFixedSTA); otherwise, this field is not present. | CAQ, US |

8.2.6.1.6 WSM Information Values

TGaf Editor: Change second paragraph of 8.2.6.1.6 as follows:

The Device Class field is defined in 8.2.6.1.1 (Device class). The Device Class field is set to a value identifying the Device Class used by the WSM and determines the length of the channel availability tuple consisting of the channel number, the maximum power level and the validity fields, which is repeated as the Length field of WSM element. If the Device Class field is 0, the Validity field in Channel Availability is not present. Otherwise, the Validity field shall exist in the Channel Availability field.

TGaf Editor: Change Table 8-14k WSM information value fields as follows:

| Name | Length (octets) | Value | Scope |
| --- | --- | --- | --- |
| Device Class | 1 | The Device Class field is defined in 8.2.6.1.1. The Device Class field is set to a value identifying the Device Class used by the WSM as follows: 0: GDCnonAPSTA, 1: GDCAP, 2: GDCFixedSTA, 3-255: Reserved. Single TLV comprised of fields in Table 8-14b (Device Class definition). | WSM,US |
| Map ID | 1 | Bit 0: Type indicates whether the following channel list is a full channel list or a partial channel list. If the Type bit is set to 1, the following channel list is a full channel list and if the Type bit is set to 0, the following channel list is a partial channel list. Bits 1-7: Map version identifies the version of the WSM. | WSM, US |
| Channel Number | 1 | The Channel Number field is a positive integer value as defined in Annex D.1 (External regulatory references) that indicates the available TV channel for WLAN operation. The length of the Channel Number field is 1 octet. When the Channel Number and Maximum Power Level pairs are repeated, they shall be listed in ascending TV channel order. | WSM, US |
| Channel Number | 3 | The Channel Number field is three positive integer values (k, n, m) that indicates the available lower and upper frequency boundaries for WLAN operation. A lower frequency is specified as (470 + 8k + 0.2n) MHz, with the corresponding upper frequency specified as (470 + 8k + 0.2m) MHz, where 0 ≤ k ≤ 39, 0 ≤ n ≤ 39, 1 ≤ m ≤ 40, and n < m. The length of the Permissible Frequency field is 3 octets. When the Permissible Frequency and Maximum Power Level pairs are repeated, they shall be listed in ascending TV channel order. | WSM, UK |
| Maximum Power Level | 1 | The Maximum Transmit Power Level field indicates the maximum power, in units of 0.5 dBm, allowed to be transmitted on the Channel Number. | WSM, US |
| Maximum Power Level | 1 | A maximum permitted EIRP spectral density, specified in units of dBm/(0.2 MHz), between each lower frequency boundary and its corresponding upper frequency boundary. | WSM, UK |
| Validity (Optional) | 1 | The Validity field indicates the time duration in minutes for which the Channel Number is available with the allowed Maximum Power Level, where the Validity is provided for each available Channel Number. If the Device Class value is set to 0, the Validity field is not present. If else, the Validity field for each Channel Number shall be provided. | WSM, US, UK |

NOTE—In US, an example of full Map 1 for non-AP STA describing two available Personal/Portable TV channels with power limits of 100 mW and 40 mW is shown as: <ANA>, 0x06, 0x00, 0x03, 0x15, 0x17, 0x33, 0x13.

Type is <ANA>, Length is 0x06, Device Class is 0x00, a full map with MapID 1 is 0x03, TV channel 21 is 0x15, 20 dBm Maximum Power Level is 0x17, TV channel 51 is 0x33, 16 dBm Maximum Power Level is 0x13.

In UK, an example of full Map 1 for non-AP STA describing two available Personal/Portable TV channels with power limits of 100 mW during 60 minutes and 40 mW during 90 minutes is shown as: <ANA>, 0x0C, 0x00, 0x03, 0x05, 0x0A, 0x1E, 0x17, 0x3C, 0x09, 0x00, 0x28, 0x13, 0x5A.

Type is <ANA>, Length is 0x0C, Device Class is 0x00, a full map with MapID 1 is 0x03, TV channel 26 (512MHz~516MHz) is 0x05, 0x0A and 0x1E, 20 dBm Maximum Power Level is 0x17, 60 minutes validy time is 0x3C, TV channel 30 (542MHz~550MHz) is 0x09, 0x00 and 0x28, 16 dBm Maximum Power Level is 0x13, 90 minutes validy time is 0x5A.