IEEE P802.11
Wireless LANs

| 802.11 TGac WG Letter Ballot LB187 Proposed resolution to CID 4475 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-04-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian STEPHENS | Intel Corporation | 64, CB24 8TA, U.K. |  | Adrian.p.stephens@intel.com |

Abstract

This submission contains proposed comment resolutions to comments received during WG letter ballot 187.

The comment included is CID 4475,  which is owned by the MAC ad-hoc.

The comment is as follows:

| CID | Page | Clause | Comment | Proposed Change | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- |
| 4475 |  | 10.3 | Your baseline includes 11ad, which has made many changes to this subclause | Apply the changes from your baseline | MAC |

Proposed Resolution:

Revised.  Make changes as shown in <this document>.

Make the following changes:

- STA authentication and association - Association, reassociation, and disassociation - PCP/AP association receipt procedures Change the 2nd paragraph by inserting a lettered list element h) following element g) and re-lettering subsequent elements as follows: Upon receipt of an Association Request frame from a non-PCP/non-AP STA for which the state is State 2, State 3, or State 4, the AP's MLME shall associate with the non-PCP/non-AP STA using the following procedure: - The SME shall refuse an association request from an HT STA that does not support all the MCSs in the BSSBasicMCSSet parameter. - The SME shall refuse an association request from a VHT STA that does not support all the MCSs in the VHTBSSBasicMCSSet parameter.(#3182) - PCP/AP reassociation receipt procedures Change the 2nd paragraph by inserting a lettered list element g) following element f) and re-lettering subsequent elements as follows: Upon receipt of an Association Request frame from a non-AP STA for which the state is State 2, State 3, or State 4, the PCP/AP's MLME shall associate with the non-AP STA using the following procedure: f) The SME shall refuse an association request from an HT STA that does not support all the MCSs in the BSSBasicMCSSet parameter. g) The SME shall refuse a reassociation request from a VHT STA that does not support all the MCSs in the VHTBSSBasicMCSSet parameter.(#3182)