IEEE P802.11
Wireless LANs

| 802.11 TGac WG Letter Ballot LB187 Proposed resolutions to some miscellaneous MAC comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-05-02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian STEPHENS | Intel Corporation | 64, CB24 8TA, U.K. |  | Adrian.p.stephens@intel.com |

Abstract

This submission contains proposed comment resolutions to comments received during WG letter ballot 187.

The comments included are : 4780,  5240,  4793,  4760,  5243,  5308,  4709,  4821,  4541,  4475,  4546,  4470,  4847

| CID | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 4780 |  |  | "non-A-MPDU" is explicitly (though confusingly) defined in clause 3.2 asan MPDU which is either not in an A-MPDU or is in an A-MPDU but is aVHT single MPDU. However the spec is not consistent in using this | 8.2.4.5.4: Change "When not carried in an A-MPDU subframe or carried in a VHT single MPDU" back to "In a frame that is a non-A-MPDU frame".9.12.8: Change "The rules for VHT single MPDU operation are the same as the rules for non-A-MPDU operation even though the MPDU is carried in an A-MPDU." to "The rules for VHT single MPDU operation are the same as the rules for operation with other types of non-A-MPDU frame."9.25.4: Change "may transmit a +CF-ACK non-A-MPDU frame or VHT single MPDU" to "may transmit a +CF-ACK non-A-MPDU frame", and check the following "non-A-MPDU" is correct, i.e. that it is intended to cover VHT single QoS Data+HTC MPDUs.9.31.1: Clarify whether "non-A-MPDU" is really intended to include VHT single MPDUs here. |

Changes proposed by commenter:

8.2.4.5.4 Ack Policy subfield

| - Ack Policy subfield in QoS Control field of QoS data frames |  |  |
| --- | --- | --- |
| Bits in QoS Control field |  | Meaning |
| Bit 5 | Bit 6 |  |
| 0 | 0 | Normal Ack or Implicit Block Ack Request. In a frame that is a non-A-MPDU frameIn a frame that is a non-A-MPDU frame: The addressed recipient returns an ACK or QoS +CF-Ack frame after a short interframe space (SIFS) period, according to the procedures defined in 9.3.2.9 (ACK procedure) and 9.19.3.5 (HCCA transfer rules). For QoS Null (no data) frames, this is the only permissible value for the Ack Policy subfield.(#2759) In a frame that is part of an A-MPDUOtherwise: The addressed recipient returns a BlockAck MPDU, either individually or as part of an A-MPDU starting a SIFS after the PPDU carrying the frame, according to the procedures defined in 9.3.2.10 (BlockAck procedure), 9.20.7.5 (Generation and transmission of BlockAck by an HT STA), 9.20.8.3 (Operation of HT-delayed Block Ack), 9.24.3 (Rules for RD initiator), 9.24.4 (Rules for RD responder) and 9.28.3 (Explicit feedback beamforming) |

9.12.8 Transport of VHT single MPDUs

The rules for VHT single MPDU operation are the same as the rules for non-A-MPDU operation

with other types of non-A-MPDU frame.

9.25.4 Rules for responder

An RD responder in the OBand(11ad) may transmit a +CF-ACK non-A-MPDU frame in response to a non-A-MPDU QoS Data+HTC MPDU that has the Ack Policy field set to Normal Ack and the RDG/More PPDU subfield set to 1.

(commenter states: “and check the following "non-A-MPDU" is correct, i.e. that it is intended to cover VHT single QoS Data+HTC MPDUs.”)

9.31.1 NDP rules

An NDP sequence contains at least one non-NDP PPDU and at least one HT NDP PPDU. Only one PPDU in the NDP sequence may contain an NDP announcement. An NDP sequence begins with an NDP announcement. The NDP sequence ends at the end of the transmission of the last HT NDP PPDU that is announced by the NDP announcement. A STA that transmits the first PPDU of an NDP sequence is the NDP sequence owner. In the NDP sequence, only PPDUs carrying HT NDP and PPDUs carrying non-A-MPDU control frames may follow the NDP sequence’s starting PPDU.

(Commenter states: “Clarify whether "non-A-MPDU" is really intended to include VHT single MPDUs here”)

Proposed resolution:

Revised.

Make changes as indicated by the commenter.

In reply to: “and check the following "non-A-MPDU" is correct, i.e. that it is intended to cover VHT single QoS Data+HTC MPDUs”.  The following non-A-MPDU is correct, i.e., there is no restriction against using a VHT single MPDU in this context.

In reply to: “Clarify whether "non-A-MPDU" is really intended to include VHT single MPDUs here”,  there is no need to constrain the use of VHT single MPDU here.  This sequence will only take place when initiated by an HT beamformer.  As such, a VHT beamformee cannot transmit a VHT single MPDU to the beamformer, and there is no need to add any additional constraint at this point.

| 5240 | 31.12 | 8.2.4.6.3 | Table 8-13a."The STBC Indication subfields contains:Set to 0 if STBC is not transmittedSet to 1 if STBC is transmitted""transmitted" is not well defined; | add something like: "STBC Indication is set to 1 if the frame from which the MFB was compouted used STBC." |
| --- | --- | --- | --- | --- |

Commenter’s Proposed change:

| MSI/STBC(#3749) | MRQ sequence identifier/STBC indication | If the Unsolicited MFB subfield is 0(#3749) and the MRQ subfield is(#3354) 1, the MSI/STBC subfield contains a sequence number in the range 0 to 6 that identifies the specific request.(#3749) If the Unsolicited MFB subfield is 0(#3749) and the MRQ subfield is(#3354) 0, the MSI/STBC subfield is reserved. If the Unsolicited MFB subfield is 1, the MSI/STBC field contains the Compressed MSI and STBC Indication subfields as shown in Error: Reference source not found. The STBC Indication subfields contains: Set to 0 if STBC is not transmitted Set to 1 if STBC if the frame from which the MFB was computed used STBC. The Compressed MSI contains a sequence number that identifies the specific request. It is in the range 0 to 3 if STBC Indication equals 0 or in the range 0 to 2 if STBC Indication equals 1.(#3749) |
| --- | --- | --- |

Proposed change:

| MSI/STBC(#3749) | MRQ sequence identifier/STBC indication | If the Unsolicited MFB subfield is 0(#3749) and the MRQ subfield is(#3354) 1, the MSI/STBC subfield contains a sequence number in the range 0 to 6 that identifies the specific request.(#3749) If the Unsolicited MFB subfield is 0(#3749) and the MRQ subfield is(#3354) 0, the MSI/STBC subfield is reserved. If the Unsolicited MFB subfield is 1, the MSI/STBC field contains the Compressed MSI and STBC Indication subfields as shown in Error: Reference source not found. The STBC Indication subfields contains: Set to 1 if STBC if the PPDU from which the MFB was computed used STBC. Set to 0 otherwise. The Compressed MSI contains a sequence number that identifies the specific request. It is in the range 0 to 3 if STBC Indication equals 0 or in the range 0 to 2 if STBC Indication equals 1.(#3749) |
| --- | --- | --- |

Proposed resolution:

Revised.  At cited location  change : “Set to 0 if STBC is not transmitted

Set to 1 if STBC is transmitted” to

“Set to 1 if STBC if the PPDU from which the MFB was computed used STBC.

Set to 0 otherwise.”

| 4793 | 36.00 | 8.3.2.1 | The old note needs to be amended | Change the new proposed NOTE to a NOTE1 and renumber the existing NOTE to a NOTE2, changing it to say "The maximum frame body size for a data frame carried in a non-VHT PPDU is 7951 octets for CCMP encryption of a maximum-size A-MSDU (note that TKIP encryption is not allowed in this case and any Mesh Control fields are part of the A-MSDU subframes). The maximum frame body size if A-MSDUs are not used is 2338 octets for CCMP encryption of a maximum-size MSDU and 2342 octets for TKIP encryption of a maximum-size MSDU, including in both cases an 18-octet Mesh Control field. The frame body size might in all these cases be greater if a vendor-specific cipher suite is used." |
| --- | --- | --- | --- | --- |

Context (D2 p 36): + proposed change

- Data frame format Change Data frame as shown (changing Frame Body field size range to 0-11424 and inserting the note that follows):(#2087) Octets: 2; 2; 6; 6; 6; 2; 6; 2; 4; 0-7599 0-11424; 4. Frame Control; Duration/ ID; Address 1; Address 2; Address 3; Sequence Control; Address 4; QoS Control; HT Control; Frame Body; FCS. MAC Header. - Data frame. NOTE 1 — The maximum Frame Body size (11 424 octets) is arrived at by subtracting the length of the shortest QoS Data frame MAC header (26 octets) and FCS from the maximum MPDU length of 11 454 octets. NOTE 2—The maximum Frame Body size for a data frame carried in a non-VHT PPDU is 7951 octets for CCMP encryption of a maximum-size A-MSDU (note that TKIP encryption is not allowed in this case and any Mesh Control fields are part of the A-MSDU subframes). The maximum frame body size if A-MSDUs are not used is 2338 octets for CCMP encryption of a maximum-size MSDU and 2342 octets for TKIP encryption of a maximum-size MSDU, including in both cases an 18-octet Mesh Control field. The frame body size might in all these cases be greater if a vendor-specific cipher suite is used.

Proposed Resolution:

Accepted

| 4760 | 69.06 | 8.4.2.160.2 | The information about which MMPDUs given IEs are present in is, and should be, in 8.3 and should not be duplicated in 8.4 | Delete "in the VHT Capabilities element included in Beacon, Probe Response, Association Response and Reassociation Response frames" and "in the VHT Capabilities element included in Association Request, Reassociation Request and Probe Request frames" |
| --- | --- | --- | --- | --- |

Commenter’s proposed change: (D2.0 69.06)

When transmitted by a VHT AP : Set to 0 if the VHT AP does not support VHT TXOP Power Save in the BSS. Set to 1 if the VHT AP supports TXOP Power Save in the BSS. When transmitted by a VHT non-AP STA : Set to 0 when the VHT STA is not in TXOP Power Save Mode. Set to 1 when the VHT STA is in TXOP Power Save Mode.

Proposed resolution:

Accepted.

| 5243 | 69.42 | 8.4.2.160.2 | "Set to 0 if Rx antenna pattern might change during association.Set to 1 if Rx antenna pattern does not change during association."Not clear what 'during association' means | mention 'after association' or 'while associated'; also add a reference to the appropriate section |
| --- | --- | --- | --- | --- |

Context (D2):

| Rx Antenna Pattern Consistency | Indicates the possibility of Rx antenna pattern change | Set to 0 if Rx antenna pattern might change during association. Set to 1 if Rx antenna pattern does not change during association.(#3398) |
| --- | --- | --- |
| Tx Antenna Pattern Consistency | Indicates the possibility of Tx antenna pattern change | Set to 0 if Tx antenna pattern might change during association. Set to 1 if Tx antenna pattern does not change during association.(#3398) |

Discussion:

It’s clear that the intended meaning was “during the lifetime of an association”.

Proposed Resolution:

Change “during association” to “during the lifetime of the current association”.  Make this change at p69 lines 43, 46, 49 and 51.

Add “See 10.38.6.” after lines  46 and 51.

| 5308 | 73.49 | 8.4.2.162 | Even though at P72L54, it is stated that the Extended BSS Load element reports utilization information "by MU capable STAs", it is not clear from the definitions of "VHT [40/80/160] MHz Utilization" whether the equations are also applicable to SU. | Please insert the following paragraph at P73L49, prior to the paragraph starting with "The VHT 40 MHz Utilization field is defined":?The utilization of the VHT channels by MU-capable STAs is determined by the equations defined in the following paragraphs.? |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.  From the equations at 73.55, 73.64 and 74.07, and the definition at 74.19 it is clear that the [VHT] xx MHz Utilization fields include both SU and MU transmissions.   The proposed addition is in conflict with the existing text.

| 4709 | 77.11 | 8.5 | Earlier versions of the spec (e.g. mb/D8.0) included a restriction on the number of elements in certain MMPDUs. This restriction was deleted from the text by mb/D12.0 because it was covered by a catch-all "The maximum MMPDU size is 2304 octets". However, the maximum MMPDU size is now more than 2304 octets | Add back sentences of the form "The number and length of the Foo elements in aBar frame is limited to 2304 octets." in Diagnostic Request, Diagnostic Report, BSS Transition Management Query, BSS Transition Management Request and BSS Transition Management Response frames' description |
| --- | --- | --- | --- | --- |

Rejected.

The commenter does not indicate why such a limitation should be necessary.  It a VHT device is already capable of receiving a longer MMPDU, there is no obvious need to arbitrarily limit the MMPDU lengths of these specific MMPDUs.

| 4821 | 98.42 | 9.7.6.5.3 | "all VHT MCSs and all MCSs that X" is ambiguous | Change "all VHT MCSs and all MCSs that" to "all VHT MCSs and the MCSs that", or even better, put the "all VHT MCSs" as the second term of the Boolean (make this change in a) above too) |
| --- | --- | --- | --- | --- |

Discussion:

98.42 contains the text:  “Eliminate from the CandidateMCSSet all VHT MCSs and all MCSs that have an index that is higher than the index of the MCS of the received frame.”

The issue here is that VHT and HT MCSs mean different things.   VHT does not embed N_SS,  whereas the HT MCS does.

For example, consider the case when the frame eliciting the Response is a 40 MHz HT PPDU N_SS=2,  BPSK, R=0.5.  This is MCS index 8.

The condition:  “all VHT MCSs … that have an index that is higher” eliminates the VHT MCS 9,  but leaves MCSs 0..8,  i.e. up to 324Mbps (N_SS=2, CH=40 MHz).

This comparison is non-sensical.   It also makes no sense except in the case that a VHT STA is responding to a VHT STA has transmitted data using an HT format PPDU.   There is no reason why the responding STA cannot use an HT MCS in this case.

See also at 98.35:

- If the frame eliciting the response is within a non-HT PPDU, - Eliminate from the CandidateMCSSet all VHT MCSs and the MCSs that have a data rate greater than the data rate of the received PPDU (the mapping of MCS to data rate is defined in 20.6). - Find the highest indexed MCS from the CandidateMCSSet. The index of this MCS is the index of the MCS that is the primary MCS for the response transmission. - If the CandidateMCSSet is empty, the primary MCS is the lowest indexed MCS of the mandatory MCSs.

This implies comparison between HT and VHT MCSs,  and HT MCSs with N_SS>1 will win.

Generally any comparison between HT and VHT MCS index values is meaningless.

So we have a choice:

- Reword the MCSs selection rules to allow meaningful comparison between VHT and HT MCSs

  - Compare N_SS first, then compare index offset from start of table for that N_SS.

- Constrian the control response PPDU type so that comparison between VHT and HT MCSs is never necessary.

  - non-HT -> either HT or VHT,  but type of response is known prior to 9.7.6.5.3

  - HT->HT MCSs only considered

  - VHT -> VHT MCSs only considered

Straw Poll:   “which of these do you prefer?”

- Reword MCSs selection rules

- Constrain control response PPDU type

| 4541 | 125.43 | 9.29.2.2 | "in the HT format" is ambiguous -- is the VHT variant of the HT control field in the HT format or not? | It would be much simpler simply to have the HT Control field and the VHT Control field, then drop "variant" and "in the HT format". In that case this addition and the addition on line 52 are unnecessary. |
| --- | --- | --- | --- | --- |

Context: (D2, 125.36)

9.29.2.2 Unidirectional implicit transmit beamforming Insert the following as the first paragraph of this section: This section assumes that the HT Control field is in the HT format. 9.29.2.3 Bidirectional implicit transmit beamforming Insert the following as the first paragraph of this section: This section assumes that the HT Control field is in the HT format. 9.29.2.4 Calibration 9.29.2.4.3 Sounding exchange for calibration Insert the following as the first paragraph of this section: This section assumes that the HT Control field is in the HT format. 9.29.2.4.4 CSI reporting for calibration Insert the following as the first paragraph of this section: This section assumes that the HT Control field is in the HT format.

Discussion “in the HT format” is colloquial language without definition.

We have several ways of fixing this:

- (Option 1) Fix up the language shown above to something unambiguous

  - “For the purpose of this subclause, all references to the HT Control field should be read as referring to the HT variant HT Control field.”

- (Option 2) Add a default interpretation to HT variant and remove any “HT variant” or “HT format” statements elsewhere that are not required for clarity.

  - e.g. at 28.66 add:  “References to the “HT Control field” without any additional qualification implicitly refer to the HT variant of the HT Control field.

- (Option 3) Refer to the HT variant as the HT control field and the VHT variant as the VHT control field throughout.

  - This is probably the cleanest solution, and the most work.

My long-term preference is the last one (Option 3).  Such changes can’t affect implementations because what we’re talking about here is purely terminology.   So we can afford to take our time to get it right.

But we can’t ignore the specific issue of ambiguity raised by the commenter,  so I propose  making minimal changes at this point to satisfy the specific objection raised,  and we can consider a submission  that might propose option 3 in a later ballot.

I also show a proposed resolution for Option 2, should the group prefer to address the commenter’s specific issue with change of smaller scope.

Proposed Resolution (Option 3):  (preferred)

Revised. Agree that the statement is ambiguous.

Replace “in the HT format” with “the HT variant” at 125.43, 125.52, 125.65 and 126.07.

This resolves the ambiguity raised by the commenter.

The commenter does not provide sufficient information to resolve the comment as he or she preferred. A submission from the commenter listing all the places which need changes to change from "HT/VHT variant HT Control" to "HT/VHT Control" terminology would be necessary to achieve his or her preferred outcome.

Proposed Resolution (Option 2): (alternative)

Revised.   Agree that the statement is ambiguous.  This is resolved by defining a default interpretation for the format of the HT control field and removing the cited statement.

At 28.66 add a new para: ‘References to the “HT Control field” without any additional qualification implicitly refer to the HT variant of the HT Control field.’

Delete 125.30 (9.29 Transmit Beamforming) to 126.07.

Delete 121.58 (9.27 Sounding PPDUs) to 121.66.

Delete 122.08-122.12.

At 103.13, change “An HT variant” to “A VHT variant”.

| 4475 | 135.28 | 10.3 | Your baseline includes 11ad, which has made many changes to this subclause | Apply the changes from your baseline |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Revised.  Make changes as shown in <this document>.

Make the following changes:

- STA authentication and association - Association, reassociation, and disassociation - PCP/AP association receipt procedures Change the 2nd paragraph by inserting a lettered list element h) following element g) and re-lettering subsequent elements as follows: Upon receipt of an Association Request frame from a non-PCP/non-AP STA for which the state is State 2, State 3, or State 4, the AP's MLME shall associate with the non-PCP/non-AP STA using the following procedure: - The SME shall refuse an association request from an HT STA that does not support all the MCSs in the BSSBasicMCSSet parameter. - The SME shall refuse an association request from a VHT STA that does not support all the MCSs in the VHTBSSBasicMCSSet parameter.(#3182) - PCP/AP reassociation receipt procedures Change the 2nd paragraph by inserting a lettered list element g) following element f) and re-lettering subsequent elements as follows: Upon receipt of an Association Request frame from a non-AP STA for which the state is State 2, State 3, or State 4, the PCP/AP's MLME shall associate with the non-AP STA using the following procedure: f) The SME shall refuse an association request from an HT STA that does not support all the MCSs in the BSSBasicMCSSet parameter. g) The SME shall refuse a reassociation request from a VHT STA that does not support all the MCSs in the VHTBSSBasicMCSSet parameter.(#3182)

| 4546 | 144.43 | 10.39 | "has a value of true for" sounds like this is an attribute of an AP, not the value of a variable. | Replace "that has a value of true for dot11VHTOptionImplemented" with "whose dot11VHTOptionImplmented attribute is true" and "the STA that has a value of false for dot11VHTOptionImplemented." with "a STA whose dot11VHTOptionImplemented attribute is false." |
| --- | --- | --- | --- | --- |

Context:

Only an AP that has a value of true for dot11VHTOptionImplemented may transmit a Group ID Management frame. A Group ID Management frame shall not be sent to the STA that has a value of false for dot11VHTOptionImplemented.

Discussion:

Agree that “has a value of true for” is awkward.  However the proposed resolution introduces the word “attribute”, which is contrary to WG11 style.  This is the concern of the commenter.

(See also comment 4186, (MU ad-hoc,  assigned to Yongho).  Resolution of this comment is likely to remove the second reference to dot11VHTOptionImplemented in the text shown in context.

Proposed change:

An AP may transmit a Group ID Management

frame only if dot11VHTOptionImplemented is true . A Group ID Management frame shall not be sent to the STA that has a value of false for dot11VHTOptionImplemented.

| 4470 | 144.44 | 10.39 | the STA | a STA |
| --- | --- | --- | --- | --- |

Discussion:

Agree with the change,  but any change that resolves 4186 is likely to touch this text.

Proposed resolution:

Accepted

| 4847 | 144.65 | 10.39 | What is "a valid response"? | Change to "... acknowledged Group ID Management frame transmitted to the STA" |
| --- | --- | --- | --- | --- |

Context:

An MU PPDU shall be transmitted to a STA based on the content of the most recently transmitted Group ID Management frame to the STA with valid response.

Proposed resolution:

Change cited sentence to read:

An MU PPDU shall be transmitted to a STA based on the content of the Group ID Management

frame most recently transmitted to the STA and for which an acknowledgement was received.