IEEE P802.11
Wireless LANs

| LB187 - CID 4775 Comment Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-05-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Wei Shi | CSR | Churchill House, Cambridge, CB4 0WZ, UK |  | wei.shi@csr.com |

Abstract

This document discusses CID 4775 for LB187 and proposes a resolution.

## Comment:

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 4775 |  |  | A non-VHT STA using L_LENGTH to determine the end of the PPDU will only be able to do this with about 4 us accuracy. This is not sufficient accuracy to determine exactly when the end of the SIFS following the PPDU is, i.e. the slot boundary | Add a NOTE to clarify how non-VHT STAs can accurately determine the next slot boundary |  |

## Discussion

The comment says “non-VHT” but it actually applies to any STA that does not understand short GI. The problem raised is where precisely the end of a frame is which is used as a reference for SIFS, slot boundaries etc.

In IEEE Std 802.11-2012 Clause 9.3.2.3.3 states SIFS is the time from the end of the last symbol. Likewise, Clause 9.3.7 also states timings are from the end of the last symbol. So the definition is very clear in Clause 9. However Clause 20.3.20.5 (Packet alignment) defines PHY-TXEND.confirm and PHY-CCA.indication(idle) primitives to be at the 4 us boundary after the end of the last symbol. This implies that SIFS should be measured from the 4 us boundary after the end of the last symbol. So there is ambiguity regarding where SIFS is measured from.

Using the 4 us boundary after the end of the last symbol seems correct to me for both HT-MF and VHT where the Length field in L-SIG is set to a “duration” in 4 us units. STAs that understand short GI are able to determine the exact time at which a short GI PPDU ends. STAs that do not understand short GI use the Length in L-SIG. To ensure fair channel access contention it is better to use the 4us boundary after the end of the last symbol for SIFS. Otherwise, STAs could become “mis-aligned”. For HT-GF, SIFS could start at the end of the last symbol (but perhaps rounded up to the nearest microsecond).

4 us boundaries

3.6 us

3.6 us

3.6 us

End of the last symbol

4us boundary after end of the last symbol

Data Sym

Data Sym

Data Sym

Draft 2.1 covers the PMD_TXEND.confirm primitive in Clause 22.6.5.6.1 where it states the 4 us boundary after the end of the last symbol shall be used. But there is no text that describes when to issue a PHY-CCA.indication(idle). We could simply add some text here to link PHY-CCA.indication(idle) to the 4 us boundary after the end of the last symbol. However Clause 22.3.21 (PLCP receive procedure) states (Lines 31-34):

If a valid L_SIG parity bit is indicated, the VHT PHY shall maintain

PHY-CCA.indication(BUSY, channel-list) for the predicted duration

of the transmitted PPDU, as defined by RXTIME in Equation (), …

There is a missing equation reference but assuming that gets fixed RXTIME is given on page 262 of D2.1 where it is already in 4 us units.

This just leaves Clause 9.3.2.3.3 and 9.3.7 which need additional text because for HT-MF and VHT short GI PPDUs SIFS is no longer from the end of the last symbol. This will affect existing 11ac implementation but we do need to resolve this ambiguity because if two devices used different references for SIFS, this would result in responses arriving up to 3.6 us (>20% of SIFS) earlier or later than expected. So I am proposing two options:

- Add the following text in Clause 9.3.2.3.3 and 9.3.7:

SIFS shall be measured from the 4us boundary after the end of the last symbol if the PPDU is a:

  - Clause 20 PPDU transmitted with the TXVECTOR FORMAT parameter set to HT_MF and GI_TYPE set to SHORT_GI; or

  - Clause 22 PPDU transmitted with the TXVECTOR_FORMAT parameter set to VHT and GI_TYPE set to SHORT_GI.

- Add text in Clause 22 to say that SIFS is from the end of the last symbol despite the fact that PMD-TXEND.confirm and PHY-CCA.indication are issued on 4 us boundary.

## Proposal: