IEEE P802.11
Wireless LANs

| Resolution for LB187 CID 5317 - Sequence Control numbers for VHT Compressed Beamforming frame |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-05-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Erik Lindskog | CSR | 1390 Kifer Road, Sunnyvale, CA 94086, USA | +1-408-392-8384 | Erik.Lindskog@csr.com |
| Wei Shi | CSR | CB4 0WZ, U.K. | +44 1223 692000 | Wei.Shi@csr.com |
| Mark Rison | CSR | CB4 0WZ, U.K. | +44 1223 692000 | Mark.RISON@csr.com |

Abstract

This document proposes resolutions for LB187 CID 5317 on P802.11ac/D2.0, regarding Sequence Control numbers for VHT compressed beamforming frames.

r0: Initial revision.

r1: Added text about not storing Sequence Control numbers in cache.

r2: Minor editorial changes

## Discussion

The MAC Sequence Control numbers are intended to avoid duplicate reception of frames (after failed ACKs). However, a VHT Compressed Beamforming frame is only sent once in response to an NDP frame or a Beamforming Report Poll frame, thus there can be no duplicate frames of this type. Thus a MAC sequence numbers are not required for VHT Compressed Beamforming frames and/or can be ignored by the recipient of such frames.

Furthermore, all the information needed to keep track of the feedback arriving in the VHT Compressed Beamforming frames is contained in the VHT MIMO Control field. This is another reason why MAC Sequence Control numbers are not required for a VHT Compressed Beamforming frames.

## Proposed changes

The changes are relative to D2.0.  The changes are shown using Word change tracking.  Select “Final Showing Markup” or “Final” as appropriate.  Editorial instructions are shown using bold italics.  Any Word comments should be ignored.

Change the 16th paragraph of 8.3.2.1 as follows:

The Sequence Control field is defined in 8.2.4.4. The Sequence Control field for QoS (+)Null frames and VHT Compressed Beamforming frames is ignored by the receiver upon reception.

Change the last sentence in the 4th paragraph of 9.3.2.10 as follows:

Sequence numbers for QoS (+)Null frames and VHT Compressed Beamforming frames may be set to any value.

Change the last sentence in the 5th paragraph of 9.3.2.10 as follows:

A receiving STA should omit tuples obtained from group addressed frames, QoS(+)Null frames, VHT Compressed Beamforming frames and ATIM frames from all caches.

## Proposed resolution

Revised.  See proposed changes in IEEE 802.11-12/522r2.