IEEE P802.11
Wireless LANs

| 802.11 TGac WG Letter Ballot LB187 Proposed resolutions to comments on transmitter block diagram |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-05-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yusuke ASAI | NTT | 1-1, Hikarinooka, Yokosuka-Shi, Kanagawa, 239-0847, Japan | +81 46 859 3494 | asai.yusuke@lab.ntt.co.jp |

Abstract

This submission contains proposed comment resolutions to comments received during WG letter ballot 187.

The comments included are non-editorial comments on transmitter block diagram.

There are 21 such comments: 4499, 5122, 4643, 5123, 5124, 5125, 4585, 5127, 5126, 5128, 5129, 4569, 5131, 4077, 5130, 5432, 5132, 5133, 5134, 5334 and 5135.

R0: Initial Version

- The following six comments are related to the multiplication of the first column of PVHTLTF to VHT-SIG-B.

| CID | By | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4499 | DANING GONG | 176.27 | 22.3.3 | There is a conflict in the use of symbol P, which is also reused in 22.3.18.5.2 to denote the total transmit power. Furthermore, there is no explicit definition of matrix P. | Please change matrix P to matrix P_VHTLTF, where _VHTLTF means that VHTLTF is the subscript of P. |
| 5122 | Sigurd Schelstraete | 176.27 | 22.3.3 | Notation is not defined | The notation [P]_u is not defined anywhere. Either define or reword bullet item k). |
| 5124 | Sigurd Schelstraete | 177.35 | 22.3.3 | Use of [P]_u not needed for SU packet | Use of [P]_u implies MU behavior. This contradicts the caption which explicitly mentions SU. |
| 5125 | Sigurd Schelstraete | 177.35 | 22.3.3 | Find better label for block that is currently labeled as "multiply by 1st column of [P]_u" | Instead of labeling the block "multiply by 1st column of [P]_u", a more appropiate name should be found for this operation. This also applies to all subsequent figures in 22.3.3. |
| 4585 | Jens Tingleff | 177.38 | 22.3.3 | (also page 178 figure 22-5) The drawings use [P]u where the equations appear to have P subscript VHTLTF. | Change [P]u to PVHTLTF (in line with words in subclause 22.3.4.8 "h)" on page 186 line 12) or define [P]u |
| 5126 | Sigurd Schelstraete | 178.06 | 22.3.3 | For MU operation, us of [P]_u should show actual user indices | For all users, [P]_u is used. This should be replace with [P]_0, ..., [P]_(Nu-1) - but don't forget to define [P]_u first ... |

Discussions:

On the context of 22.3.3, P is referred as the pattern of VHT-LTF; however, in TGac Draft D2.0, P is defined as pilot subcarriers defined in 18.3.5.10 (OFDM modulation) or total transmit power in 22.3.18.5.2. As the comments point out, PVHTLTF should be used instead of P.

In addition, the phrase “the first column of [PVHTLTF]u“ is erroneous in both SU and MU cases.

- In SU case, user index is not defined.

- In MU case, as defined at 204.18 (22.3.8.2 VHT portion of VHT format preamble), [Q]N indicates a matrix consisting of the first N columns of matrix Q. [PVHTLTF]u means the first u columns of PVHTLTF; therefore, “the first column of [PVHTLTF]u“ always provides the first column of PVHTLTF. Fig. I illustrates this problem. It is obvious that the error should be fixed. One of the simple solutions is to replace [PVHTLTF]u with PVHTLTF for user u.

(Of course, this is not exact explanation for “a part of the first column of PVHTLTF which corresponds to user u.” But it is enough to describe overview of the transmitter block diagram. The exact numerical formula is in Equation (58) in Clause 22.)

Fig. I: The current and correct definitions of the 1st column of [PVHTLTF]u

Proposed resolutions (CIDs 4499, 5122, 5124, 5125, 4585 and 5126): Revised.

At 176.17:  Change as follows.

k) Multiply by 1st column of [P]u PVHTLTF

At 177.34: Replace Figure 22-4 to the following one.

At 178.4: Replace Figure 22-5 to the following one.

- The following comment is related to the transmitter block diagrams for VHT-STF and VHT-LTF.

| CID | By | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4643 | Juho Pirskanen | 177.27 | 22.3.3. | None of the block figures are explaining how the VHT-STF and VHT -LTF are transmitted | Insert new figures or indicate in existing figures which of existing figures are applicable. |

Discussions:

Actually, there are no block diagrams for VHT-STF and VHT-LTF. It follows the existing transmit block diagrams for non-HT and HT PPDUs in IEEE802.11 – 2012 (see the figures listed below).

- Figure 18-12: The transmitter and receiver block diagram for the OFDM PHY

- Figure 20-2: The transmitter block diagram for non-HT portion of HT PPDU

- Figure 20-3: The transmitter block diagram for HT portion of HT PPDU

It is reasonable to include no block diagrams for VHT-STF and VHT-LTF to keep consistency with the existing standard. In addition, how to create VHT-STF and VHT-LTF are explicitly defined in 22.3.4.6 and 22.3.4.7, which are enough for clarification.

Proposed resolution: Rejected.

- The following three comments are related to revisions for Figures 22-4 through 22-6.

| CID | By | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 5123 | Sigurd Schelstraete | 177.35 | 22.3.3. | Improve Figure 22-4 | Put curly bracket underneath blocks shown in Figure 22-4 to show N_STS spatial streams and N_TX transmit chains (similar to what is done in Figure 22-3) |
| 5127 | Sigurd Schelstraete | 178.04 | 22.3.3 | Improve Figure 22-5 | Put curly bracket underneath blocks shown in Figure 22-5 to show N_STS,u, N_STS,total and N_TX (similar to what is done in Figure 22-3) |
| 5128 | Sigurd Schelstraete | 179.04 | 22.3.3 | Improve Figure 22-6 | Put bracket underneath blocks shown in Figure 22-6 to show N_ES, N_SS, N_STS,total and N_TX (similar to what is done in Figure 22-3) |

Discussions:

To show the numbers of spatial streams, stpace-time streams, transmitter chains in transmit block diagrams will be useful for understanding.

Proposed resolution to CIDs 5126, 5127 and 5128: Accepted.

(The revised figure is based on the resolution to CIDs 4499, 5122, 5124, 5125, 4585 and 5126 in this document.)

At 177.34: Replace Figure 22-4 to the following one.

At 178.4: Replace Figure 22-5 to the following one.

At 179.4: Replace Figure 22-6 to the following one.

- The following comments are related to revisions for Figures 22-6, 22-7, 22-8 and 22-9.

| CID | By | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4569 | Eldad Perahia | 179.43 | 22.3.3 | The encoder parser is only used for BCC | Change "Encoder parser" to "BCC encoder parser" in Figure 22-6, 8, 9 |
| 4077 | Adrian Stephens | 180.14 | 22.3.3 | p170.30: "indexed by u, where u takes values 0 to NUM_USERS-1." Figure 22-7 shows 1-based indexing | Change figure 22-7 to show 0-based indexing. (lines 14 and 25) |
| 5130 | Sigurd Schelstraete | 180.14 | 22.3.3 | Users should be labelled 0 - Nu-1 | In Figure 22-7, users are label 1-Nu. In Figure 22-5, they are labeled 0 - Nu-1. The latter is more consistent with practice used in the text. Change labeling to 0 - Nu-1 in Figure 22-7. |
| 5432 | Yusuke Asai | 180.14 | 22.3.3 | The smallest and largest numbers of user indices should be 0 and N_u - 1as well as Figure 22-5, respectively. | Change "User 1" to "User 0" and "User N_u" to "user N_u-1." |

Discussions:

All of the proposed changes are reasolable. In addition, the blocks of LDPC tone mappers in Figures 22-6, 22-8 and 22-9 should be deleted because they ares examples using BCC encoders as well as the subblock of user 0 in Figure 22-7.

Proposed resolution to CID 5128, 4569, 4077, 5130 and 5432: Accepted.

At 179.4: Replace Figure 22-6 to the following one.

(The revised figure is based on the resolution to CIDs 5126, 5127 and 5128 in this document.)

At 180.4: Replace Figure 22-7 to the following one.

At 181.4: Replace Figure 22-8 to the following one.

At 182.4: Replace Figure 22-9 to the following one.

- The following four comments are related to the lavel “Spatial Mapping” for Figures 22-6 to 22-9.

| CID | By | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 5129 | Sigurd Schelstraete | 179.04 | 22.3.3 | Figure 22-6 should also cover case with beamforming | Replace label "Spatial Mapping" with "Spatial Mapping/Steering Matrix" |
| 5131 | Sigurd Schelstraete | 180.04 | 22.3.3 | Figure 22-7 should also cover case with beamforming | Replace label "Spatial Mapping" with "Spatial Mapping/Steering Matrix" |
| 5132 | Sigurd Schelstraete | 181.04 | 22.3.3 | Figure 22-8 should also cover case with beamforming | Replace label "Spatial Mapping" with "Spatial Mapping/Steering Matrix" |
| 5133 | Sigurd Schelstraete | 182.04 | 22.3.3 | Figure 22-9 should also cover case with beamforming | Replace label "Spatial Mapping" with "Spatial Mapping/Steering Matrix" |

Discussion:

TGac Draft D2.0 and Std. 802.11-2012 explain the function of “Spatial Mapping” as follows:

- In 22.3.4.9 (Construction of the Data field in an SU PPDU) of TGac D2.0:

[at 187.9] “Spatial Mapping: Apply the Q matrix as described in 22.3.10.11.1 (Transmission in VHT format).”

- In 22.3.10.11.1 (Transmission in VHT format) of TGac D2.0:

[at 240.60] “Note that implementations are not restricted to the spatial mapping matrix examples listed in Section 20.3.11.11.2 (Spatial mapping) and the number of transmit chains NTX could be up to 8.

- In 20.3.11.11.2 (Spatial mapping) of Std 802.11-2012:

[at 1722] “Below are examples of spatial mapping matrices that might be used. There exist many other alternatives; implementation is not restricted to the spatial mapping matrices shown. The examples are:”

 following the sentence, four types of spatial mapping are introduced.

[at the second to the last line in page 1,723]

“d) Beamforming steering matrix: Qk is any matrix that improves the reception in the receiver based on some knowledge of the channel between the transmitter and the receiver.”

Therefore, beamforming is included in one of the cases for “Spatial Mapping.” There is no need to change the label.

Proposed Resolutions to CIDs 5129, 5132, 5132 and 5133: Rejected.

- The following three comments are related to lack of explanation for Figure 22-5.

| CID | By | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 5134 | Sigurd Schelstraete | 182.51 | 22.3.3 | Explicitly add that Figure 22-4 shows SU PPDU | Replace "VHT-SIG-B field of the VHT PPDU" with "VHT-SIG-B field of a VHT SU PPDU" |
| 5434 | Yusuke Asai | 182.51 | 22.3.3 | In subsection 22.3.3, there is no specific explanation for Figure 22-5. | Change the sentence which starts on P182L51 as "Figure 22-4 shows..." to "Figures 22-4 and 22-5 show the transmit process for generating the VHT-SIG-B field of SU and MU VHT PPDUs, respectively." |
| 5135 | Sigurd Schelstraete | 182.53 | 22.3.3 | Description of Figure 22-5 is missing | Add description of Figure 22-5 before description of Figure 22-6 |

Discussion:

There is no specific explanation for Figure 22-5 in section 22.3.3 and should be added. During the comment resolution for TGac Draft D2.0, we have deciced to use the term “VHT SU PPDU / VHT MU PPDU” instead of “SU VHT PPDU / MU VHT PPDU.” (see the resolutions for CIDs 5399 and 5400 in 12/0226r7.)

Proposed Resolutions to CIDs 5434 and 5135: Revised.

At 182.51 Change the sentence as follows:

Figure 22-4 and Figure 22-5 shows the transmit process for generating the VHT-SIG-B fields of the VHT SU PPDU and VHT MU PPDU, respectively.

Appendex

The resulting figures applying all of the proposed resolutions without redlines are as follows: