IEEE P802.11
Wireless LANs

| 802.11 TGac WG Letter Ballot LB187 Proposed resolutions to comments on timing-related parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-05-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yusuke ASAI | NTT | 1-1, Hikarinooka, Yokosuka-Shi, Kanagawa, 239-0847, Japan | +81 46 859 3494 | asai.yusuke@lab.ntt.co.jp |

Abstract

This submission contains proposed comment resolutions to comments received during WG letter ballot 187.

The comments included are non-editorial comments on timing-related parameters.

There are 7 such comments: CIDs 4080, 5143, 5144, 5145, 5146, 5147 and 5148.

R0: Initial Version

R1: Revise the text for resolutios and change the cited page numbers. Reference draft was changed from TGac D2.0 to D2.2.

- The following comment is related to the definition of NCBPSSI.

| CID | By | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4080 | Adrian Stephens | 22.3.6 | 22.3.6 | N_CBPSSI depends on N_CBPSS, which (according to the previous row) is undefined for MU PPDUs. | Add statement to Explanation column of N_CBPSSI that this value is defined only for SU PPDUs. |

Discussions:

The parameter NCBPSSI is used in section 22.3.10.8 (BCC interleaver), which describes:

[At 225.04]: For ease of explanation, the operation of the interleaver is described for the SU case. In an MU transmission, the interleaver operates in the same way on the output bits for each user from the stream parser. That is, the operation of the interleaver is the same as if the transmission were an SU one, consisting of bits from that user only.

Actually, BCC interleaver is defined in user u of MU transmission and we should have the definition of per-user NCBPSSI for better clarification.

Proposed resolution to CID 4080:

Revised. 11-12/0526r1 provides the proposed changes for the definitions of NCBPSSI and NCBPSSI,u,and for the correspondint text to NCBPSSI,u.

Change D2.2 22.3.6 (Timing-related parameters) as follows:

At 188.28:  Change the 4th row of Table 22-6 as follows.

| Symbol | Explanation |
| --- | --- |
| NCBPSSI, NCBPSSI,u | Number of coded bits per symbol per spatial stream per BCC interleaver block For SU PPDUs, For MU PPDUs for user u, u = 0, 1, 2, 3, NCBPSSI,u=NCBPSS,u,for 20, 40 and 80 MHz PPDUsNCBPSS,u2,for 160 and 80+80 MHz PPDUs |

Change D2.2 22.3.10.8 (BCC interleaver) as follows:

At 225.04: Change the 1st paragraph in 22.3.10.8 (BCC interleaver) as follows:

For ease of explanation, the operation of the interleaver is described for the SU case. In user u of an MU transmission, the interleaver operates in the same way on the output bits for each the user from the stream parser by replacing NSS, NCBPSS, NCBPSSI and NBPSCS with NSS,u, NCBPSS,u, NCBPSSI,u and NBPSCS,u, respectively. That is, the operation of the interleaver is the same as if the transmission were an SU one, consisting of bits from that user only.

- The following two comments are related to the multiplication of the first column of PVHTLTF to VHT-SIG-B.

| CID | By | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 5143 | Sigurd Schelstraete | 189.30 | 22.3.6 | Replace "complex data numbers" with "data subcarriers" | N_SD is described as "Number of complex data numbers per frequency segment". This is inconsistent with e.g. definition of N_ST. It is also clearer to define N_SD as "Number of data subcarriers per frequency segment" |
| 5144 | Sigurd Schelstraete | 189.30 32 | 22.3.6 | Replace "pilot values" with "pilot subcarriers" | N_SP is described as "Number of pilot values per frequency segment". This is inconsistent with e.g. definition of N_ST. It is also clearer to define N_SP as "Number of pilot subcarriers per frequency segment" |

Discussions:

In Clause 18 (non-HT) of Std 802.11-2012, NSD is defined as the number of data subcarriers; however, in Clause 20 (HT), the definition of the phrase has been changed (see Table 20-6). The reason of it is described on NOTE 1 of Table 20-6.

NOTE 1—NST=NSD+NSP except in the cases of MCS 32 and non-HT duplicate, where the number of data subcarriers differs from the number of complex data numbers, and the number of pilot subcarriers differs from the number of pilot values. In those cases, data numbers and pilot values are replicated in upper and lower 20 MHz portions of 40 MHz signal to make a total of 104 subcarriers.

Clause 22 defines non-HT duplicate transmission. There exist the conditions where the number of data subcarriers differs from the number of complex data numbers, and the number of pilot subcarriers differs from the number of pilot values. Therefore, current descritions for NSD and NSP are appropriate.

Proposed resolution to CIDs 5143 and 5144:

Rejected. Clause 22 defines non-HT duplicate transmission. There exist the conditions where the number of data subcarriers differs from the number of complex data numbers, and the number of pilot subcarriers differs from the number of pilot values. Therefore, current descritions for NSD and NSP are appropriate.

- The following four comments are related to add user index u. to NCBPS and R.

| CID | By | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 5145 | Sigurd Schelstraete | 190.43 | 22.3.6 | N_CBPS is a per-user parameter | Note that N_CBPS is a per-user parameter. Even if the notation N_CBPS,u is not used in the document, it is implied in several places that this parameter applies per user (see e.g. 22.3.10.6, where MU operation is essentially described as SU operation carried out per user). Define N_CBPS,u similarly to other per-user parameters. |
| 5146 | Sigurd Schelstraete | 190.62 | 22.3.6 | N_BPSC is a per-user parameter | N_BPSC should be defined as per-user parameter |
| 5147 | Sigurd Schelstraete | 190.64 | 22.3.6 | N_BPSCS is a per-user parameter | N_BPSCS should be defined as per-user parameter |
| 5148 | Sigurd Schelstraete | 191.44 | 22.3.6 | R is a per-user parameter | R should be defined as per-user parameter |

Discussion for CIDs 5145 and 5148 (NCBPS and R):

In SU transmission, Equation (74) uses NCBPS to calculate Navbits, which is a parameter of the available bits per PPDU using LDPC encoder. On the other hands, as described in 22.3. 10.5.4 (LDPC coding), the derivation of Navbits in MU transmission is defined by referring to 20.3.11.7.5 (LDPC PPDU encoding process).

[At 221.14 in TGac D2.2]

Then, for each LDPC user in the MU PPDU, compute the LDPC encoding parameters based on steps (a) to (d) in 20.3.11.7.5 (LDPC PPDU encoding process), with the exception that Equation (77) is used to compute Npld,u instead of Equation (20-35).

Npld,u=NSYM_max_initNDBPS,u			(77)

[At 1713 in Std. 802.11-2012]

To encode an LDPC PPDU, step a) through step g) shall be performed in sequence:

- Compute the number of available bits, Navbits, in the minimum number of OFDM symbols in which the Data field of the packet may fit.

Npld=length×8+16 				(20-35)

Navbits=NCBPS×mSTBC×NpldNCBPS×R×mSTBC	 (20-36)

This implicitly instructs Equation (20-36) is used to calculate Navbits for each user. In addition, the parameters NCBPS and R in Equation (20-36), must be per-user parameter in MU case. Therefore, the definitions of NCBPS,u and Ru are needed.

Discussion for CIDs 5146 (NBPSC):

NBPSC is used in Std. 802.11-2012.

- In clause 18, this parameter is used to define s, an interleaver parameter.

- In Clause 20, instead of NBPSC, NBPSCS is used for the definition of interleaver to support unequal modulation. NBPSC exists only for MCS tables for unequal modulation (see Tables 20-39 through 20-44).

TGac D2.0 also defines NBPSC but the parameter is no longer needed because the draft does not support unequal modulation.

Discussion for CIDs 5147 (NBPSCS):

NBPSCS is refrered to define stream parser and interleaver for BCC encoder(s). The parameter actuall exists per user and we need to define NBPSCS,u for clarification.

Discussion for CIDs 5145, 5148 and 5147 (NCBPS, R and NBPSCS):

As described above, we need to define NCBPS,u, Ru, and NBPSCS,u. Tables 22-29 to 22-32, Tables 22-37 to 22-40, Tables 22-45 to 22-48 and Tables 22-53 to 22-57 use these parameters and should be revised.

(Because TGac D2.0 defines that the maximum number of NSS,u is set to four, The MCSs tables for NSS is more than four is only for SU transmission.)

Proposed resolution to CIDs 5145, 5146, 5147 and 5148:

Revised. 11-12/0526r1 provides the following proposed changes:

- To add the definition of NCBPS,u, NBPSCS,u, and Ru

- To delete the redundant definition of NBPSC

- To change the sentences to clarify the encoding processing for MU PPDUs and parameters for VHT MCSs for MU transmission.

Change D2.2 22.3.6 (Timing-related parameters) as follows:

At 188.26:  Change the 1st row of Table 22-6 as follows:

| Symbol | Explantion |
| --- | --- |
| NCBPS, NCBPS,u | Number of coded bits per symbol for user u, u = 0, 1, 2, 3. For an SU PPDU, NCBPS = NCBPS,0 For an MU PPDU, NCBPS is undefined |

At 188.45:  Delete the 5th row of Table 22-6 as follows:

| Symbol | Explantion |
| --- | --- |
| NBPSC | Number of coded bits per subcarrier over all spatial streams |

At 188.47: Change the 6th row of Table 22-6 as follows:

| Symbol | Explantion |
| --- | --- |
| NBPSCS, NBPSCS,u | Number of coded bits per subcarrier per spatial streams for user u, u = 0, 1, 2, 3. For an SU PPDU, NBPSCS = NBPSCS,0 For an MU PPDU, NBPSCS is undefined |

At 189.34:  Change the second last row of Table 22-6 as follows.

| Symbol | Explantion |
| --- | --- |
| R, Ru | Coding rate for user u, u = 0, 1, 2, 3. For SU PPDUs, R = R0 For MU PPDUs, R is undefined |

Change D2.2 22.3.10.5.4 (LDPC coding) as follows:

At 221.14: Change the paragraph as follows and renumber the equations:

Then, for each LDPC user in the MU PPDU, compute the LDPC encoding parameters based on steps (a) to (d) in 20.3.11.7.5 (LDPC PPDU encoding process), with the exception that Npld,u, Navbits,u and NSYM,u defined by Equations (22-62), (22-63) and (22-64)is are used to compute Npld,u instead of Npld,  Navbits and NSYM defined by Equations (20-35), (20-36) and (20-41), respectively.

Npld,u=NSYM_max_initNDBPS,u				(22-62)

Navbits,u=NCBPS,u×mSTBC×Npld,uNCBPS,u×Ru×mSTBC	(22-63)

NSYM,u=Navbits,u/NCBPS,u	(22-64)

Let NSYM,u be the NSYM computed by Equation (20-41) in step (d) of 20.3.11.7.5 (LDPC PPDU encoding

process) for user u.

Change D2.2 22.5 (Parameters for VHT MCSs) as follows:

At 269.05: Add the following sentences:

Tables 22-29 to 22-32, Tables 22-37 to 22-40, Tables 22-45 to 22-48 and Tables 22-53 to 22-56  define VHT MCSs not only for SU transmission but also for user u of MU transmission. In the case of VHT MCSs for MU transmission, the parameters, NSS, R, NBPSCS, NCBPS, NDBPS, and NES are replaced with NSS,u, Ru, NBPSCS,u, NCBPS,u, NDBPS,u, and NES,u, respectively.