IEEE P802.11
Wireless LANs

| 802.11 TGac WG Letter Ballot LB187 Proposed resolutions to comments on channelization |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-05-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yusuke ASAI | NTT | 1-1, Hikarinooka, Yokosuka-Shi, Kanagawa, 239-0847, Japan | +81 46 859 3494 | asai.yusuke@lab.ntt.co.jp |

Abstract

This submission contains proposed comment resolutions to comments received during WG letter ballot 187.

The comments included are non-editorial comments on timing-related parameters.

There are 4 such comments: CIDs 4208, 5386, 4100 and 5200.

R0: Initial Version

- The following two comments are related to the entities of dot11CurrentChannelBandwidth.

| CID | By | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4208 | Allert Van Zelst | 245.35 | 22.3.14 | The entities of dot11CurrentChannelBandwidth are not consistent in the spec, cf. e.g. table 22-21, 22.2.4.3, 22.3.7 and 22.3.14 | Fix this |
| 5386 | Youhan Kim | 245.35 | 22.3.14 | Possible values for dot11CurrentChannelBandwidth are not consistent throughout the spec. Some places use 20/40/80/160/80+80 MHz, while some other places use cbw20/40/80/160/80p80. | Unify the possible values. |

Discussions:

In TGac D2.0, the possible values of dot11CurrentChanelBandwidth are defined as cbw20, cbw40, cbw80, cbw160 and cbw80p80 in Table 22-21; however, these values never appear. Instead, 20 MHz, 40 MHz, 80 MHz, 160 MHz and 80+80 MHz are mainly used.

Proposed resolution (CIDs 4208 and 5386): Accept.

At 174.54: Change the 2nd paragraph of 22.2.4.3 (Support for HT formats):

… if dot11CurrentChannelBandwidth is CBW20 20 MHz,…

At 245.35:  Change the 1st row of Table 22-21 as follows.

| Field | Meaning |
| --- | --- |
| Dot11CurrentChannelBandwidth | Channel bandwidth. Possible values are cbw20, cbw40, cbw80, cbw160 and cbw80p80. 20 MHz, 40 MHz, 80 MHz, 160 MHz and 80+80 MHz. |

- The following comment is related to the channel parameters when center frequency is 5.000 GHz.

| CID | By | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 4100 | Adrian Stephens | 246.18 | 22.3.14 | "A channel center frequency of 5.000 GHz shall be indicated by dot11ChannelStartingFactor = 8000 and dot11CurrentPrimaryChannel = 200." Is is it an example, or a special rule for 5 GHz? | "For example, a channel center frequency of 5.000 GHz can be indicated by dot11ChannelStartingFactor = 8000 and dot11CurrentPrimaryChannel = 200." |

Discussions:

The paragraph for the definition of the center frequency of the primary 20 MHz channel is as follows:

[At 246.9]

The center frequency of the primary 20 MHz channel is given by Equation (114).

Primary 20 MHz channel center frequency [MHz]

= Channel starting frequency + 5 dot11CurrentPrimaryChannel		(114)

The channel starting frequency is defined as dot11ChannelStartingFactor × 0500 kHz.

It is the special case where the primary 20 MHz channel center frequency is 5.000 GHz that there may be a couple of parameter sets to indicate 5.000GHz of the primary 20 MHz channel center frequency:

- ChannelStartingFactor = 10 000 and dot11CurrentPrimaryChannel = 0

  - Channel Starting frequency = 5 000 MHz

  - 5 dot11CurrentPrimaryChannel = 0 MHz

- ChannelStartingFactor = 8 000 and dot11CurrentPrimaryChannel = 200

  - Channel Starting frequency = 4 000 MHz

  - 5 dot11CurrentPrimaryChannel = 1000 MHz

The sentence, “A channel center frequency of 5.000 GHz shall be indicated by dot11ChannelStartingFactor = 8000 and dot11CurrentPrimaryChannel = 200”, limits only the parameter set of b) is valid. Because Table 22-21 specifies that the valid range for dot11CurrentPrimaryChannel is 1 to 200 and 0 is invalid.

The similar sentence also exists on 20.3.15.3 (Channel allocation in 5 GHz band) in Std. 802.11-2012.

[At 1738 in Std. 802.11-2012 ]

20.3.15.3 Channel allocation in the 5 GHz band

Channel center frequencies are defined at every integral multiple of 5 MHz above 5 GHz. The relationship between center frequency and channel number is given in Equation (20-88).

Channel center frequency = Channel starting frequency + 5 × nch(MHz) 	(20-88)

Where

nch = 1, ..., 200

Channel starting frequency is defined as dot11ChannelStartingFactor × 500 kHz or is defined as 5.000 GHz for systems where dot11OperatingClassesRequired is false or not defined. A channel center frequency of 5.000 GHz shall be indicated by dot11ChannelStartingFactor = 8000 and nch = 200.

Therefore, the sentence should remain as it is.

Proposed resolution to CID 4100: Rejected.

- The following comment is related to the example of the parameters for 80 MHz channel.

| CID | By | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 5200 | Sigurd Schelstraete | 246.30 | 22.3.14 | Mention channel starting frequency for example | Add "channel starting frequency = 5000 kHz" |

Discussions:

In each example, channel starting frequency is needed to identify the exact center frequency.

Proposed resolution (CIDs 5200): Accept.

At 246.30: Change the 6th to the last paragraphs of 22.3.14 (Channelization) as follows:

For example, a channel specified by

channel starting frequency = 5 000 kHz

dot11CurrentChannelBandwidth = 80 MHz

dot11CurrentChannelCenterFrequencyIndex1 = 42

dot11CurrentPrimaryChannel = 36

is an 80 MHz channel with a center frequency of 5210 MHz and the primary 20 MHz channel centered at 5180 MHz.

A channel specified by

channel starting frequency = 5 000 kHz

dot11CurrentChannelBandwidth = 160 MHz

dot11CurrentChannelCenterFrequencyIndex1 = 50

dot11CurrentPrimaryChannel = 56

is a 160 MHz channel with a center frequency of 5250 MHz and the primary 20 MHz channel centered at 5280 MHz.

A channel specified by

channel starting frequency = 5 000 kHz

dot11CurrentChannelBandwidth = 80+80 MHz

dot11CurrentChannelCenterFrequencyIndex1 =155

dot11CurrentChannelCenterFrequencyIndex2 = 106

dot11CurrentPrimaryChannel = 161

is an 80+80 MHz channel in which frequency segment 1 has 80 MHz bandwidth and center frequency of 5775 MHz. Frequency segment 2 also has 80 MHz bandwidth and center frequency of 5530 MHz. The primary 20 MHz channel is centered at 5805 MHz.