IEEE P802.11
Wireless LANs

| Resolution for Comments on RDG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-05-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Bin Chen | Huawei | 3601 Algonquin Road, Rolling Meadows, IL 60008 | +1 (847) 818 1778 | Bin.Chen@huawei.com |
| Eric Zhang | Huawei | Huawei Base Bantian Shenzhen |  | Zhangxingxin@huawei.com |
| David Xun Yang | Huawei |  |  |  |
| Osama | Huawei |  |  |  |
| Wangxuehuan | Huawei |  |  |  |

Abstract: Resolutions of D2.0 comments on RDG: 4224, 4233

##### CIDs 4224, 4233

| 4224 | Bin Chen | 8.2.4.1.8 | 27.38 | Buffer data indication with More data field in response frame (e.g. CTS/ACK) can be a guidance of RDG. | The more data bit is set to 1 in individual response frame (e.g. CTS/ACK), if there is buffer data for intended STA. The intended STA could use this info to decide initiate RDG or not. |  |
| --- | --- | --- | --- | --- | --- | --- |
| 4233 | Bin Chen | 8.2.4.1.8 | 27.38 | In a TXOP, the owner doesn't know whether there is buffer data in the peer STA that need to be sent to the owner. This will cause a mis-granted of RDG to the peer STA who doesn't need it. | The more data bit is set to 1 in individual response frame (e.g., CTS/ACK) if there is buffer data for the TXOP owner. The TXOP owner could use this info to decide initiate RDG or not. |  |

Discussions:

In a TXOP, the owner doesn't know whether there is buffered data in the peer STA that need to be sent to the owner. This will cause a mis-granted of RDG to the peer STA who doesn't need it.

Buffered data indication by More Data field in some response frames(e.g.  CTS/ACK/BA) can be a guidance of RDG granting.

Proposed resolution:

TGac Editor: please add a paragraph in section 8.2.4.1.8, page 27 line 31 as below:

8.2.4.1.8 More Data field

Insert the following after the 5th paragraph:

The More Data field is optionally set to 1 in CTS/BA frames transmitted as a response to indicate that the STA has at least one buffered MSDU for the peer STA. The peer STA may consider this info to decide initiating RDG or not.

In a VHT BSS where the TXOP power save feature is supported at both the AP and an individual STA (as determined from its VHT capabilities element), the More Data field indicates that more BUs are buffered for that STA at the AP (see 10.2.1.4a (Power management during VHT transmissions) for the operation of TXOP power save). A value of 1 in individually addressed frames transmitted by the VHT AP to a VHT STA in TXOP PS mode indicates that at least one additional buffered BU is present for the same STA.