IEEE P802.11
Wireless LANs

| LB187 resolutions for various MU CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-05-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Nihar Jindal | Broadcom. |  |  |  |
| Matthew Fischer | Broadcom |  |  |  |

Abstract

This document proposes resolutions for the following MU CIDs: 4656, 4658, 5038, 5250, 5251.

| CID | Commenter | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 4656 | kaiying Lv | 44.10 | 8.4.1.46 | The description of subfield"Remaining Segments" of Table 8-53b is inconsistent with the comment resolution for 3476. | It should be "Set to a value between 1 and 7 for a segment that is not the last segment of a segmented frame." | ACCEPT. Change text according to 11/1469r2 |
| 4658 | kaiying Lv | 82.38 | 8.5.23.2 | Table 8-281aj--"VHT Compressed Beamforming Report frame fields present" is inconsistent with the comment resolution for 3476. | Make changes according to comment 3476 | ACCEPT. Change text according to 11/1469r2 |

Discussion:  Resolution to CID 3476 was agreed upon in 11/1469r2, but D2.1 does not reflect those changes (instead D2.1 seems to reflect the earlier changes from 11/1276r1, but 11/1469r2 over-writes those changes).

Proposed Resolution: Make changes agreed upon in 11/1469r2.

Tgac Editor:

Please revise appropriate fields of Table 8-53b as follows:

| Subfield | Description |
| --- | --- |
| Feedback Type | Set to 0 for SU Set to 1 for MU |
| Remaining Segments | Indicates(#3282) the number of remaining segments(#2793) for the associated VHT Compressed Beamforming frame. Set to 0 for the last segment of a segmented frame or the only segment of an unsegmented frame. Set to a value between 1 and 7 for a segment that is not the last segment of a segmented frame. Set to 7 if the VHT Compressed Beamforming Report field and MU Exclusive Beamforming Report field are not present in the frame.(#3476) In(#3286) a retransmitted segment, the field is set to the same value associated with the segment in the original transmission.(#3285) |
| First Segment | Set to 1 for the first segment of a segmented frame or the only segment of an unsegmented frame; set to 0 if it is not the first segments or if the VHT Compressed Beamforming Report field and MU Exclusive Beamforming Report field are not present in the frame.. In(#3286) a retransmitted segment, the field is set to the same value associated with the segment in the original transmission.(#3285) |

Please change the last paragraph of 8.5.23.2 (VHT Compressed Beamforming frame format) to:

The conditions under which VHT Compressed Beamforming Report field and the MU Exclusive Beamforming Report field are absent are defined in 9.31.5 (VHT sounding protocol); Remaining Segments, Feedback Type and First Segment  subfields of the VHT MIMO Control field are set as defined in Table 8-281aj*.(#3476)(#3477);

And change Table 8-281aj to:

Table 8-YY—VHT Compressed Beamforming Report frame fields present

| VHT MIMO Control field |  |  | VHT Compressed Beamforming Report field present | MU Exclusive Beamforming Report field present |
| --- | --- | --- | --- | --- |
| Feedback Type | Remaining Segments | First Segment |  |  |
| SU | 0-7 | 1 | Yes | No |
| SU | 0-6 | 0 | Yes | No |
| MU | 0-7 | 1 | Yes | Yes |
| MU | 0-6 | 0 | Yes | Yes |
| SU or MU* | 7 | 0 | No | No |

| CID | Commenter | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 5038 | Sandhya Patil | 123.47 | 9.28.3 | This sentence reffers to the heading of its own section. | Re-word the sentence | ACCEPTED: remove offending part of sentence |

Discussion:  The referenced sentence discusses the BW subfield of the MFB subfield for the case of solicited MFB.  The BW subfield is reserved for the case of solicited MFB  (see Table 8-13b), and thus the referred to clause is not needed.

Proposed Resolution: eliminate clause discussing BW subfield for solicited MFB.

Tgac Editor:

Please revise 9.28.3 as follows:

When computing the MFB estimate for an MFB requester, the MCS, N_STS and, for unsolicited MFB, BW

field values in the MFB subfield from the MFB responder shall indicate a combination which is in the responder's

VHT Rx Supported MCS Set (see 9.7.11.1 (VHT Rx Supported MCS Set)) and the requester’s VHT

Tx Supported MCS Set (see 9.7.11.2 (VHT Tx Supported MCS Set)), where the BW for solicited MFB is that

defined in 9.28.3 (Link adaptation using the VHT variant HT Control field).

| CID | Commenter | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 5250 | Simone Merlin | 124.45 | 9.28.3 | "If the N_STS subfield is set to a smaller value than the RXVECTOR.." use small capital 'l' "the MFB requester will transmit the first N_STS space-time streams in the corresponding PPDU carrying MRQ." not well defined what "first N_STS" streams means; is there an ordering defined? | clarify | ACCEPT |

Discussion: The commenter is correct that the “first N_STS streams” is not clearly defined.

Proposed Resolution:  Define the first N_STS streams to correspond to the first N_STS columns of the spatial mapping matrix Q when the MFB is based on an NDP or on an SU PPDU.  If the MFB is based on an MU PPDU, the first N_STS streams correspond to columns M_u +1, ,...,M_u+N_STS,u   where u  is the MFB requestee’s user number within the MU PPDU.

Tgac Editor:

Please add the following note to 9.28.3 (Link adaptation using the VHT variant HT Control field) immediately after the paragraph ending with “will transmit the first N_STS space-time streams in the corresponding PPDU carrying the MRQ.”

Note: If the MFB is based on an NDP or an SU PPDU, the first N_STS space-time streams correspond to columns 1,..,N_STS of the spatial mapping matrix Q.  If the MFB is based on an MU PPDU, the first N_STS space-time streams correspond to columns M_u + 1,..,M_u + N_STS,u  of the spatial mapping matrix Q, where u is the MFB requestee’s user number within the MU PPDU.

| CID | Commenter | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 5251 | Simone Merlin | 124.53 | 9.28.3 | "under the assumption that the MFB requester" this para applies also to unsolicited, in which case there is no requester | reword to be consistent with the rest of paragraph | Accepted |

Discussion: The full sentence in question is: “If the MFB is in the same PPDU as a VHT Compressed Beamforming frame, the MFB responder shall estimate the recommended MFB under the assumption that the MFB requester will use the steering matrices contained therein for performing an SU beamformed transmission”

In the case of solicited or unsolicited MFB, it is the “beamformer” that will use the steering matrices.

Proposed Resolution: Change “MFB requester” to “beamformer”

Tgac Editor:

On page 124, line 53 (D2.0) please replace “MFB requester” to “beamformer”