IEEE P802.11
Wireless LANs

| [place document subject title text here] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: YYYY-MM-DD |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Nihar Jindal | Broadcom |  |  |  |
| Joonsuk Kim | Broadcom |  |  |  |

Abstract

This document proposes a resolution to CID 5163.

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5163 | 207.40 | 22.3.8.2.3 | Ambiguity in use of field "BEAMFORMED" | It is not clear what qualifies as a "beamforming steering matrix". Where is the boundary between a spatial mapping matrix and a beamforming steering matrix? Is there an underlying requirement here that can be captured better? What was the initial intent of this BEAMFORMED field? | Revised |

Discussion:

A beamforming steering matrix is clearly defined in 20.3.11.11.2, so there is no ambiguity in that regard here. Because the intention of the BEAMFORMED parameter is to imply if smoothing should be applied or not, a note has been added in this regard.  Note that this definition is identical to the one proposed for the BEAMFORMED field in TXVECTOR (CID 5115).

Comment Status: 	Revised.

TGac editor: please modify the content of Value column in Table 22-11, section 22.3.8.2.3 pg207/ln41 as following.

Set to 1 if a Beamforming steering matrix is applied to the waveform in an SU transmission as described in 20.3.11.11.2 (Spatial mapping), set to 0 otherwise.

NOTE – when beamforming bit = 1 smoothing is not recommended