IEEE P802.11
Wireless LANs

| 802.11 TGac WG Letter Ballot LB187 Proposed resolutions to comments on Clauses 8 and 9 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-05-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eric Wong | Broadcom Corporation | 190 Mathilda Place, Sunnyvale, CA 94086 | +1 408 922 6672 | ewong@broadcom.com |
| Matthew Fischer | Broadcom Corporation | 190 Mathilda Place, Sunnyvale, CA 94086 | +1 408 543 3370 | mfischer@broadcom.com |

Abstract

This submission contains proposed comment resolutions to comments received during WG letter ballot 187.

The comments assigned to the author in Clauses 8 and 9 are:

4342, 4391, 4393, 4478, 4491, 4496, 4786, 4800, 4801, 4823, 4918, 4982, 5031, 5032, 5033, 5039, 5066, 5067, 5068, 5082, 5083, 5084, 5258, 5263, 5376

Deferred:

4967

# Comments

CID 4342

| 4342 | Brian Hart | 76.33 | 8.4.2.166 | AID defined in 8.4.1.8 refers to 8.2.4.2 where it says MSBs = 1,1 "AID in PS-Poll frames" | Now sent in more that just PS-Poll frames - update this language |
| --- | --- | --- | --- | --- | --- |

Discussion:

Language of 8.2.4.2 relates only to the Duration/ID field and TGac is not changing the definition of the Duration/ID field – i.e. while the commenter is correct in that AID appears in frames other than PS-Poll, the AID does not appear in those other frames within the Duration/ID field.

However, in those additional frames that per the TGac draft will now carry the AID, the AID is described incorrectly. Specifically,

Propossed Resolution:

Revise – Tgac editor to make changes shown under the heading CID 4342 within document (11-12-0584r3) which make changes that correctly account for the differences among the AID value, some portion of the AID value, the AID field, and the AID element.

TGac editor, change the name of the “AID” field found in figure “Figure 8-291 – STA Info Field” and within table “Table 8-18a—STA Info subfields” within subclause “8.3.1.20 VHT NDP Announcement frame format” of 802.11 TGac draft 2.1, to “AID12” and change the text in the Description box for the “AID” row of table “Table 8-18a—STA Info subfields” as shown:

Contains the 12 least significant bits of the AID of a STA expected to process

the following NDP frame and prepare the sounding

feedback. Equal to 0 if the STA is an AP, mesh

STA or STA that is a member of an IBSS.

TGac editor, change the contents of the Notes box of the row containing the value “AID” in the Information column of table “Table 8-199—DLS Request frame Action field format” within subclause “8.5.4.2 DLS Request frame format” of TGac Draft 2.2 as shown:

The AID element

of the STA sending the frame is present if

dot11VHTOptionImplemented is true.

TGac editor, change the contents of the Notes box of the row containing the value “AID” in the Information column of table “Table 8-200—DLS Response frame Action field format” within subclause “8.5.4.3 DLS Response frame format” of TGac Draft 2.1 as shown:

The AID element

of the STA sending the frame is present if

dot11VHTOptionImplemented (#4028)is true.

TGac editor, do what you just did for Table 8-199 and Table 8-200 for the tables found in subclauses “8.5.13.2 TDLS Setup Request frame format” and “8.5.13.3 TDLS Setup Response frame format” of TGac Draft 2.

CID 4391, 4823 and 5031

| 4391 | Brian Hart | 106.29 | 9.12.5 | or the TXVECTOR parameter | or the RXVECTOR parameter |
| --- | --- | --- | --- | --- | --- |
| 4823 | Mark RISON | 106.30 | 9.12.5 | On receive you don't directly know the TXVECTOR, only the RXVECTOR | Change "TXVECTOR" to "RXVECTOR" |
| 5031 | Sandhya Patil | 106.29 | 9.12.5 | it should be "the RXVECTOR parameter FORMAT is equal to 1 | Change to "RXVECTOR" |

Context: (TGac draft 2.2 P108L36 )
“A received PSDU is determined to be an A-MPDU when the associated PHY-RXSTART.indication primitive RXVECTOR parameter AGGREGATION parameter is equal to 1 or the TXVECTOR parameter FORMAT is equal to VHT.”

Commenter proposed change:

“A received PSDU is determined to be an A-MPDU when the associated PHY-RXSTART.indication primitive RXVECTOR parameter AGGREGATION parameter is equal to 1 or the RXVECTOR parameter FORMAT is equal to VHT.”

Proposed Resolution:

Accept. Change text at cited location from “TXVECTOR” to “RXVECTOR”.

CID 4393

| 4393 | Brian Hart | 107.60 | 9.12.8 | "does not generate" seems to strong | cannot solicit. Ditto P107L63 |
| --- | --- | --- | --- | --- | --- |

Context: (TGac draft 2.2 P109L52)
“A data MPDU cannot indicate an Ack Policy of “Implicit Block Ack”, and does not generate a Block Ack response.”

“A data MPDU could indicate an Ack Policy of “Normal Ack”, which generates an ACK immediate response.

No Block Ack agreement is necessary in this case.”

Commenter proposed change:

“A data MPDU cannot indicate an Ack Policy of “Implicit Block Ack”, and does not solicit a Block Ack response.”

“A data MPDU could indicate an Ack Policy of “Normal Ack”, which solicit an ACK immediate response.

No Block Ack agreement is necessary in this case.”

Proposed Resolution:

Accept. Change text at cited location from “generate” to “solicit”.

CID 4478

| 4478 | Carlos Cordeiro | 104.51 | 9.12.2 | The para starts with "An STA shall not transmit .... in the HT Capabilities element". DBand STAs do not use the HT Capabilities element. Therefore, this statement is wrong for DBand STAs. | Include "non-DBand" before "STA" |
| --- | --- | --- | --- | --- | --- |

Context: (TGac draft 2.2 P106L57)
“An STA shall not transmit an A-MPDU in an HT_MF or HT_GF format PPDU that is longer than the value indicated by the Maximum A-MPDU Length Exponent field in the HT Capabilities element sent by the intended receiver.”

Commenter proposed change:

“A non-DBand STA shall not transmit an A-MPDU in an HT_MF or HT_GF format PPDU that is longer than the value indicated by the Maximum A-MPDU Length Exponent field in the HT Capabilities element sent by the intended receiver.”

Proposed Resolution:

Accept. Change text at cited location from “An STA” to “A non-DBand STA”.

CID 4496

| 4496 | Clint Chaplin | 107.46 | 9.12.8 | No editor's instruction for this sub-clause | Provide editors instruction for this subclause |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accept. However, no change is required since this change has already been made in TGac draft 2.2.

CID 4786

| 4786 | Mark RISON | 106.11 | 9.12.4 | There is no Minimum MPDU Start Spacing subfield in the A-MPDU Parameters fields of VHT Capabilities elements | Either add an A-MPDU Parameters field back in the VHT Capabilities element and put a MMSS subfield in there or say you just use the MMSS from the HT Capabilities element |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Revise. No change is required, since this text will be removed by CID 5263.

CID 4800

| 4800 | Mark RISON | 69.36 | 8.4.2.160.2 | "ignored" is a bit wooly | Change "ignored" to "reserved" |
| --- | --- | --- | --- | --- | --- |

Discussion:

This sentence is removed in TGac draft 2.2, because of CID 4036.

Proposed Resolution:

Revise. The resolution of another comment (CID#4036) has deleted the cited text.

CID 4801

| 4801 | Mark RISON | 72.35 | 8.4.2.161 | "Set to 0" is a bit close-ended | Change "Set to 0" to "Reserved" |
| --- | --- | --- | --- | --- | --- |

Context: (TGac draft 2.2 P74L21)
“Set to 0 for 20 MHz or 40 MHz operating channel width.”

Commenter proposed change:

“Reserved otherwise.”

Proposed Resolution:

Revise.  Change text at cited location “Set to 0 for 20 MHz or 40 MHz operating channel width.” to “Reserved otherwise.”

CID 4918

| 4918 | Matthew Fischer | 104.54 | 9.12.2 | Wrong verb sense. The intended receiver sends lots of things which the transmitter might never have received. The transmitter behavior must be predicated on the information that is available to the transmitter, and that information is the frames that have been received from the intended recipient. | Change "sent by" to "received from" - another instance needs changing on line 56 |
| --- | --- | --- | --- | --- | --- |

Context:  (TGac draft 2.2 P106L62)
“A STA shall not transmit an A-MPDU in an HT_MF or HT_GF format PPDU that is longer than the value indicated by the Maximum A-MPDU Length Exponent field in the HT Capabilities element sent by the intended receiver.”

Commenter proposed change:

“A STA shall not transmit an A-MPDU in an HT_MF or HT_GF format PPDU that is longer than the value indicated by the Maximum A-MPDU Length Exponent field in the HT Capabilities element received from the intended receiver.”

Proposed Resolution:

Accept. Change text in cited location “sent by” to “received from”.

CID 4967

| 4967 | Nir Shapira | 71.18 | 8.4.2.160.3 | RX/TX highest supported data rate is in 1Mb/s resolution. Since actual rates are not integer multiple of 1Mb/s (e.g. 1462.5), suggest to spefically mention data rate should be rounded up in this field. Otherwise there is a potential for confusion. | change as suggested |
| --- | --- | --- | --- | --- | --- |

Discussion:

Commenter is correct. However, rounding up can be misleading since it can indicate a rate that is not supported. Therefore, proposed following revision.

Context: (TGac draft 2.2 P73L15 and L28)
“In units of 1 Mb/s.”

Commenter proposed change:

“Largest rate in units of 1 Mb/s less than or equal to actual data rate.”

Proposed Resolution:

Deferred. Request to defer after discussion in the group.

CID 4982

| 4982 | Osama Aboulmagd | 107.35 | 9.12.7 | The EoFis supposed to be set to one in VHT single MPDU format. Shouldn't the "may" be "switched to "shall" in this statement? | change if necessary. |
| --- | --- | --- | --- | --- | --- |

After discussion in the group, the group has decided to change reject the resolution.

Proposed Resolution:

Reject. The transmitter may still set EOF field to 0, even for a single MPDU included in an A-MPDU.

CID 5032

| 5032 | Sandhya Patil | 108.07 | 9.12.6 | For the SU PPDU case, the padding process is similar to 802.11n and is taken care by pre-EOF padding. Is this EOF padding applicable to a user in a SU PPDU | Remove "SU PPDU" |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Revise. See editing instructions for CID 4392, 4660 which remove the text.

CID 5033

| 5033 | Sandhya Patil | 107.26 | 9.12.6 | EOF padding shall not be added before any A-MPDU subframe with non zero length. | add "with non-zero MPDU length" |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Reject. The original statement is correct, since EOF pad octet (not EOF delimiter) is appended at the end of all A-MPDU subframes. Refer to Figure 8-503 (TGac draft 2.2). Also refer to CID 4392, which proposed clarification for this EOF pad octet, and it is not EOF delimiter.

CID 5039

| 5039 | Sandhya Patil | 67.24 | 8.4.2.160.2 | It is common that an AP overrides the STA's SINR and MCS feedback due to change in group, transmit power offset, change in traffic pattern and size of MPDU, Etc. To compute an MCS optimal for the STA, knowledge of Its receiver capability is helpful. When VHT Link Adaptation Capable subfield = 0, (i.e. does not support link adaptation) this information of STA receiver capability will greatly assist the AP in computing the MCS level and reduces trial and error. The VHT capabilities info field does not include any information on the STA's receiver capability. | The VHT Capabilities Info field should include some minimal information of the receiver capability to achieve quick convergance of MCS. |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Reject. The commenter highlights a way to further improve MCS selection. However, it is still unclear that the amount of improvement that can be achieved is worthy of the extra complexity.

CID 5066

| 5066 | Sigurd Schelstraete | 68.36 | 8.4.2.160.1 | Confusing definition | Replace definition with "If SU beamformee capable, set to the maximum number of supported beamformer antennas minus one" |
| --- | --- | --- | --- | --- | --- |

Context: (TGac draft 2.2 P70L36)
“If SU beamformee capable, set to maximum value minus 1.”

Commenter proposed change:

“If SU beamformee capable, set to the maximum number of supported beamformer antennas value minus 1.”

Proposed Resolution:

Accept. Change text at cited location to “If SU beamformee capable, set to the maximum number of supported beamformer antennas value minus 1” from “If SU beamformee capable, set to maximum value minus 1”

CID 4491 and 5067

| 4491 | Clint Chaplin | 68.43 | 8.4.2.160.2 | "If SU beamformer capable, set to value minus 1" | "If SU beamformer capable, set to maximum value minus 1" |
| --- | --- | --- | --- | --- | --- |
| 5067 | Sigurd Schelstraete | 68.43 | 8.4.2.160.1 | Confusing definition | Replace definition with "If SU beamformee capable, set to the maximum supported value of NUM_STS minus one" |

Context: (TGac draft 2.2 P70L43)
“If SU beamformer capable, set to value minus 1.”

Commenter proposed change:

“If SU beamformer capable, set to the maximum supported value of NUM_STS value minus 1.”

Proposed Resolution:

Accept. Change text at cited location from “If SU beamformer capable, set to value minus 1.” to “If SU beamformer capable, set to the maximum supported value of NUM_STS value minus 1.”

CID 5068

| 5068 | Sigurd Schelstraete | 69.44 | 8.4.2.160.1 | Meaning of "association" is ambiguous | "association" could mean either the process of associating or the total time that a STA remains associated with the AP. Change wording to clarify that the latter is intended here. Same comment for lines 46, 48 and 50. |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accept. Similar changes have already been made in TGac draft 2.2 for resolutions to CIDs 5268 and 5269.

CID 5082 and 5083

| 5082 | Sigurd Schelstraete | 107.04 | 9.12.6 | Value of subframe padding octet unspecified | No value is specified for subframe padding octet. Explicitly state that the value is unspecified and vendor discretionary. |
| --- | --- | --- | --- | --- | --- |
| 5083 | Sigurd Schelstraete | 107.10 | 9.12.6 | Value of EOF padding octet unspecified | No value is specified for EOF padding octet. Explicitly state that the value is unspecified and vendor discretionary. |

Proposed Resolution:

Accept. Add new sentence at cited location below “The value of EOF padding octet is unspecified.”

TGac editor, please change subclause “9.31.5 VHT sounding protocol” 802.11 TGac draft 2.2 as shown:

Padding is then added for each user such that the resulting A-MPDU contains exactly PSDU_LENGTH octets

for that user as follows:

— First, while A-MPDU_Length < PSDU_LENGTH for that user and A-MPDU_Length mod 4 != 0,

add a subframe padding octet and increment A-MPDU_Length by 1

— Then, while A-MPDU_Length + 4 <= PSDU_LENGTH for that user, add an A-MPDU subframe

with 0 in the MPDU Length field and 1 in the EOF field and increment A-MPDU_Length by 4

— Finally, while A-MPDU_Length < PSDU_LENGTH for that user, add an EOF padding octet and

increment A-MPDU_Length by 1

The value of EOF padding octet is unspecified.

CID 5084

| 5084 | Sigurd Schelstraete | 107.33 | 9.12.7 | Use of EOF field for non-zero length subframes | Is there a reason the last A-MPDU subframe with non-zero length can not have EOF set to one, even if it's not the only subframe? This looks like a valid use of EOF. |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Reject. A non-zero length MPDU delimiter with EOF set to 1 is the condition that defines the VHT single PPDU when only one non-zero length MPDU delimiter is present in the VHT PPDU. The response to a VHT single PPDU is a normal ACK, instead of a BA frame. To allow a non-zero length MPDU delimiter with EOF set to 1 anywhere within a multi-MPDU VHT PPDU would potentially confuse the response state machine of a MAC implementation, or at least, complicate it. There is no added value in allowing the last MPDU of a VHT AMPDU that is not the first MPDU to have EOF=1, but allowing this condition does potentially complicate the design of a MAC implementation.

CID 5263

| 5263 | Simone Merlin | 106.10 | 9.12.4 | "the value of minimum MPDU start spacing that applies is the maximum value in the Minimum MPDU Start Spacing subfields of the A-MPDU Parameters fields of the VHT Capabilities elements across all VHT STAsa ssociated with the transmitting AP or across all peer VHT mesh STAs of the transmitting mesh STA." there is no such a subfield in VHT Capab. | refer to the HT field |
| --- | --- | --- | --- | --- | --- |

Context: (TGac draft 2.2 P108L11)
“If the PPDU is an HT PPDU, the value of minimum MPDU start spacing that applies is the maximum value in the Minimum MPDU Start Spacing subfields of the A-MPDU Parameters fields of the HT Capabilities elements across all HT STAs associated with the transmitting AP or across all peer HT mesh STAs of the transmitting mesh STA. If the PPDU is a VHT PPDU, the value of minimum MPDU start spacing that applies is the maximum value in the Minimum MPDU Start Spacing subfields of the A-MPDU Parameters fields of the VHT Capabilities elements across all VHT STAs associated with the transmitting AP or across all peer VHT mesh STAs of the transmitting mesh STA.”

Commenter proposed change:

“If the PPDU is an HT or VHT PPDU, the value of minimum MPDU start spacing that applies is the maximum value in the Minimum MPDU Start Spacing subfields of the A-MPDU Parameters fields of the HT Capabilities elements across all HT or VHT STAs associated with the transmitting AP or across all peer HT or VHT mesh STAs of the transmitting mesh STA.”

Proposed Resolution:

Accept. Change cited text from “all HT” to “all HT or VHT”.

CID 5258 and 5376

| 5258 | Simone Merlin | 69.54 | 8.4.2.160.2 | "An AP that sets MU Beamformer Capable to 1 can transmit an MU VHT PPDU with only one non-zero MU[p] NSTS subfield, for . However, a STA that sets MU Beamformee Capable to 0 is not required to be able to demodulate an MU VHT PPDU with only one non-zero MU[p] NSTS subfield" this note should be normative, unless already implied by other normative text | make it normative in appropriate clause |
| --- | --- | --- | --- | --- | --- |
| 5376 | Youhan Kim | 69.54 | 8.4.2.160.2 | There should be a normative statement prohibiting an AP from transmitting an MU packet with only one non-zero MU[p] NSTS subfield to a STA which has set MU Beamformee Capable to 0. | Add "An AP that sets MU Beamformer Capable to 1 shall not transmit an MU VHT PPDU with only one non-zero MU[p] NSTS subfield, for 0<=p<=3, to a STA that sets MU Beamformee Capable to 0". This normative sentence may need to be inserted in another section. |

Discussion:

Agree with commenters.

Proposed Resolution:

Revise. This note is edited in document 11-12/540r0 (CID 4315 and 4317). After discussion in the group, add new subclause 9.29.4 VHT Beamforming as indicated.

9.29.4 VHT Beamforming

A VHT MU Beamformer may transmit a VHT MU PPDU with only one non-zero TXVECTOR NUM_STS[p] subfield, for 0 <= p < = 3.

A VHT MU Beamformer shall not transmit a VHT MU PPDU with only one non-zero RXVECTOR NUM_STS[p] subfield, for 0 <= p <=3 to a STA whose MU Beamformee Capable field is equal to 0.