IEEE P802.11
Wireless LANs

| 11af PHY Clause 4 proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-07-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Peter Ecclesine | Cisco Systems | 170 W. Tasman Dr., MS SJ-14-4, San Jose, CA 95134-1706 | +1-408-527-0815 | petere@cisco.com |

Abstract

Submission for candidate P802.11af draft Clause 4 PHY text for next draft.

The editing instructions are shown in bold italic. Four editing instructions are used: change, delete, insert, and replace. Change is used to make corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed by using strikethrough (to remove old material) and underscore (to add new material). Delete removes existing material. Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instruction. Replace is used to make changes in figures or equations by removing the existing figure or equation and replacing it with a new one. Editorial notes will not be carried over into future editions because the changes will be incorporated into the base standard.

This amendment’s baseline is IEEE Std 802.11™–2012, as amended by

- Amendment 1 802.11ae-2012

- Amendment 2 802.11aa-2012

- Amendment 3 P802.11ad Draft 8.0

- Amendment 4 P802.11ac Draft 3.0

Across the globe, there are a large number of broadcast channel frequencies and rules, however in the United States, the FCC channelization rules are known and can be specified in detail. Other countries using 6 MHz broadcast channelization may allow operation above channel 51, so the global table for 6 MHz channelization states no limits.

TGaf Editor: Insert new Clause 4.3.10b text as follows:

Insert a new subclause 4.3.10b following 4.3.10a Very High Throughput (VHT) station (STA):(#2123)

4.3.10b TV High Throughput (TVHT) station (STA)

The IEEE 802.11 TVHT STA operates in broadcast frequency bands below 1 GHz.

A TVHT STA is a VHT STA that excludes features supported as an HT STA, and supports VHT features identified in Clause 8, Clause 9, Clause 10, Clause 13(#4781)(#5334) and Clause 23. References to VHT features in Clause 6, Clause 8, Clause 9, Clause 10, Clause 13 and Clause 23 apply to TVHT features as well, unless otherwise stated.   The main PHY features in a TVHT STA that are not present in a VHT STA are the following(#5004):

- Mandatory support for 6 MHz, 7 MHz and/or 8 MHz channel widths (single width channels)

- Mandatory support for TVHT PPDUs(#4734)

- Optional support for two non-adjacent single width channels

- Optional support for two non-adjacent double width channels

The main MAC features in a TVHT STA are the following(#5005):

- Mandatory support for the A-MPDU padding of TVHT PPDU

- Mandatory support for VHT single MPDU

- Mandatory support for responding to BW indication in RTS

- Optional support for MPDUs of up to 11 454 octets

- Optional support for A-MPDU(#4271) pre-EOF padding of up to 1 048 575 octets

These TVHT features are available to VHT STAs associated with a TVHT AP in a BSS. A subset of the TVHT features is available for use between two TVHT STAs that are members of the same IBSS. (#4931)

The support for TVHT transmit beamforming sounding and MU PPDUs in a TVHT AP and more than one TVHT STA within(#4272) a TVHT BSS enables the optional use of downlink MU-MIMO.(#4975) The use of 20 MHz, 40 MHz, 80 MHz or 160 MHz channel widths is not permitted for STAs operating as TVHT STAs. The use of HT features, such as RIFS, is not permitted for STAs operating as TVHT STAs.