IEEE P802.11
Wireless LANs

| 802.11 TGac WG Letter Ballot LB187 Proposed resolutions to comments on Subclause 9.19.2.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Liwen Chu | STMicroelectronics | 2525 Augustine Drive Santa Clara, CA 95054 |  | Liwen.chu@st.com |

Abstract

This submission contains proposed comment resolutions to comments received during WG letter ballot 188.

The comments included are non-editorial comments on Subclause 9.19.2.5.

There are xx such comments: 6108, 6109, 6110, 6111, 6382.

| CID | Page | Clause | Comment | Proposed Change | Proposed Resolution |
| --- | --- | --- | --- | --- | --- |
| 6108 | 130.09 | 9.19.2.5 | VHT beamforming frame sequence is missing here. | Add the following text after L39: ----The recognition of one or more VHT Compressed Beamforming frames sent by the beamforme shall also be accepted as successful acknowledgment of the NAD Announcement+NDP frame or Beamforming Report Poll frame. | Revise. Editorial Instruction: Add the following text after P130 L39: ----The recognition of one or more VHT Compressed Beamforming frames sent by the beamforme shall be accepted as successful acknowledgment of the NDP Announcement+NDP frame or Beamforming Report Poll frame. |
| 6109 | 131.01 | 9.19.2.5 | This contradicts with the successful transmission definition. | Change to "In addition, the backoff procedure may be invoked for an EDCAF when the non-initial transmission by the TXOP holder fails." | Agree. Discussion: When a non-initial transmission by the TXOP holder is an A-MPDU, the acknowledgement is BlkAck. So the failure decision per one or more MPDUs in a non-initial frame PPDU is not right. |
| 6110 | 130.09 | 9.19.2.5 | Adding "of an MPDU" In L9 is wrong since in A-MPDU case the response frame is not for "an MPDU". "After transmitting an MPDU (even if it is carried in an A-MPDU or as part of an MU PPDU)" is not right since in A-MPDU case the response frame is not for "an MPDU". | Change the text from L9 to L18 to: For the purposes of this subclause, successful transmission and transmission failure of are defined as follows: ---- After transmitting an MPDU or multiple MPDUs in a PPDU that requires an immediate frame as a response, the STA shall wait for a timeout interval of duration of aSIFSTime + aSlotTime + aPHY-RX-START-Delay, starting at the PHYTXEND. confirm. If a PHYRXSTART.indication does not occur during the timeout interval, the STA concludes that the transmission of the MPDU has failed. | Counter. Discussion: A transmission may include one non-A-MPDU frame, an A-MPDU or multiple A-MPDUs. The commenter is right that for an A-MPDU, the successful of the transmission is not an MPDU. The BlkAck means the successful of the A-MPDU (some of the MPDUs in the A-MPDU may not be acknowledged). In an MU PPDU, if there are several A-MPDUs that do not require immediate acknowledge, the A-MPDU in MU PPDU that requires immediate acknowledge will decide if the transmission is successful. Editorial Instruction: Change the text from L9 to L18 to: A transmission can include a non-a-MPDU frame, an A-MPDU or multiple A-MPDUs in a PPDU. For the purposes of this subclause, successful transmission and transmission failure are defined as follows: ---- After transmitting an non-a-MPDU frame or A-MPDU that requires an immediate frame as a response, the STA shall wait for a timeout interval of duration of aSIFSTime + aSlotTime + aPHY-RX-START-Delay, starting at the PHYTXEND. confirm. If a PHYRXSTART.indication does not occur during the timeout interval, the STA concludes that the transmission of the non-a-MPDU frame or the A-MPDU has failed. Editorial Instruction: Add the following text after P130 L39: ---- The transmission of an A-MPDU that does not require an immediate frame as a response is defined as a successful transmission, unless it is in a MU PPDU which includes an A-MPDU with Implicit Blockack Request indication or a non-A-MPDU frame with Normal ACK indication. |
| 6111 | 130.45 | 9.19.2.5 | This changes the behavior that is defined in 11n. In 11n, if the blkack of the last A-MPDU is received, the value of CW[AC] shall be reset to CWmin[AC]. The text here says that the value of CW[AC] will not be reset to CWmin[AC] if blkack does not acknowledge all MPDUS in the last A-MPDU. | Change to the original text in 802.11TM-2012. | Revise. Editorial Instruction: Change P130L45 bullet b) to the text in 802.11TM-2012: b) The final transmission by the TXOP holder initiated during the TXOP for that AC was successful and the TXNAV timer has expired. Change P130L45 bullet c) to: c) The initial transmission of a TXOP of that AC fails. |
| 6382 | 130.13 | 9.19.2.5 | Not sure why we need "(even if it is carried in an A-MPDU or as part of an MU PPDU)". An MPDU that requires an immediate response is exactly that. | Suggest that we remove the quoted text. | Revise. See 6110. |