IEEE P802.11
Wireless LANs

| 802.11 TGac WG Letter Ballot LB187 Proposed resolutions to comments on Subclause 9.19.2.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Liwen Chu | STMicroelectronics | 2525 Augustine Drive Santa Clara, CA 95054 |  | Liwen.chu@st.com |

Abstract

This submission contains proposed comment resolutions to comments received during WG letter ballot 188.

The comments included are non-editorial comments on Subclause 9.19.2.5.

There are xx such comments: 6108, 6109, 6110, 6111, 6382.

| CID | Page | Clause | Comment | Proposed Change | Proposed Resolution |
| --- | --- | --- | --- | --- | --- |
| 6108 | 130.09 | 9.19.2.5 | VHT beamforming frame sequence is missing here. | Add the following text after L39: ----The recognition of one or more VHT Compressed Beamforming frames sent by the beamforme shall also be accepted as successful acknowledgment of the NAD Announcement+NDP frame or Beamforming Report Poll frame. | Reject. The first bullet and the second bullet already cover the VHT beamforming frame sequence. |
| 6109 | 131.01 | 9.19.2.5 | This contradicts with the successful transmission definition. | Change to "In addition, the backoff procedure may be invoked for an EDCAF when the non-initial transmission by the TXOP holder fails." | Revise. Discussion: transmission result of “more MPDUs” is not described in P130L9 to P130L39. It is better to remove it. Editorial Instruction: Change P131L01to P131L02 as following: “In addition, the backoff procedure may be invoked for an EDCAF when the transmission of the MPDU in a non-initial frame PPDU by the TXOP holder fails” |
| 6110 | 130.09 | 9.19.2.5 | Adding "of an MPDU" In L9 is wrong since in A-MPDU case the response frame is not for "an MPDU". "After transmitting an MPDU (even if it is carried in an A-MPDU or as part of an MU PPDU)" is not right since in A-MPDU case the response frame is not for "an MPDU". | Change the text from L9 to L18 to: For the purposes of this subclause, successful transmission and transmission failure of are defined as follows: ---- After transmitting an MPDU or multiple MPDUs in a PPDU that requires an immediate frame as a response, the STA shall wait for a timeout interval of duration of aSIFSTime + aSlotTime + aPHY-RX-START-Delay, starting at the PHYTXEND. confirm. If a PHYRXSTART.indication does not occur during the timeout interval, the STA concludes that the transmission of the MPDU has failed. | Reject. “an MPDU” in P130L12 is from 802.11TM-2012. The commenter is welcome to submit the comment to MC. |
| 6111 | 130.45 | 9.19.2.5 | This changes the behavior that is defined in 11n. In 11n, if the blkack of the last A-MPDU is received, the value of CW[AC] shall be reset to CWmin[AC]. The text here says that the value of CW[AC] will not be reset to CWmin[AC] if blkack does not acknowledge all MPDUS in the last A-MPDU. | Change to the original text in 802.11TM-2012. | Revise. Discussion: transmission result of “all MPDUs” is not described in P130L9 to P130L39. It is better to remove it. Editorial Instruction: Change P131L45 to P131L48 as following: “b) The final transmission of the MPDU in the final PPDU transmitted by the TXOP holder initiated during the TXOP for that AC was successful as defined in this subclause and the TXNAV timer has expired.” |
| 6382 | 130.13 | 9.19.2.5 | Not sure why we need "(even if it is carried in an A-MPDU or as part of an MU PPDU)". An MPDU that requires an immediate response is exactly that. | Suggest that we remove the quoted text. | Reject. 802.11TM-2012 includes “even if it is carried in an A-MPDU”. This should be removed. . |