IEEE P802.11
Wireless LANs

| 802.11 TGac WG Letter Ballot LB187 Proposed resolutions to comments on Subclause 8.2.5.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-07-26 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Liwen Chu | STMicroelectronics | 2525 Augustine Drive Santa Clara, CA 95054 |  | Liwen.chu@st.com |

Abstract

This submission contains proposed comment resolutions to comments received during WG letter ballot 188.

The comments included are non-editorial comments on Subclause 8.2.5.2.

There are xx such comments: 6084, 6085, 6234, 6670, 6706, 6792, 6793.

| CID | Page | Clause | Comment | Proposed Change | Proposed Resolution |
| --- | --- | --- | --- | --- | --- |
| 6084 | 38.34 | 8.2.5.2 | Currently in Tgac D3.0, multiple protection is used for NAPA/BFRP, and TXOP limit can't be the limit to the beamforming report. The rules make things clearer and provide flexibility. But it is not enough for a STA to set the Duration field. | fix the problem. | Revise. See the comment 6084 discussion and proposed editorial instruction in 11-12/0856 |
| 6085 | 39.03 | 8.2.5.2 | NDPA is missing from the pending frame | Change to "Any NDPA, NDP or Beamforming Report Poll frame transmissions and explicit feedback response frames" | Reject. The first bullet in P38 L65 already includes NDPA. |
| 6234 | 38.52 | 8.2.5.2 | "always use" -> needs a normative reference or to be moved to clause 9 | Add a clause 9 reference or move to clause 9. Also "multiple protection settings" or just "M/P setting"? | Reject. IEEE 802.11TM 2012 uses the same language in Duration/Id setting for PSMP etc. The commenter is welcome to submit the comment to Revmc group. |
| 6670 | 39.18 | 8.2.5.2 | "They are transmitted at a rate no lower than that which would be used if they were control response frames (see 9.7.5.6 (Rate selection for other data and management frames))." The rules in that section do not define the behavior implied by this bullet. Also this behavior should not be enforced. STA shoud decide on the rate of big packets. | remove this bullet | Accept. |
| 6706 | 39.12 | 8.2.5.2 | The estimated duration for a VHT Compressed Beamforming frame response can be optimized when the response MCS is approximately known. Even though the text is clear that the duration is an estimated duration, it may be more clear to change "is determined" into "may be determined". | On P39L12 change "is determined" into "may be determined". | Revise. See CID 6670. |
| 6792 | 39.17 | 8.2.5.2 | Not certain if you should be using a pronoun as the subject within a bulleted list. I assume that the antecedent for the pronoun would be the text preceding the list, and not text within a preceding bullet item. | Change "They" to "All segments" | Revise. See comment 6670 that the bullet is removed. |
| 6793 | 39.19 | 8.2.5.2 | Not clear where these fields are found. | Change "values" to "value in all segments" | Revise. Editorial instruction: Change P39 L19 to “”The subfield values of VHT MIMO Control field in each segment are as follows:” |

Discussion:

Comment 6084:

The original NAV updating algorithm assumes that the TXOP holder fully controls the frame transmission time. Now the beamforming training breaks such assumption. The NAV updating algorithm would not work anymore.

As the commenter indicated, in Tgac D3.0, the multiple protection is used for NDPA/BFRP, and the beamforming training can be transmitted without considering TXOP limit. Here is an example: the TXOP holder tries to do beamforming training to 4 TXOP responders before transmitting data frames (in DL MU PPDU) based on its extimation of beamforming training time plus data frames requirement and TXOP limit. Since multiple TXOP responders use more time than the TXOP holder’s extimation, the AP can not transmit DL MU PPDU any more. However the AP decides to finish the beamforming training although the transmission of Beamforming Report Poll to the third beamforme will be more than the TXOP limit. This will not violate the rules in 9.19.2.2.

The following issues should be fixed:

1), The calculated Duration per the NAV updating algorithm may be <=0 before finishing the frame sequence when TXOP Limit is 0.

2), The TXOP holder first decides to do beamforming training and transmit MPDU, A-MPDU, MU-PPDU when the TXOP Limit is not 0. Then the TXOP holder finds that the remaining TXOP is not enough for the following MPDU, A-MPDU, MU-PPDU transmission. Since there is no MDPU transmission in the TXOP, the TXOP holder may not obey the original TXOP limit.

3), The TXOP holder decides to do beamforming training and the estimated time of beamforming training is more than TXOP Limit. Since there is no MDPU transmission in the TXOP, the TXOP holder may not obey the original TXOP limit.

4), The responding frame may calculate a Duration value which is <0.

When the TXOP limit is 0, per 8.4.2.31, a MSDU/A-MPDU/MU-PPDU can be transmitted. But the algorithm may not allow this if the TXOP responder uses the time more than the time that the TXOP holder estimated. For example, the TXOP holder uses TSINGLE-MSDU=1000ms for an AC that TTXOP is 0 with the assumption that the TXOP responder will use 500ms for beamforming report and it will use the remaining 400ms for single MSDU/A-MPDU/MU-PPDU transmission. If the TXOP responder uses 700ms for the beamforming report and the TXOP holder continues the single MSDU/A-MPDU/MU-PPDU transmission, “D” in b) 2) may be smaller than 0. The Bullet b) 2) should be changed to:

Else if TTXOP = 0 and TEND_NAV >0, then

{

If there is no immediate responding Beamforming Report, then D = TEND_NAV -     TPPDU

Else D = TREMAINING_NAV - TPPDU

}

Add the following text after the psedo algorithm in bullet b): TREMAINING_NAV is the estimated time required for the transmission of the remaining frames of the allowed frame exchange sequence defined in 8.4.2.31 (for a TXOP limit value of 0), including applicable IFS durations.

When the TXOP limit is not 0 and the TXOP holder wants to transmit MSDU, A-MPDU, MM-PPDU, the TXOP limit should be respected. Otherwise the extimated time for the beamforming should be used for Duration calculation. So the Bullet b) 3) should be changed to:

Else if TEND_NAV = 0, then

{

If at least one data MPDU will be transmitted in the TXOP, then min (TPENDING, TTXOP - TPPDU) <= D <= TTXOP - TPPDU

Else min (TBEAMFORMING - TPPDU, TTXOP - TPPDU) <=D <= max (TBEAMFORMING - TPPDU, TTXOP - TPPDU)

}

The Bullet b) 4) should be changed to:

Else

{

If at least one data MPDU can be transmitted in the TXOP, then TEND_NAV - TPPDU <= D <= TTXOP_REMAINING - TPPDU

Else min (TBEAMFORMING_REMAINING - TPPDU,TTXOP_REMAINING - TPPDU) <=D = max (TBEAMFORMING_REMAINING - TPPDU,TTXOP_REMAINING - TPPDU)

}

Add the following text after the psedo algorithm in bullet b): “TBEAMFORMING_REMAINING is the estimated time required for the transmission frames of the remaining beamforming sequence.”

Add another sentence after the first sentence in 2nd paragraph in 9.19.2.4 to “The TXNAV is adjusted to the time required for the transmission of the remaining frames of the allowed frame exchange sequence.”

When the TXOP responder uses the medium time that is more than the TXOP holder estimated, the Duration/ID in Beamforming Report frame may be smaller than 0. To avoid this, at the end of 8.2.5.8, the following text should be added: If the calculated value for Duration/ID of the responding frame is smaller than 0, the Duration/ID field is 0.