IEEE P802.11
Wireless LANs

| 802.11 TGac WG Letter Ballot LB187 Proposed resolutions to comments on Subclause 8.2.5.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-07-26 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Liwen Chu | STMicroelectronics | 2525 Augustine Drive Santa Clara, CA 95054 |  | Liwen.chu@st.com |

Abstract

This submission contains proposed comment resolutions to comments received during WG letter ballot 188.

The comments included are non-editorial comments on Subclause 8.2.5.2.

There are xx such comments: 6084, 6085, 6234, 6670, 6706, 6792, 6793.

| CID | Page | Clause | Comment | Proposed Change | Proposed Resolution |
| --- | --- | --- | --- | --- | --- |
| 6084 | 38.34 | 8.2.5.2 | Currently in Tgac D3.0, multiple protection is used for NAPA/BFRP, and TXOP limit can't be the limit to the beamforming report. The rules make things clearer and provide flexibility. But it is not enough for a STA to set the Duration field. | fix the problem. | Revise. See the comment 6084 discussion and proposed editorial instruction in 11-12/0856 |
| 6085 | 39.03 | 8.2.5.2 | NDPA is missing from the pending frame | Change to "Any NDPA, NDP or Beamforming Report Poll frame transmissions and explicit feedback response frames" | Reject. The first bullet in P38 L65 already includes NDPA. |
| 6234 | 38.52 | 8.2.5.2 | "always use" -> needs a normative reference or to be moved to clause 9 | Add a clause 9 reference or move to clause 9. Also "multiple protection settings" or just "M/P setting"? | Reject. IEEE 802.11TM 2012 uses the same language in Duration/Id setting for PSMP etc. The commenter is welcome to submit the comment to Revmc group. |
| 6670 | 39.18 | 8.2.5.2 | "They are transmitted at a rate no lower than that which would be used if they were control response frames (see 9.7.5.6 (Rate selection for other data and management frames))." The rules in that section do not define the behavior implied by this bullet. Also this behavior should not be enforced. STA shoud decide on the rate of big packets. | remove this bullet | Accept. |
| 6706 | 39.12 | 8.2.5.2 | The estimated duration for a VHT Compressed Beamforming frame response can be optimized when the response MCS is approximately known. Even though the text is clear that the duration is an estimated duration, it may be more clear to change "is determined" into "may be determined". | On P39L12 change "is determined" into "may be determined". | Revise. See CID 6670. |
| 6792 | 39.17 | 8.2.5.2 | Not certain if you should be using a pronoun as the subject within a bulleted list. I assume that the antecedent for the pronoun would be the text preceding the list, and not text within a preceding bullet item. | Change "They" to "All segments" | Revise. See comment 6670 that the bullet is removed. |
| 6793 | 39.19 | 8.2.5.2 | Not clear where these fields are found. | Change "values" to "value in all segments" | Revise. Editorial instruction: Change P39 L19 to “”The subfield values of VHT MIMO Control field in each segment are as follows:” |

Discussion:

Comment 6084:

When the TXOP responder uses the medium time that is more than the TXOP holder estimated, the Duration/ID in Beamforming Report frame may be smaller than 0. It is no harm to gerneralize this to all responding frames. To avoid this, at the end of the secondary paragraph in Subclause 8.2.5.1 of IEEE 802.11TM—2012, the following text should be added: “If the calculated Duration of the responding frame is negative, the Duration/ID field is set to 0”.

When the TXOP Limit is 0 and the Duration value is negative in the TXOP holder in a TXOP with beamforming, the Duration/ID field needs to  be set 0.

8.2.5 Duration/ID field

8.2.5.1 General

Editorial Instruction: Apply the change in 11-12/321r7.

8.2.5.2 Setting for single and multiple protection under enhanced distributed channel

access (EDCA)

Editorial Instruction: Change bullet  b)2) in Subclause 8.2.5.2 of IEEE 802.11TM—2012 as following:

2) Else if TTXOP = 0 and TEND_NAV > 0, then D = max (0, TEND-NAV –TPPDU)