IEEE P802.11
Wireless LANs

| 802.11 TGac WG Letter Ballot LB187 Proposed resolutions to comments on Subclause 9.19.2.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Liwen Chu | STMicroelectronics | 2525 Augustine Drive Santa Clara, CA 95054 |  | Liwen.chu@st.com |

Abstract

This submission contains proposed comment resolutions to comments received during WG letter ballot 188.

The comments included are non-editorial comment on Subclause 9.19.2.2 and one non-editorial comment on Subclause 9.19.2.2 on Subclause 8.2.5.

There are xx such comments: 6803, 6436.

| CID | Page | Clause | Comment | Proposed Change | Proposed Resolution |
| --- | --- | --- | --- | --- | --- |
| 6803 | 125.60 | 9.19.2.2 | Does the response include the SIFS or only the response frame itself? | Please clarify. | Revise |

Discussion:

Comment 6803: The responding frame includes SIFS before it.

Editorial Instruction: Change P125 L51 to L63 In subclause 9.192.2 per 11-12/xxxxr0.

When the TXOP limit is nonzero, a STA shall fragment an individually addressed MSDU so that the transmission

of the first MPDU of the TXOP does not cause the TXOP limit to be exceeded at the PHY rate selected for the initial transmission attempt of that MPDU. The TXOP limit may be exceeded, when using a lower PHY rate than selected for the initial transmission attempt of the first MPDU, for a retransmission of an MPDU, for the initial transmission of an MPDU if any previous MPDU in the current MSDU has been retransmitted, or for group addressed MSDUs. The TXOP limit may also be exceeded by transmitting a VHT NDP Announcement frame and NDP or Beamforming Report Poll frame in the sense that they fit within the TXOP limit but the response and its previous SIFS causes the TXOP limit to be exceeded. When the TXOP limit is exceeded due to the retransmission of an MPDU at a reduced PHY rate, the STA shall not transmit more than one MPDU in the TXOP.

| CID | Page | Clause | Comment | Proposed Change | Proposed Resolution |
| --- | --- | --- | --- | --- | --- |
| 6436 | 38.32 | 8.2.5 | The change in 8.2.5.1 proposed in 321r7 and agreed to by the TG in motion 54, has not been made | At the end of paragraph 2 of 8.2.5.1 add the sentence: If a calculated duration results in a negative value, the value 0 is inserted in the Duration/ID field. | Revise. See Comment 6804. |