IEEE P802.11
Wireless LANs

| 802.11 Update for ITU-R M.1801 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-7-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Peter Ecclesine | Cisco Systems | 170 W. Tasman Dr. MS SJ-10-5 San Jose, CA 95134-1706 | +1-408-527-0815 | petere@cisco.com |

Abstract

This document contains the 802.11 Working Group Update for ITU-R M.1801, modified editorially to show the predecessor amendment names in Table 6 for amendments which were integrated into IEEE Std. 802.11-2012. The URL for ITU-R M.1801 is http://www.itu.int/rec/R-REC-M.1801-1-201004-I/en

| Radiocommunication Study Groups |  |
| --- | --- |
| Received: 17 July 2012 Question: Question ITU-R 238/8 | Document 5A/xxx |
|  | 15 JULY 2008 |
|  | English only |
| Institute of Electrical and Electronics Engineers (IEEE) |  |
| Revision of Recommendation ITU-R M.1801 |  |

# 1	Source information

This contribution was developed by IEEE Project 802®, the Local and Metropolitan Area Network Standards Committee (“IEEE 802”), an international standards development committee organized under the IEEE and the IEEE Standards Association (“IEEE-SA”).

The content herein was prepared by a group of technical experts in IEEE 802 and industry and was approved for submission by the IEEE 802.11™ Working Group on Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY), the IEEE 802.18 Radio Regulatory Technical Advisory Group, and the IEEE 802 Executive Committee, in accordance with the IEEE 802 policies and procedures, and represents the view of IEEE 802.

# 2	Background

IEEE is in receipt of the “LIAISON STATEMENT TO EXTERNAL ORGANIZATIONS –REQUEST FOR INPUT FOR A REVISION OF RECOMMENDATION ITU-R M.1801” and appreciates the opportunity to submit updated material for a revision of this Recommendation.

This contribution contains the proposed replacement of the material describing IEEE 802.11 RLANs, with additional modifications to Annex 6.

Revisions to:

https://mentor.ieee.org/802.18/file/08/18-08-0043-01-IMTA-802-20-m1801-revision.doc  ?

Annex 1

Broadband radio local area networks

Radio local area networks (RLAN) offer an extension to wired LANs utilizing radio as the connective media. They have applications in commercial environments where there may be considerable savings in both cost and time to install a network; in domestic environments where they provide cheap, flexible, connectivity to multiple computers used in the home; and in campus and public environments where the increasing use of portable computers, for both business and personal use, while travelling and due to the increase in flexible working practices, e.g., nomadic workers using laptop personal computers not just in the office and at home, but in hotels, conference centres, airports, trains, planes and automobiles. In summary, they are intended mainly for nomadic wireless access applications, with respect to the access point (i.e., when the user is in a moving vehicle, the access point is also in the vehicle).

Broadband radio local area network standards are included in Recommendation ITU-R M.1450, and can be grouped as follows:

–	IEEE 802.11

–	ETSI BRAN HIPERLAN

–	ARIB HiSWANa

# 1	IEEE 802.11

The IEEE 802.11™ Working Group has developed a standard for RLANs, IEEE Std 802.11-2012, which is part of the IEEE 802 series of standards for local and metropolitan area networks. The medium access control (MAC) unit in IEEE Std 802.11 is designed to support physical layer units as they may be adopted dependent on the availability of spectrum. IEEE Std 802.11 operates in the 2 400-2 500 MHz band and in the bands comprising 3 650-3 700 MHz, 4.94-4.99 GHz, 5.03-5.091 GHz, 5.15-5.25 GHz, 5.25-5.35 GHz, 5.47-5.725 GHz and 5.725-5.850 GHz. IEEE Std 802.11 employs the frequency hopping spread spectrum (FHSS) technique, direct sequence spread spectrum (DSSS) technique and orthogonal frequency division multiplexing (OFDM) technique.

Approved amendments to the IEEE 802.11-2012 base standard include Prioritization of Management Frames (IEEE 802.11ae), Video Transport Streams (IEEE 802.11aa). PHY and MAC amendments with available drafts include Very High Throughput 60 GHz (IEEE 802.11ad) and Very High Throughput below 6 GHz (IEEE 802.11ac).

The URL for the IEEE 802.11 Working Group is http://www.ieee802.org/11. The IEEE Std 802.11-2012 standard and some amendments are available at no cost through the Get IEEE 802™ program at http://standards.ieee.org/about/get/, and future amendments will become available for no cost six months after publication. Approved amendments and some draft amendments are available for purchase at http://www.techstreet.com/ieeegate.html.

# 2	ETSI BRAN HIPERLAN

The HiperLAN 2 specifications were developed by ETSI TC (Technical Committee) BRAN (broadband radio access networks). HiperLAN 2 is a flexible RLAN standard, designed to provide high-speed access up to 54 Mbit/s at physical layer (PHY) to a variety of networks including internet protocol (IP) based networks typically used for RLAN systems. Convergence layers are specified which provide interworking with Ethernet, IEEE 1394 and ATM. Basic applications include data, voice and video, with specific quality-of-service parameters taken into account. HiperLAN 2 systems can be deployed in offices, classrooms, homes, factories, hot spot areas such as exhibition halls and, more generally, where radio transmission is an efficient alternative or complements wired technology.

HiperLAN 2 is designed to operate in the bands 5.15-5.25 GHz, 5.25-5.35 GHz and 5.47-5.725 GHz. The core specifications are TS 101 475 (physical layer), TS 101 761 (data link control layer), and TS 101 493 (convergence layers). All ETSI standards are available in electronic form at: http://pda.etsi.org/pda/queryform.asp, by specifying the standard number in the search box.

ETSI TC BRAN has also developed conformance test specifications for the core HIPERLAN 2 standards, to assure the interoperability of devices and products produced by different vendors. The test specifications include both radio and protocol testing.

ETSI TC BRAN has worked closely with IEEE-SA (Working Group 802.11) and with MMAC in Japan (Working Group High Speed Wireless Access Networks) to harmonize the systems developed by these three fora for the 5 GHz bands.

# 3	MMAC HSWA

MMAC HSWA has developed and ARIB has approved and published, a standard for broadband mobile access communication systems. It is called HiSWANa (ARIB STD-T70). The scope of the technical specifications is limited to the air interface, the service interfaces of the wireless subsystem, the convergence layer functions and supporting capabilities required to realize the services.

The technical specifications describe the PHY and MAC/DLC layers, which are core network independent, and the core network-specific convergence layer. The typical data rate is from 6 to 36 Mbit/s. The OFDM technique and TDMA-TDD scheme are used. It is capable of supporting multimedia applications by providing mechanisms to handle the quality-of-service (QoS). Restricted user mobility is supported within the local service area. Currently, only Ethernet service is supported.

The HiSWANa system is operated in the 5 GHz bands (4.9-5.0 GHz and 5.15-5.25 GHz).

Multimedia Mobile Access Communication Systems Promotion Council (now called “Multimedia Mobile Access Communication Systems Forum” or “MMAC Forum”).

2	High Speed Wireless Access Committee.

3	Association of Radio Industries and Businesses.

Annex 6

Table 6 provides a summary of key characteristics of each standard.

TABLE 6

Key technical parameters

| Standard | Nominal RF channel bandwidth | Modulation/ coding rate(1) – upstream – downstream | Coding support | Peak channel transmission rate per 5 MHz channel (except as noted) | Beam-forming support (yes/no) | Support for MIMO (yes/no) | Duplex method | Multiple access method | Frame duration | Mobility capabilities (nomadic/ mobile) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IEEE 802.16 WirelessMAN/ ETSI HiperMAN (Annex 3) | Flexible from 1.25 MHz up to 28 MHz. Typical bandwidths are: 3.5, 5, 7, 8.75, 10 and 20 MHz | Up: – QPSK-1/2, 3/4 – 16-QAM-1/2, 3/4 – 64-QAM-1/2, 2/3, 3/4, 5/6 Down: – QPSK-1/2, 3/4 – 16-QAM-1/2, 3/4 – 64-QAM-1/2, 2/3, 3/4, 5/6 | CC/CTC Other options: BTC/ LDPC | Up to 17.5 Mbit/s with SISO Up to 35 Mbit/s with (2 × 2) MIMO Up to 70 Mbit/s with (4 × 4) MIMO | Yes | Yes | TDD/ FDD/ HFDD | OFDMA TDMA | 5 msec Other options: 2, 2.5, 4, 8, 10, 12.5 and 20 msec | Mobile |
| T1.723-2002 I-CDMA spread spectrum systems air interface standard (Annex 4) | 1.25 MHz | Up: – QPSK, – 0.325-0.793 Down: – QPSK, – 0.325-0.793 | Block TPC BCH | Up: 1.228 Mbit/s Down: 1.8432 Mbit/s | Not explicit but not preclu-ded | Not explicit but not preclu-ded | FDD | CDMA | Tier 1: 13.33 ms Tier 2: 26.67 ms | Nomadic |
| ATIS-0700001.2004 MCSB physical, MAC/LLC, and network layer specification (Annex 4) | 5 MHz | Up: – QPSK, 8-PSK – 16-QAM R-S (18, 16) Down: – QPSK, 8-PSK – 64-QAM R-S (18, 16) | Reed-Solomon (18, 16) | Up: 6.4 Mbit/s Down: 24 Mbit/s | Yes | Not speci-fied | TDD | CDMA | 10 ms | Nomadic |

TABLE 6 (continued)

| Standard | Nominal RF channel bandwidth | Modulation/ coding rate(1) – upstream – downstream | Coding support | Peak channel transmission rate per 5 MHz channel (except as noted) | Beam-forming support (yes/no) | Support for MIMO (yes/no) | Duplex method | Multiple access method | Frame duration | Mobility capabilities (nomadic/ mobile) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ATIS-0700004.2005 high capacity-spatial division multiple access (HC-SDMA) (Annex 4) | 0.625 MHz | Up: – BPSK, QPSK, 8-PSK, 12-QAM, 16-QAM 3/4 Down: – BPSK, QPSK, 8-PSK, 12-QAM, 16-QAM, 24-QAM 8/9 | Convolu-tional and block code | Up: 2.866 Mbit/s × 8 sub-channels × 4 spatial channels = 91.7 Mbit/s Down: 2.5 Mbit/s × 8 sub-channels × 4 spatial channels = 80 Mbit/s | Yes | Yes | TDD | TDMA/FDMA/ SDMA | 5 ms | Mobile |
| T1.716/7-2000 (R2004) air interface standard for broadband direct sequence CDMA for fixed wireless PSTN access – Layer 1/ Layer 2 (Annex 4) | 2 × 5 to 2 × 20 MHz (in 3.5 or 5 MHz increments) | Up: – QPSK, – 1/2 Down: – QPSK, – 1/2 | Convolu-tional | Up: 1.92 Mbit/s Down: 1.92 Mbit/s | No | No | FDD | CDMA | 19 ms max | Nomadic |
| Next-generation PHS (Annex 5) | 1.25 MHz 2.5 MHz 5 MHz 10 MHz 20 MHz | Up and down: BPSK 1/2 QPSK 1/2, 3/4 16-QAM 3/4 32-QAM 4/5 64-QAM 5/6 256-QAM 7/8 | Convolu-tional code Trellis code | Up: 8.0 Mbit/s Down: 11.2 Mbit/s (in case of SISO, symmetry) | Yes (option) | Yes (option) | TDD | OFDMA TDMA | 5 ms | Mobile |
| IEEE 802.11-2012 (formerly 802.11b) (Annex 1) | 22 MHz | Upand down: DQPSK CCK BPSK PBCC – 1/2 QPSK PBCC – 1/2 | Uncoded/ CC | 2.5 Mbit/s | No | No | TDD | CSMA/ CA, SSMA | Variable frame duration | Nomadic |

TABLE 6 (continued)

| Standard | Nominal RF channel bandwidth | Modulation/ coding rate(1) – upstream – downstream | Coding support | Peak channel transmission rate per 5 MHz channel (except as noted) | Beam-forming support (yes/no) | Support for MIMO (yes/no) | Duplex method | Multiple access method | Frame duration | Mobility capabilities (nomadic/ mobile) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IEEE 802.11-2012 (formerly 802.11a) (Annex 1) | 5 MHz 10 MHz 20 MHz | Up and down: 64-QAM OFDM – 2/3, 3/4 16-QAM OFDM –1/2, 3/4 QPSK OFDM – 1/2, 3/4 BPSK OFDM – 1/2, 3/4 | CC | 13.5 Mbit/s | No | No | TDD | CSMA/ CA | Variable frame duration | Nomadic |
| IEEE 802.11-2012 (formerly 802.11g) (Annex 1) | 20 MHz | Up and down: 64-QAM OFDM – 2/3, 3/4 16-QAM OFDM – 1/2, 3/4 QPSK OFDM – 1/2, 3/4 BPSK OFDM – 1/2, 3/4 8-PSK PBCC – 2/3 64-QAM DSSS-OFDM – 2/3, 3/4 16-QAM DSSS-OFDM – 1/2, 3/4 QPSK DSSS-OFDM – 1/2, 3/4 BPSK DSSS-OFDM – 1/2, 3/4 | CC | 13.5 Mbit/s | No | No | TDD | CSMA/ CA | Variable frame duration | Nomadic |
| IEEE 802.11-2012 (formerly 802.11n) (Annex 1) | 20 MHz 40 MHz | Up and down : 64-QAM OFDM – 2/3, 3/4, 5/6 16-QAM OFDM –1/2, 3/4 QPSK OFDM – 1/2, 3/4 BPSK OFDM – 1/2 | CC and LDPC | 75 Mbit/s | Yes | Yes | TDD | CSMA/ CA | Variable frame duration | Nomadic |
| IEEE 802.11ac | 20 MHz 40 MHz 80 MHz 160 MHz 80+80 MHz | Up and down, 256-QAM OFDM 2/3, ¾, 5/6 64-QAM OFDM – 2/3, 3/4, 5/6 16-QAM OFDM –1/2, 3/4 QPSK OFDM – 1/2, 3/4 BPSK OFDM – 1/2 | CC and LDPC | 216 Mbit/s | Yes | Yes | TDD | CSMA/CA | Variable frame duration | Nomadic |
| ETSI BRAN HiperLAN 2 (Annex 1) | 20 MHz | 64-QAM-OFDM 16-QAM-OFDM QPSK-OFDM BPSK-OFDM both upstream and downstream | CC | 6, 9, 12, 18, 27, 36 and 54 Mbit/s in 20 MHz channel (only 20 MHz channels supported) | No | No | TDD | TDMA | 2 ms | Nomadic |

TABLE 6 (continued)

| Standard | Nominal RF channel bandwidth | Modulation/ coding rate(1) – upstream – downstream | Coding support | Peak channel transmission rate per 5 MHz channel (except as noted) | Beam-forming support (yes/no) | Support for MIMO (yes/no) | Duplex method | Multiple access method | Frame duration | Mobility capabilities (nomadic/ mobile) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ARIB HiSWANa (Annex 1) | 4 × 20 MHz (5.15-5.25 GHz) 4 × 20 MHz (4.9-5.0 GHz) | – BPSK 1/2 – BPSK 3/4 – QPSK 1/2 – QPSK 3/4 – 16-QAM 9/16 – 16-QAM 3/4 – 64-QAM 3/4 | Convolu-tional | 6-54 Mbit/s in 20 MHz | No | No | TDD | TDMA | 2 ms | Nomadic |
| IMT-2000 CDMA Direct Spread (Annex 2) | 5 MHz | Up: QPSK Down: 16-QAM, QPSK | Convolu-tional turbo | Up: 5.7 Mbit/s Down: 14 Mbit/s | Yes | No (For inclu-sion in Rel-7) | FDD | CDMA | 2 ms and 10 ms | Mobile |
| IMT-2000 CDMA Multi-Carrier (Annex 2) | 1.25 MHz | Up: BPSK, QPSK, 8-PSK Down: 16-QAM, 8-PSK, QPSK | Convolu-tional/ turbo | Up: 1.8 Mbit/s Down: 3.1 Mbit/s Per 1.25 MHz channel | Yes | No | FDD | CDMA | Down: 1.25, 1.67 2.5, 5, 10, 20, 40, 80 ms Up: 6.66, 10, 20, 26.67, 40, 80 ms | Mobile |
| IMT-2000 CDMA TDD (Annex 2) | HCR: 5 MHz LCR: 1.6 MHz (high chip rate/low chip rate) | HCR Up: 16-QAM, QPSK LCR Up: 8-PSK, QPSK HCR Down: 16-QAM, QPSK LCR Down: 16-QAM, 8-PSK, QPSK | Convolu-tional turbo | HCR Up: 9.2 Mbit/s LCR Up: 2 Mbit/s/1.6 MHz carrier(2) HCR Down: 10.2 Mbit/s LCR Down: 2.8 Mbit/s/ 1.6 MHz carrier(2) | Yes | No | TDD | HCR: TDMA/CDMA LCR: TD-SCDMA | HCR: 10 ms LCR: 5 ms | Mobile |

TABLE 6 (end)

| Standard | Nominal RF channel bandwidth | Modulation/ coding rate(1) – upstream – downstream | Coding support | Peak channel transmission rate per 5 MHz channel (except as noted) | Beam-forming support (yes/no) | Support for MIMO (yes/no) | Duplex method | Multiple access method | Frame duration | Mobility capabilities (nomadic/ mobile) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IMT-2000 TDMA Single-Carrier (Annex 2) | 2 × 200 kHz 2 × 1.6 MHz | Up: – GMSK – 8-PSK – B-OQAM – Q-OQAM 0.329 – 1/1 Down: – GMSK – 8-PSK – B-OQAM – Q-OQAM 0.329 – 1/1 | Punctured convolu-tional | Up: 16.25 Mbit/s 20.312 Mbit/s Down: 16.25 Mbit/s 20.312 Mbit/s | Not explicit but not preclu-ded | Not explicit but not preclu-ded | FDD TDD (for 1.6 MHz) | TDMA | 4.6 ms 4.615 ms | Mobile |
| IMT-2000 FDMA/TDMA (Annex 2) | 1.728 MHz | Up and down: GFSK π/2-DBPSK π/4-DQPSK π/8-D8-PSK 16-QAM, 64-QAM | Depends on service: CRC, BCH, Reed-Solomon, Turbo | 20 Mbit/s | Partial | Partial | TDD | TDMA | 10 ms | Mobile |
| 1) Including all applicable modes, or at least the maximum and the minimum. (2) LCR TDD has a channelization equal to 1.6 MHz. In 5 MHz three LCR TDD carriers can be deployed. A 7.68 Mchip/s TDD option is also available. This requires 10 MHz bandwidth and provides double the data rates of HCR TDD. |  |  |  |  |  |  |  |  |  |  |

_____________

Multimedia Mobile Access Communication Systems Promotion Council (now called “Multimedia Mobile Access Communication Systems Forum” or “MMAC Forum”).

High Speed Wireless Access Committee.

Association of Radio Industries and Businesses.