IEEE P802.11
Wireless LANs

| 802.11 TGac WG Letter Ballot LB188 Proposed resolutions to comments on clause 9.12 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eric Wong | Broadcom Corporation | 190 Mathilda Place, Sunnyvale, CA 94086 | +1 408 922 6672 | ewong@broadcom.com |

Abstract

This submission contains proposed comment resolutions to comments received during WG letter ballot 188.

The comments assigned to the author in Clauses 9.12 are:

6038, 6039, 6040, 6113, 6114, 6284, 6286, 6555, 6556, 6676, 6752, 6753, 6754, 6755, 6759

The changes marked in this document are based on TGac Draft 3.1.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

# Comments for Clause 9.12.2

## CID 6752

| 6752 | David Hunter | 9.12.2 | 117.39 | "in the DMG Capabilities element." loses the requirement that this element has to be from the intended receiver. | Replace "element." with "element received from the intended receiver." |
| --- | --- | --- | --- | --- | --- |

Discussion:

Agree with commenter.

Proposed resolution:

Accept.

Instruction to Editor:

Please make the following changes in TGac D3.1 P123L28 under subclause 9.12.2.

“A DMG STA shall not transmit an A-MPDU that is longer than the value indicated by the Maximum A-MPDU Length Exponent field in the DMG Capabilities element from the intended receiver.”

## CID 6753

| 6753 | David Hunter | 9.12.2 | 117.44 | "NOTE--The restriction limits": what restriction? The immdediately preceding sentence was about MaxTime on the medium. That is what restricts the length to 4095? Doubt that. | Replace "The restriction" to something that refers specifically to which restriction. |
| --- | --- | --- | --- | --- | --- |

Discussion:

These two sentences are added because of CID 4084 in LB188. See 11-12/0593r0. The restriction applies to a PPDU not exceeding aPPDUMaxTime, and aPPDUMaxTime is 5.484ms for VHT PHY. This is equivalent to a maximum value of LENGTH field in L-SIG field to 4095.

“A STA shall not transmit a VHT PPDU if the PPDU duration exceeds aPPDUMaxTime defined in Table 22-29.

NOTE—The restriction limits the LENGTH field in the L-SIG field of a VHT PPDU to 4095.”

Proposed resolution:

Accept.

Instruction to Editor:

Please make the following changes in TGac D3.1 P123L33 under subclause 9.12.2.

NOTE—The restriction of PPDU duration not exceeding aPPDUMaxTime limits the LENGTH field in the L-SIG field of a VHT PPDU to 4095.”

## CID 6754

| 6754 | David Hunter | 9.12.2 | 117.46 | Removal of this NOTE about the scope of the A-MPDU length is removal of a helpful note about HT implementations. | Instead of removing this note, specify it as applying to HT transmissions and supplement that with relevant material about the scope of the A-MPDU length in VHT implementations. |
| --- | --- | --- | --- | --- | --- |

Discussion:

NOTE was removed in D0.4 because of CID 1145. See 11-11/394r2.

“NOTE—The A-MPDU length limit applies to the maximum length of the PSDU that might be received. If the A-MPDU includes any padding delimiters (i.e., delimiters with the Length field equal to 0) in order to meet the MPDU start spacing requirement, this padding is included in this length limit.”

Agree in principle. The deleted NOTE can be informative.

Proposed resolution:

Revise.

Instruction to Editor:

Please keep the NOTE at the end of subclause 9.12.2 with the following changes in TGac D3.1.

“NOTE—For an HT STA or a DMG STA, the A-MPDU length limit applies to the maximum length of the PSDU that may be received. The A-MPDU length limit for an HT STA or a DMG STAincludes any padding delimiters (i.e., delimiters with the Length field equal to 0) added to meet the MPDU start spacing requirement.”

# Comments for Clause 9.12.3

## CID 6755

| 6755 | David Hunter | 9.12.3 | 117.56 | Deletion of "HT STA and a DMG" from this requirement opens this requirement to all IEEE 802.11 implementations, even pre-HT legacy. This change is not within the scope of 11ac. | Instead of deleting "An HT STA and a DMG STA", replace it with "HT, VHT and DMG STAs". |
| --- | --- | --- | --- | --- | --- |

Discussion:

Pre-HT legacy STA does not support A-MPDU, and this requirement would not apply. Therefore, there is no need to make any changes here.

Rather in subclause 9.12.2, corrections are needed. The original first sentence describes the maximum A-MPDU length for both an HT STA and DMG STA. In this sentence, this description is separated into two sentences – one for HT STA, and another for DMG STA. A third sentence is added for a VHT STA.

Proposed resolution:

Revise.

Instruction to Editor:

Please make the following changes in TGac D3.1 P122L59 under subclause 9.12.2.

“An HT STA and a DMG STA indicates a value in the Maximum A-MPDU Length Exponent field in its HT Capabilities element or DMG Capabilities element, respectively, that defines the maximum A-MPDU length that it can receive in an HT_MF or HT_GF PPDU. A VHT STA indicates in the Maximum A-MPDU Length Exponent field in its VHT Capabilities element the maximum length of the A-MPDU pre-EOF padding that it can receive in a VHT PPDU. A DMG STA indicates in the Maximum A-MPDU Length Exponent field in its DMG Capabilities element the maximum A-MPDU length that it can receive.”

Please make the following changes in TGac D3.1 P123L21 under subclause 9.12.2.

“An HT STA shall not transmit an A-MPDU in an HT_MF or HT_GF PPDU that is longer than the value indicated by the Maximum A-MPDU Length Exponent field in the HT Capabilities element received from declared by the intended receiver. A VHT STA shall not transmit an A-MPDU in a VHT PPDU where the A-MPDU pre-EOF padding length is longer than the value indicated by the Maximum A-MPDU Length Exponent field in the VHT Capabilities element received from the intended receiver. A DMG STA shall not transmit an A-MPDU that is longer than the value indicated by the Maximum A-MPDU Length Exponent field in the DMG Capabilities element.”

# Comments for Clause 9.12.6

## CID 6038

| 6038 | Adrian Stephens | 9.12.6 | 119.32 | "A VHT STA that delivers one or more A-MPDUs to the PHY (using PHY-DATA.request primitives) as one or more PSDUs" This introductory sentence may confuse people as to the mapping from A-MPDUs to PSDUs -i.e. it appears to allow more than one A-MPDU per PSDU. | Replace the first sentence with: "A VHT STA that transmits a VHT PPDU, which contains one or more PSDUs, each of which contains an A-MPDU, shall construct the A-MPDU(s) as described in this subclause." |
| --- | --- | --- | --- | --- | --- |

Discussion:

Agree with commenter.

Proposed resolution:

Accept.

Instruction to Editor:

Please make the following changes in TGac D3.1 P125L13 under subclause 9.12.6.

“A VHT STA that transmits a VHT PPDU, which contains one or more PSDUs, each of which contains an A-MPDU, shall construct the A-MPDU(s) as described in this subclause.”

## CID 6039 / 6113 / 6040 / 6114 / 6556

| 6039 | Adrian Stephens | 9.12.6 | 119.41 | "A-MPDU subframes with 0 in the MPDU Length field provided that each added subframe and the complete A-MPDU meet all the following:" The "provided that" condition should apply to both bullets - i.e. the constraints are true even when no zero-length subframes have been added. | Promote the "provide that.." to body text, and turn the bullet list that follows into a dashed list at level 1. |
| --- | --- | --- | --- | --- | --- |
| 6113 | Liwen Chu | 9.12.6 | 119.42 | "provided that each...(see 9.19.2.2 (EDCA TXOPs)) for the primary AC" should be applied to non-zero-length A-MPDU subframe and zero-length A-MPDU subframe. | Change L41L42 to: ----A-MPDU subframes with 0 in the MPDU Length field Each added subframe and the complete A-MPDU meet all the following: |
| 6040 | Adrian Stephens | 9.12.6 | 120.01 | "have 0 in the MPDU Length field provided that each added subframe and the complete A-MPDU meet all the following:" The "provided that" condition should apply to both bullets - i.e. the constraints are true even when no zero-length subframes have been added. | Promote the "provide that.." to body text, and turn the bullet list that follows into a dashed list at level 1. Merge the sentence "and provided that" that follows into this dashed list. |
| 6114 | Liwen Chu | 9.12.6 | 120.01 | "provided that each..." should be applied to non-zero-length A-MPDU subframe and zero-length A-MPDU subframe. | Change L1L2 to: ----A-MPDU subframes with 0 in the MPDU Length field Each added subframe and the complete A-MPDU meet all the following: |
| 6556 | Sigurd Schelstraete | 9.12.6 | 120.03 | Clarify scope of sub-bullets on lines 3-9. Probably, these sub-bullets are conditions for both of the bullets "- have a TID ..." and "- have 0 in the MPDU Length field ..." Currently, the positioning of the sub-bullets would indicate they only appy to the second bullet. | Clarify and modify text accordingly. |

Discussion:

Agree with both commenters.

Proposed resolution:

Revise.

Instruction to Editor:

For CID 6039 and CID 6113,

Please make the following changes in TGac D3.1 P125L23 under subclause 9.12.6.

“— A-MPDU subframes with 0 in the MPDU Length field

provided that each added subframe and the complete A-MPDU meet all the following:

— A-MPDU content constraints (see 9.12.1 (A-MPDU contents)) for the intended recipient

— length limit constraints (see 8.6.1 (A-MPDU format) and 9.12.2 (A-MPDU length limit rules)) for the intended recipient

— minimum MPDU start spacing constraints (see 9.12.3 (Minimum MPDU Start Spacing field)) for the intended recipient

— TXOP duration limits (see 9.19.2.2 (EDCA TXOPs)) for the primary AC”

For CID 6040, CID 6114 and 6556,

Please make the following and similar changes in TGac D3.1 P125L48 under subclause 9.12.6.

“— have 0 in the MPDU Length field

provided that each added sub-frame and the complete A-MPDU meets all of the following:

— A-MPDU content constraints (see 9.12.1 (A-MPDU contents)) for the intended recipient

— length limit constraints (see 8.6.1 (A-MPDU format) and 9.12.2 (A-MPDU length limit rules)) for the intended recipient

— MPDU start spacing constraints (see 9.12.3 (Minimum MPDU Start Spacing field)) for the intended recipient”

## CID 6284 / 6555

| 6284 | Brian Hart | 9.12.6 | 119.56 | "once for each" but only one PPDU containing multipke A-MPDUs | once for the |
| --- | --- | --- | --- | --- | --- |
| 6555 | Sigurd Schelstraete | 9.12.6 | 119.55 | Further clarify relationship between initial A-MPDU length and APEP_LENGTH. | Change "The A-MPDU_Length[n] for user n ..." to "This initial value of A-MPDU_Length[n] for user n". Also add sentence at end of paragraph: "The same initial value of A-MPDU_Length[n] shall also be used as APEP_LENGTH[n] in TXVECTOR." |

Discussion:

For CID 6555, agree with commenter.

For CID 6284, agree in principle. There is only one VHT PPDU. PLME-TXTIME.request is used to determine the time required to transmit a particular PSDU for each user. The change specified to this CID is “VHT PPDU” to “user”.

Proposed resolution:

Revise.

Instruction to Editor:

Please make the following changes in TGac D3.1 P125L36under subclause 9.12.6.

“This initial value of A-MPDU_Length[n] for user n is used as the APEP_LENGTH[n] parameter value for the PLME-TXTIME.request (see 6.5.7 (PLME-TXTIME.request)) primitive, which is then invoked once for each user. The PLME-TXTIME.confirm (see 6.5.8 (PLME-TXTIME.confirm)) primitive provides the TXTIME parameter and PSDU_LENGTH[] parameters for all the users for the transmission. The same initial value of A-MPDU_Length[n] shall also be used as APEP_LENGTH[n] in TXVECTOR.”

## CID 6676

| 6676 | Simone Merlin | 9.12.6 | 119.61 | Subsequently, for each user n; I understad that rules listed below will limit the users interested in this part, but it may be clearer to state: "for each REMINING user n, not in the set of users that are destinations of previously added A-MPDUs" | clarify as suggested |
| --- | --- | --- | --- | --- | --- |

Discussion:

I think the original wording (i.e. for each user n) is appropriate, since the paragraph here is intended to be loop through every user in the set. Use of “remaining” would means some users have already been process prior to this.

Proposed resolution:

Reject.

# Comments for Clause 9.12.7

## CID 6759

| 6759 | David Hunter | 9.12.7 | 120.41 | It is not the field that is nonzero or not, but the value of the field. Also, values of numeric fields are clearer when written as numerals. | Replace the first two sentences with: "The EOF field in an A-MPDU subframe may be set to 1 when the value of the subframe's MPDU Length field is not 0 and that is the only A-MPDU subframe whose MPDU Length field is not 0 in an A-MPDU carried in a VHT PPDU. The EOF field in each A-MPDU subframe shall be set to 1 when that subframe is not the only A-MPDU subframe in the A-MPDU whose MPDU Length field value is 0." |
| --- | --- | --- | --- | --- | --- |

Discussion:

Agree in principle. Nonzero is used widely within the baseline specification, and do not think it is any less unclear.

Proposed resolution:

Revise.

Instruction to Editor:

Please make the following changes in TGac D3.1 P126L23 under subclause 9.12.7.

“The EOF field in the only A-MPDU subframe with a nonzero(#6773) value MPDU Length field in an A-MPDU carried in a VHT PPDU may be set to 1. The EOF field in each A-MPDU subframe with a nonzero(#6773) value MPDU Length field that is not the only A-MPDU subframe with a nonzero(#6773) value MPDU Length field in the A-MPDU carried in a VHT PPDU shall be set to 0.”

# Comments for Clause 9.12.8

## CID 6286

| 6286 | Brian Hart | 9.12.8 | 120.62 | Bullets 3, 4 and 5 are missing references. Notes arguing for a specific behavior without references are very dangerous - oftentimes the normative language just doesn't exist | Add references (3x) |
| --- | --- | --- | --- | --- | --- |

Discussion:

Agree with commenter.

Proposed resolution:

Revise.

Instruction to Editor:

Please make the following changes in TGac D3.1 P126L45 under subclause 9.12.8.

“— A data MPDU cannot indicate an Ack Policy of “Implicit Block Ack”, and does not generate a Block Ack response. (see 8.2.4.5.4 (Ack Policy subfield))

— A data MPDU could indicate an Ack Policy of “Normal Ack”, which solicits(#6503) an ACK immediate response. No Block Ack agreement is necessary in this case. (see 8.2.4.5.4 (Ack Policy subfield))

— The MPDU could be a management frame that solicits an ACK response. (see 8.6.3 (A-MPDU contents))”