IEEE P802.11
Wireless LANs

| 802.11 TGac WG Letter Ballot LB188 Proposed resolutions to comments on clauses 10.39.1 through 10.39.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eric Wong | Broadcom Corporation | 190 Mathilda Place, Sunnyvale, CA 94086 | +1 408 922 6672 | ewong@broadcom.com |

Abstract

This submission contains proposed comment resolutions to comments received during WG letter ballot 188.

The MAC comments assigned to the author in Clauses 10.39.1 through 10.39.4 are:

6011, 6013, 6065, 6124, 6156, 6157, 6303, 6304, 6306, 6307, 6398, 6507, 6561, 6562, 6703, 6704, 6781, 6810

The changes marked in this document are based on TGac Draft 3.1.

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Minor edits. All CIDs, except 6124 and 6156, are discussed and resolutions are approved by straw polls. |
| R2 | Update reject reason for CID 6562. Change resolution for duplicate 6065/6507/6306/6810 from reject to revise. |
| R3 | Add resolutions for CID 6124 and 6156, and this completes all CIDs in this document |

# Comments for Clause 10.39.1

## CID 6561 / 6703

| 6561 | Sigurd Schelstraete | 10.39.1 | 158.12 | The description of how to determine the supported MCS set for the HT Capabilities element does not mention UEQM MCSs for HT. | Add sentence to paragraph: "In addition, The Rx MCS Bitmask of the Supported MCS Set field of the HT Capabilities element may indicate support for MCS 32-76." |
| --- | --- | --- | --- | --- | --- |
| 6703 | Simone Merlin | 10.39.1 | 158.12 | "A VHT STA sets the Rx MCS Bitmask of the Supported MCS Set field of its HT Capabilities element according to the setting of the Rx MCS Map subfield of the VHT Supported MCS Set field of its VHT Capabilities element as follows: for each subfield Max MCS For n SS, , of the Rx MCS Map field with a value other than 3 (no support for that number of spatial streams), the STA shall indicate support for MCSs 8(n-1) to 8(n-1)+7 in the Rx MCS Bitmask, where n is the number of spatial streams." Based on this shall rule, the rules in 9.7.11.3 would not be allowed. | remove the contraddiction |

Discussion:

CID 6561

There is no restriction for a VHT STA on setting bits 32-76 of the Rx MCS Bitmask of the Supported MCS Set field of the HT Capabilitilies elements

CID 6703

In D2.0 and 11-12/295r3 for CID 4164, an additional subclause is added to the TGac draft so that a VHT STA may exclude lower rate MCSs from rate selection. See 9.7.11.3 (Additional rate selection constraints for VHT PPDUs) in 11-12/1004r0. However, the quoted paragraph in this CID indicates otherwise, since it mandates support for all lower MCSs. The commenter has rightly pointed out the contradiction.

Hence, the proposed resolution is to append the quoted paragraph with an exception from the additional rate selection constraints.

Proposed resolution:

Revise. See changes in 11-12/988r1 under heading “CID 6561 / 6703”.

Instruction to Editor:

Please make the following changes in TGac D3.1 P164L38 under subclause 10.39.1. Note proposed changes are dependent on speculative resolutions in 11-12/1004r0.

“As a minimum, a VHT STA sets the Rx MCS Bitmask of the Supported MCS Set field of its HT Capabilities element according to the setting of the Rx MCS Map subfield of the VHT Supported MCS Set field of its VHT Capabilities element as follows: for each subfield Max MCS For n SS, 1 ≤ n  ≤ 4, of the Rx MCS Map field with a value other than 3 (no support for that number of spatial streams), the STA shall indicate support for MCSs 8(n-1) to 8(n-1)+7 in the Rx MCS Bitmask, where n is the number of spatial streams, except for those MCSs marked as unsupported as described in 9.7.11.3 (Additional rate selection constraints for VHT PPDUs).”

## CID 6011 / 6303

| 6011 | Kazuyuki Sakoda | 10.39.1 | 158.49 | Description for mesh STA is missing. | Replace "A VHT AP shall set the HT Operation" with "A VHT AP and a VHT mesh STA shall set the HT Operation" |
| --- | --- | --- | --- | --- | --- |
| 6303 | Brian Hart | 10.39.1 | 158.50 | "as appropriate" is not very helpful | Provide a reference, or define in full |

Discussion:

Agree with commenters on both points.

For CID 6303, Table 8-130 from 802.11-2012 that describes the Secondary Channel Offset subfield is included here as a reference.

Proposed resolution:

Revise. See changes in 11-12/988r1 under heading “CID 6011 / 6303”.

Instruction to Editor:

Please make the following changes in TGac D3.1 P164L57 under subclause 10.39.1.

“A VHT AP or a VHT mesh STA shall set the HT Operation element HT Operation Information field Secondary Channel Offset subfield to indicate the secondary 20 MHz channel as defined in Table 8-130 (HT Operation element fields and subfields) if the BSS operating channel width is more than 20 MHz.”

## CID 6304

| 6304 | Brian Hart | 10.39.1 | 158.56 | And the off-channel rules state "not just any 20 MHz" - they also say something like "not within this BSS's operating bandwidth" | Append with "as constrained by 10.22.6.4.2" |
| --- | --- | --- | --- | --- | --- |

Discussion:

Agree with commenter. A sentence from 10.22.6.4.2 (Basic wideband functionality) stipulates an additional restriction on 20 MHz operation.

“A TDLS peer STA shall not transmit a 20 MHz PPDU in the non-primary channel of its 80 MHz, 160 MHz or 80+80 MHz direct link.”

Proposed resolution:

Accept.

Instruction to Editor:

Please make the following changes in TGac D3.1 P164L62 under subclause 10.39.1.

“A VHT STA that is a member of a VHT BSS shall not transmit a 20 MHz VHT PPDU on a channel other than the primary 20 MHz channel of the BSS, except for a 20 MHz VHT PPDU transmission on an offchannel TDLS direct link (see also 10.22.6.4.2 (Basic wideband functionality)).”

## CID 6124 (TBD)

| 6124 | Liwen Chu | 10.39.1 | 159.15 | Without TDLS off-channel operation, TDLS VHT peer STAs can set up a wider TDLS direct link than the BSS operating channel. | Change to "Except in the case of a TDLS direct-link (which is independently constrained by 10.22.6.3), a STA shall not transmit a PPDU with a TXVECTOR parameter CH_BANDWIDTH indicating a channel bandwidth that is wider than the BSS operating channel width" |
| --- | --- | --- | --- | --- | --- |

Discussion:

Agree with commenter that the operation of TDLS peer STAs at a TDLS direct-link channel width (including off-channel) wider than that of the BSS is permitted, provided that both TDLS peer STAs have indicated it is capable of doing so.

This is similar to CID 6631, and the proposed resolution is to delete this cited paragraph in 10.39.1.

“Except in the case of a TDLS off-channel direct-link (which is independently constrained by 10.22.6.3), a STA shall not transmit a PPDU with a TXVECTOR parameter CH_BANDWIDTH indicating a channel bandwidth that is wider than the BSS operating channel width.”

Proposed resolution:

Revise. See changes in 11-12/1067r2 for CID 6631.

# Comments for Clause 10.39.2

## CID 6398

| 6398 | Bo Sun | 10.39.2 | 161.55 | The statement here only refer to an AP, but it also applies for a mesh STA | change to "the AP or the mesh STA" |
| --- | --- | --- | --- | --- | --- |

Discussion:

Agree with commenter.

Proposed resolution:

Revise. See changes in 11-12/988r1 under heading “CID 6398”.

Instruction to Editor:

Please make the following changes in TGac D3.1 P165L55 under subclause 10.39.2.

“If an AP or a mesh STA starts a VHT BSS that occupies some or all channels of any existing BSSs, the AP or mesh STA may select a primary channel of the new VHT BSS that is identical to the primary channel of any one of the existing BSSs.”

## CID 6562

| 6562 | Sigurd Schelstraete | 10.39.2 | 161.60 | According to the text an AP, can not start a 40, 80 or 160 MHz BSS with a primary 20 MHz channel that is identical to the secondary channel of an existing BSS. However, it is allowed to start a 20 MHz BSS with a primary channel that is identical to the secondary channel of an existing BSS. Is there a possible loophole here, where an AP starts a 20 MHz BSS and then changes the operating bandwidth later, resulting in a 40, 80 or 160 MHz BSS that violates the first condition? | To be clarified |
| --- | --- | --- | --- | --- | --- |

Discussion:

The next paragraph after the quoted sentence prohibits an AP to start a 20 MHz VHT BSS on a secondary 20 MHz channel of existing BSSs, and thereby it avoids the possible loophole pointed out.

Excerpts from TGac Draft 3.1 P165L59,

“If an AP or a mesh STA chooses to select a primary channel of a new VHT BSS with a 40 MHz, 80 MHz, 160 MHz or 80+80 MHz operating channel width from among the channels on which no beacons are detected during the OBSS scans, the selected primary channel

— shall not be identical to the secondary 20 MHz channel of any existing BSSs with a 40 MHz, 80 MHz, 160 MHz or 80+80 MHz operating channel width, and

— should not be overlapped with the secondary 40 MHz channel of any existing BSSs with a 160 MHz or   80+80 MHz operating channel width.

A STA that is an AP or mesh STA(#6063) should not start a VHT BSS with a 20 MHz operating channel width on a channel that is the secondary 20 MHz channel of any existing BSSs with a 40 MHz, 80 MHz, 160 MHz or 80+80 MHz operating channel width, or is overlapped with the secondary 40 MHz channel of any existing BSSs with a 160 MHz or 80+80 MHz operating channel width.”

Proposed resolution:

Reject. The next paragraph after the quoted sentence prohibits an AP to start a 20 MHz VHT BSS on a secondary 20 MHz channel of existing BSSs, and that prohibits the possible loophole.

## CID 6157

| 6157 | Liwen Chu | 10.39.2 | 161.63 | This is too strong and not fair. | Change to "shall not be identical to the secondary 20 MHz channel of any existing BSSs with a 40 MHz, 80 MHz, 160 MHz or 80+80 MHz operating channel width if not every 20MHz channels of its operating channel is the secondary 20 MHz channel of any existing BSSs, and" |
| --- | --- | --- | --- | --- | --- |

Discussion:

Disagree with commenter. The original wording is clear, since it explicitly states a secondary 20 MHz channel and not any 20 MHz channel. See definition of secondary 20 MHz channel below.

Excerpts from TGac Draft 3.1 P6L42,

“secondary 20 MHz channel: In a 40 MHz very high throughput (VHT) basic service set (BSS), the 20 MHz channel adjacent to the primary 20 MHz channel that together form the 40 MHz channel of the 40 MHz VHT BSS. In an 80 MHz very high throughput (VHT) basic service set (BSS), the 20 MHz channel adjacent to the primary 20 MHz channel that together form the primary 40 MHz channel of the 80 MHz VHT BSS. In a 160 MHz or 80+80 MHz VHT BSS, the 20 MHz channel adjacent to the primary 20 MHz channel that together form the primary 40 MHz channel of the 160 MHz or 80+80 MHz VHT BSS.”

Excerpts from TGac Draft 3.1 P165L49,

“If an AP or a mesh STA chooses to select a primary channel of a new VHT BSS with a 40 MHz, 80 MHz, 160 MHz or 80+80 MHz operating channel width from among the channels on which no beacons are detected during the OBSS scans, the selected primary channel

— shall not be identical to the secondary 20 MHz channel of any existing BSSs with a 40 MHz, 80 MHz, 160     MHz or 80+80 MHz operating channel width, and”

Proposed resolution:

Reject. The original text already achieves the objectives of the proposed changes in the comments. From earlier in the same paragraph, detecting a beacon on a channel has higher priority than that channel being a secondary 20MHz channel.

# Comments for Clause 10.39.3

## CID 6156 (TBD)

| 6156 | Liwen Chu | 10.39.3 | 162.15 | There is no channel set definition in 10.1.4.1. For example, 2G4 STA needs to define a special scanning set because of its special channelization. VHT has its special BSS operating channel decision method. A special scanning channel set decision method is required. | Clarify what is the set of the scanned channel. |
| --- | --- | --- | --- | --- | --- |

Discussion:

The proposed resolution is after discussion and agreement with the commenter.

Reference to 10.1.4.1 (General) is added by CID 4990 in LB187 for more information on scanning operation in radar bands. See 11-12/555r2.

Excerpt from D3.0 Redline,

“An OBSS scan operation is a passive or active scan of a set of channels that are potentially affected by VHT

BSS operationoperation (see 10.1.4.1)(#4990). Each channel in the set may be scanned more than once during a single OBSS scan operation. OBSS scans are performed by STAs that start a VHT BSS.”

Comment for CID 4990

Clause 10.38.3 Scanning requirements for VHT STA starts "An OBSS scan operation is a passive or active scan of a set of channels that are potentially affected by VHT BSS operation." In radar bands, it is extremely likely that active scan is prohibited. In radar bands, it is becoming very likely that initial Channel Availability Check will include adjacent channels. It is entirely appropriate for 10.38.3 VHT BSS operation to begin with meeting regulatory requirements, then discuss OBSS scans.

Proposed resolution:

Reject. The channel to be scan is always all channels in the band. The reference 10.1.4.1 was added to provide a list of channels that not to be actively scanned due to radar bands.

# Comments for Clause 10.39.4

## CID 6013

| 6013 | Kazuyuki Sakoda | 10.39.1 | 159.41 | Description for mesh STA is missing. | Replace "A VHT AP can also announce" with "A VHT AP and a mesh STA can also announce". |
| --- | --- | --- | --- | --- | --- |

Discussion:

This has been addressed by CID 6007 in TGac D3.1. The amended text is missing an “a”.

Proposed resolution:

Revise. See changes in 11-12/988r1 under heading “CID 6013”.

Instruction to Editor:

Please make the following changes in TGac D3.1 P166L57 under subclause 10.39.4.

“A VHT AP or a VHT mesh STA(#6007) can also announce a new Country string (including a new Operating Table index), new operating classes or new TPC parameters for the BSS that come into effect at the same time as the switch of operating channel, operating bandwidth, or both.”

## CID 6704

| 6704 | Simone Merlin | 10.39.1 | 160.05 | "When announcing a switch to a 40 MHz operating bandwidth using the Channel Switch Announcement element in a Beacon or Probe Response frame, then the Wide Bandwidth Channel Switch subelement shall also be present in the Channel Switch Wrapper element in the same frame." IF the wrapper is present | add condition |
| --- | --- | --- | --- | --- | --- |

Discussion:

The Channel Switch Wrapper element is optional, and quoted sentence in the CID does not imply otherwise. Agree with commenter to provide clearer text.

Proposed resolution:

Revise. See changes in 11-12/988r1 under heading “CID 6704”.

Instruction to Editor:

Please make the following changes in TGac D3.1 P167L22 under subclause 10.39.4.

“When announcing a switch to a 40 MHz operating bandwidth using the Channel Switch Announcement element in a Beacon or Probe Response frame, then the Wide Bandwidth Channel Switch subelement in the Channel Switch Wrapper element shall also be present in the same frame.

When announcing a switch to a 40 MHz operating bandwidth using the Extended Channel Switch Announcement element in a Beacon or Probe Response frame, then the Wide Bandwidth Channel Switch subelement in the Channel Switch Wrapper element may be present in the same frame.”

Please make the following and similar changes in TGac D3.1 P167L41 under subclause 10.39.4.

“When announcing a switch to a 80 MHz, 80+80 MHz or 160 MHz operating bandwidth using the Channel Switch Announcement element or Extended Channel Switch Announcement element, then a Wide Bandwidth Channel Switch subelement in the Channel Switch Wrapper element shall be present in the same frame as the Channel Switch Announcement element or Extended Channel Switch Announcement element respectively.”

## CID 6781

| 6781 | David Hunter | 10.39.1 | 160.15 | It is not clear whether "is not required" is a "may" or not. | Replace "not required" with "optional". |
| --- | --- | --- | --- | --- | --- |

Discussion:

Agree with commenter.

Proposed resolution:

Revise. See changes in 11-12/988r1 under heading “CID 6781”.

Instruction to Editor:

Please make the following changes in TGac D3.1 P167L30 under subclause 10.39.4.

“NOTE—The indicated operating class within the Extended Channel Switch Announcement element identifies the bandwidth and the relative position of the primary 20 MHz and secondary 20 MHz channels. Hence(#6780) a Wide Bandwidth Channel Switch subelement is optional when the Extended Channel Switch Announcement element is used .for a channel switch to a 40 MHz bandwidth.”

## CID 6065 / 6507 / 6306 / 6810

| 6065 | Adrian Stephens | 10.39.1 | 161.25 | "I can't discern the intended meaning of the following paragraph to correct grammar; quoted verbatim from resolution to #4252." Neither can I. | Goodness only knows how to fix this, I don't. |
| --- | --- | --- | --- | --- | --- |
| 6507 | Mark RISON | 10.39.1 | 161.25 | Discernment is clearly called for | Apply some discernment |
| 6306 | Brian Hart | 10.39.1 | 161.29 | Delete this para - it adds nothing | As in comment |
| 6810 | Matthew Fischer | 10.39.1 | 161.29 | Something is terribly, terribly wrong with this paragraph. | I cant discern the intended meaning of the following paragraph to correct it. |

Discussion:

This paragraph has been addressed by CID 6633 in TGac D3.1, and text of the paragraph copied here for reference.

“A STA that advertises a channel switch using one or more Channel Switch Announcement frames, Channel Switch Announcement elements, Extended Channel Switch Announcement frames or Extended Channel Switch Announcement elements includes a New Country subelement, Wide Bandwidth Channel Switch subelement or a New VHT Transmit Power Envelope subelement in a Channel Wrapper.(#6633)”

Proposed resolution:

Revise. See resolution for CID 6633 in TGac D3.1.

## CID 6307

| 6307 | Brian Hart | 10.39.1 | 161.39 | This means the DFS Owner in an IBSS is powerless to change channels and/or change channels & bandwidths at the same time !!?? Or by saying "member" are we deliberately excluding the BSS starter? | Would be helpful if the impact of this on DFS Ownership were made more clear |
| --- | --- | --- | --- | --- | --- |

Discussion:

Agree with commenter that DO is restricted from switching channels. The proposed solution is to add an exception to this rule for the DO.

Proposed resolution:

Revise. See changes in 11-12/988r1 under heading “CID 6307”.

Instruction to Editor:

Please make the following changes in TGac D3.1 P168L54 under subclause 10.39.4.

“A VHT STA that is a member of an IBSS adopts the values indicated by the Secondary Channel Offset Element and Wide Bandwidth Channel Switch element in received frames according to the rules in 10.1.5 (Adjusting STA timers) and shall not transmit a value for the Secondary Channel Offset Element or Wide Bandwidth Channel Switch Element that differs from the most recently adopted value, unless it is a DO STA.”

Please also make a similar change in IEEE Std 802.11-2012, last paragraph of 10.15.2 (Basic 20/40 MHz BSS functionality)in page 1091.

“An HT STA that is a member of an IBSS adopts the value of the Secondary Channel Offset field in received frames according to the rules in 10.1.5 and shall not transmit a value for the Secondary Channel Offset field that differs from the most recently adopted value, unless it is a DO STA.”