IEEE P802.11
Wireless LANs

| LB188-CID6558-MU-Exchange |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-08-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | +1 408 543 3370 | mfischer@broadcom.com |

Abstract

Proposed resolution for LB188 CID 6558 asking for text to describe the MU PPDU exchange. The proposed resolution also adds text to Annex G for the MU PPDU sequence.

Revision Notes

R2:

Modified language in 9.3.2.9a, turning a part of the text into a note.

Added restriction on BA agreement type for MU STAs.

Made exchange sequence definitive (instead of “typical”)

Changed “individual MPDUs” to MPDUs

R1:

Change figure so that one STA is an AP

CID 6558

| CID | Commenter | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 6558 | Sigurd Schelstraete | 127.31 | 9.19.2.2a | The frame exchange sequence for a MU-MIMO PPDU is illustrated in Figure 9-19b, including BAs, BARs, ... This sequence is never spelt out explicitly in the text in a single place. It would be helpful to add text to clearly describe this new frame sequence. (BTW, should this also be added in Annex G?) | Add text describing the MU-PPDU + acknowledgement frame sequence. | Revise – Tgac editor to make changes to TGac draft 3.1 as shown in document 11-12-1021r2 under the heading CID6558. |

Discussion

Note that the draft says nothing about whether a potential MU recipient should negotiate an immediate BA agreement or delayed BA agreement. I.e. in the case of a missing, expected BAR (e.g. a broken sequence without retry), should a recipient simply continue to wait for the BAR, or should it initiate its own TXOP for BA transfer? It seems implicit in the nature of the exchange shown in the proposed figure that an immediate BA agreement is necessary (needed to allow BA response to BAR), but because there currently is no text in D3.0 or D3.1 describing the MU exchange, this figure is only a guess at the intent of the group. BAR-ACK sequence could be allowed if the group desired.

Additionally, even in the context of an immediate BA agreement, it appears that there is no explicit restriction against sending a BA without a preceding solicitation in the case of a broken MU exchange, such that later users did not receive a BAR but that is a more generic, TGmc sort of issue.

Proposed changes

TGac editor: Add the following new editing instruction, heading, text and figure to TGac Draft 3.1 immediately preceding sub-clause 9.3.4:

Add a new subclause 9.3.2.9a MU acknowledgement procedure as shown:

9.3.2.9a MU acknowledgement procedure

The acknowledgement procedure performed by a STA that receives MPDUs that were transmitted within an MU PPDU is the same as the acknowledgement procedure for MPDUs that were not transmitted within an MU PPDU.

A STA shall not transmit a QoS Data frame within an MU PPDU to a recipient that does not have an outstanding BA agreement with the transmitter for the TID of those QoS Data frames.

NOTE - All MPDUs transmitted within an MU PPDU are contained within A-MPDUs and the rules specified in 8.6.3 (A-MPDU contents) ensure that there is no more than one immediate response required for an A-MPDU, and further indicate that there can only be one immediate response to an MU PPDU.

Responses to A-MPDUs within an MU PPDU that are not immediate responses to the MU PPDU are transmitted in response to explicit BAR frames by the STA transmitting the MU PPDU. An MU PPDU frame exchange sequence is shown in figure 9-9a (The MU PPDU acknowledgement procedure). Recovery within the TXOP can be performed according to the  rules of 9.19.2.4 (Multiple frame transmission in an EDCA TXOP). BAR frames can be transmitted in a TXOP separate from the one that contained the MU PPDU.

Figure 9-9a The MU PPDU acknowledgement procedure

NOTE 1 --- A BA or an ACK is sent for HT-immediate or HT-delayed BA respectively.

NOTE 2 --- No BAR sould be sent to a STA in the case where the A-MPDU to that STA contained no MPDUs requiring acknowledgement.

TGac editor: Add the following new editing instructions, headings and text to TGac Draft 3.1 immediately preceding sub-clause Annex N:

Annex G

G.1 General

Add a new row to table G-1 – Attributes applicable to frame exchange sequence definition:

| Attribute | Description |
| --- | --- |
| mu-mpdu | Frame is part of an MU PPDU |

Add a new subclause G.5 VHT sequences as shown:

G.5 VHT sequences

(* This is the MU PPDU sequence *)

mu-ppdu-sequence =	[(RTS[+HTC] CTS[+HTC]) |	CTS+self ]

Data[+HTC]+individual +QoS +block-ack +a-mpdu+mu-mpdu

1{ Data[+HTC]+individual+QoS+implicit-bar+a-mpdu+mu-mpdu }+a-mpdu-end

BlockAck[+HTC]

1{ BlockAckReq[+HTC] BlockAck[+HTC] };

END OF Proposed Changes

Alternative choices for figure

References: