IEEE P802.11
Wireless LANs

| LB188-CID6408-VHT-QMF-policy |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-08-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | +1 408 543 3370 | mfischer@broadcom.com |

Abstract

Proposed resolution for LB188 CID 6408 which asks for new text to describe the default QMF policy for VHT category action frames.

Revision Notes

R0:

Initial

CID 6408

| CID | Commenter | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 6408 | Yongho Seok | 157.55 | 10.25.1.1 | Please define a default QMF policy for VHT action frames. | As per comment | Revise – Tgac editor to make changes to TGac draft 3.1 as shown in document 11-12-1026r0 under the heading CID6408. |

Discussion

Note that the all VHT action frames are being assigned to AC_VO. The three classes thusfar defined for VHT action include:

VHT Compressed Beamforming

Group ID Management

Operating Mode Notification

VHT Compressed Beamforming will typically be sent as a response frame and is indicated as a time priority management frame, so the default QMF policy for these frames is irrelevant.

Group ID Management feels like a frame that is fairly urgent because it is controlling or managing access to the medium and therefore easily fits into the network management user priority of 7 which maps to AC_VO.

Operating Mode Notification when sent by a STA also has an urgency to it, even though it is not controlling or managing access to the medium for the entire BSS, but instead, only modifying access to and from a single STA. It too, seems appropriately assigned to AC_VO.

Remember that each of the values being assigned here is a default QMF policy which can be changed by the AP.

Proposed changes

TGac editor: Add the following new editing instructions, heading and table to TGac Draft 3.1 immediately following the end of sub-clause “10.22.6.4.5 CCA sensing and NAV assertion in an 80 MHz, 160 MHz or 80+80 MHz direct link”:

Add a new subclause “10.25.1.2 Default QMF policy” as follows:

10.25.1.2 Default QMF policy

Add a new row to “Table 10-12 – Default QMF policy” as follows:

| Description | Management Frame Subtype value from Table 8-1 | Category value from Table 8-38 | Action class | QMF access category |
| --- | --- | --- | --- | --- |
| VHT | 1101, 1110 | 21 | 0-2 | AC_VO |

END OF Proposed Changes

References: