IEEE P802.11
Wireless LANs

| 802.11 TGac WG Letter Ballot LB188 Proposed resolutions to miscellaneous comments on clause 10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eric Wong | Broadcom Corporation | 190 Mathilda Place, Sunnyvale, CA 94086 | +1 408 922 6672 | ewong@broadcom.com |

Abstract

This submission contains proposed comment resolutions to comments received during WG letter ballot 188.

The comments assigned to the author in Clauses 10 are:

6494, 6495

The changes marked in this document are based on TGac Draft 3.1.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

# CID 6494

| 6494 | Mark RISON | 10 | 155.02 | Are the rules in 10.15.3.2 on scanning requirements for a 20/40 MHz BSS required of a VHT AP operating a 20/40 MHz BSS? If so, is there a conflict with 10.38? | Resolve any conflict |
| --- | --- | --- | --- | --- | --- |

Proposed resolution:

Reject. By definition, a VHT STA is also an HT STA. A VHT AP operating in 20/40 MHz channel bandwidth will follow the rules defined in 10.15.3.2.

# CID 6495

| 6495 | Mark RISON | 10.15 | 155.01 | Do all the rules in 10.15 apply to a VHT STA in a 40 MHz BSS? | Check whether the section needs to be generalised and/or made more specific |
| --- | --- | --- | --- | --- | --- |

Proposed resolution:

Reject. Rules in 10.15 do apply to a VHT STA in a 40 MHz BSS, because by definition an HT STA is also a VHT STA.