IEEE P802.11
Wireless LANs

| LB189 D2.0 comment resolution (PHY – aCCAMidTime) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Tevfik Yucek | Qualcomm | 3105 Kifer Road, Santa Clara, CA |  | tyucek@qca.qualcomm.com |

Abstract

This document contains proposed resolution of some of the comments categorized as aCCAMidTime value  in LB189 of P802.11af D2.0. Proposed resolutions are based on 802.11af draft text D2.0.

This submission provides resolution to comments 21 and 327.

- Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGaf Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGaf Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGaf Editor:  Editing instructions preceded by “TGaf Editor” are instructions to the TGaf editor to modify existing material in the TGaf draft.   As a result of adopting the changes, the TGaf editor will execute the instructions rather than copy them to the TGaf Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# aCCAMidTime related comments

| CID | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 21 | 257.19 | 23.4.4 | aCCAMidTime seems really small | aCCMidTime is about 6 symbols in 11ac. Consider using a time corresponding to an equivalent number of symbols |
| 327 | 257.19 |  | Why is aCCAMidTime <25uS? The secondary channel is evaluated for busy PIFS before transmission. In 11n/ac PIFS is 25 uS. In 11af PIFS is 16+20=36uS or 16+24=40uS. Wouldn't it make better sense to use the full 36uS? This might aid false detect performance. | Change aCCAMidTime to < aSifsTime+aSlotTime |

Propose: Revise CID 21 and 327.

The draft 2.0 does not have the correct values from latest PHY submission which was approved during 2012 San Diego meeting. The value of aCCAMidTime should be

< 94 µs (6 or 7 MHz)

< 70 µs (8 MHz)

per the latest approved document.

We believe the correct values (94us and 70us) address CIDs 21 and 327.

# Editing instructions

TGaf editor: Change table 23-18 as shown in Table 23-29-TVHT PHY characteristics of 11-12-0809-05-00af-tgaf-phy-proposal.docx

| Table 23-18-TVHT PHY characteristics |  |
| --- | --- |
| Characteristics | Value |
| aSlotTime | 24 µs (basic channel units: 6 or 7 MHz) 20 µs (basic channel units: 8 MHz) |
| aSignalExtension | 0 µs |
| aCCATime | < 15 µs (6 or 7 MHz) < 11.25 µs (8 MHz) |
| aCCAMidTime | < 94 µs (6 or 7 MHz) < 70 µs (8 MHz) |
| aAirPropagationTime | 3 µs |
| aPPDUMaxTime | 10 ms |
| aPSDUMaxLength | 709920 octets (see NOTE) |
| NOTE—this is the maximum length in octets for SU PPDUs with a bandwidth of 32 or 16+16 MHz, MCS9 and 4 spatial streams, limited by 493 possible Short GI data symbols in aPPDUMaxTime. |  |