IEEE P802.11
Wireless LANs

| Comment resolution for multi-band CAQ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Zhou Lan | NICT | 3-4, Hikarino-oka, Yokosuka, Japan | +81-46-847-5097 | lan@nict.go.jp |
| Keiichi Mizutani | NICT |  |  | mizk@nict.go.jp |
| Gabriel Villardi | NICT |  |  | gpvillardi@nict.go.jp |
| Ha-Nguyen Tran | NICT |  |  | haguen@nict.go.jp |
| Hiroshi Harada | NICT |  |  | harada@nict.go.jp |

Abstract

Submission for comment resolution of LB 189 on multi-band operation of CAQ.

Comments CIDs covered in this document are 350, 426 and 427.

Editing instructions are based on modifying P802.11af_D2.0.

- Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGaf Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGaf Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGaf Editor: Editing instructions preceded by “TGaf Editor” are instructions to the TGaf editor to modify existing material in the TGaf draft.  As a result of adopting the changes, the TGaf editor will execute the instructions rather than copy them to the TGaf Draft.

The editing instructions are shown in bold italic. Four editing instructions are used: change, delete, insert, and replace. Change is used to make corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed by using strikethrough (to remove old material) and underscore (to add new material). Delete removes existing material. Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instruction. Replace is used to make changes in figures or equations by removing the existing figure or equation and replacing it with a new one. Editorial notes will not be carried over into future editions because the changes will be incorporated into the base standard.

This amendment’s baseline is IEEE Std 802.11™–2012, as amended by

- Amendment 1 802.11ae-2012

- Amendment 2 802.11aa-2012

- Amendment 3 P802.11ad Draft 8.0

- Amendment 4 P802.11ac Draft 3.0

| CID | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 350 | 80.27 | 10.42.4 | Add multiband operation for channel availability query | per comment |
| 426 | 80.27 | 10.42.4 | Add a function to annouce that CAQ procedure can be performed in different bands | per comment |
| 427 | 80.27 | 10.42.4 | enhance CAQ that allow CAQ be performed in other bands besides TVWS | per comment |

Discussions:

The FCC regulations of TVWS (FCC 08-260) state "To initiate contact with a fixed or Mode II device, a Mode I device may transmit on an available channel used by the fixed or Mode II TVBD or on a channel the fixed or Mode II TVBD indicates is available for use by a Mode I device on a signal seeking such contacts." This ruling allows the operation that after the mode I TVBDs hearing an enabling signal the TVBDs can obtain the list of available channels, which is required by the FCC for starting operation, in the frequencies band indicated as available. Allowing the mode I TVBDs to contact the Mode II or fixed devices in frequency bands other than the TV bands potentially offloads the system overhead, incurred by the management frames for TVBD operation, in the TV band to other bands.

The Ofcom regulatory requirements document (Regulatory requirement for white space device 4 July 2012) also has similar ruling that a slave device may talk to the master device in a non-UHF band for association with the master device by providing specific characteristics of the slave device to the master device.

Both the FCC and Ofcom regulatory requirements outline the multiband operation of a GDC dependent STA to query a GDC master STA in non-UHF band. It is necessary to reflect these regulatory requirements in the 802.11af specification.

802.11-12/1234r0 proposed a neighbour report mechanism for fast searching of BSS in other bands. We propose to accept this proposal to address comment CID 350, 426 and 427.

Propose Accepted for CIDs 350, 426, 427 per discussion and editing instructions in 11-12/1234r0.