IEEE P802.11
Wireless LANs

| LB188-CID6408-VHT-Action-AC |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-08-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | +1 408 543 3370 | mfischer@broadcom.com |

Abstract

Proposed resolution for LB188 CID 6408 asking for text to provide the default AC for VHT Action frames as it relates to 802.11ae QoS for Management Frames.

Revision Notes

R1:

Change figure so that one STA is an AP

CID 6408

| CID | Commenter | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 6408 | Yongho Seok | 157.55 | 10.25.1.1 | Please define a default QMF policy for VHT action frames. | As per comment | Revise - |

Discussion

The three VHT Action frame categories are all of the type “network control” in that each is making changes to some operating condition, and therefore, all three of the categories should be assigned to AC_VO.

The three categories are:

0 = VHT Compressed Beamforming (Note that this is a time-priority management frame)

1 = Group ID Management

2 = Operating Mode Notification

Management frame subtype values of 1101 and 1110 include both Action and Action NoAck.

Proposed changes

TGac editor: Add the following new editing instruction, heading, text and table to TGac Draft 3.1 immediately preceding sub-clause 10.39 VHT BSS operation:

Insert one new row of information into table 10-12 – Default QMF policy as shown below, excluding the header row, which is shown here only for reference, the new row to be inserted in a location according to a numerically ascending sort on the value found in the “Category value from Table 8-38” column:

| Description | Management Frame Subtype value from Table 8-1 | Category value from Table 8-38 | Action class | QMF access category |
| --- | --- | --- | --- | --- |
| VHT | 1101, 1110 | 21 | 0-2 | AC_VO |

References: