IEEE P802.11
Wireless LANs

| P802.11REVmc Adrian’s additional pre-ballot comment resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-11-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |

Abstract

This submission contains proposed resolutions of some P802.11REVmc technical comments.

Gen “review” comments: (17)

27, 63, 64, 73, 101, 102, 103, 106, 150, 290, 291, 292, 294, 295, 296, 300, 302

Other GEN comments: (55)

3, 73, 125, 218, 228, 229, 231, 183, 119, 28, 61, 35, 275, 111, 114, 152, 129, 134, 146, 116, 320, 165, 332, 234, 319, 268, 267, 266, 265, 262, 261, 255, 237, 235, 250, 248, 252, 256, 257, 251, 358, 39, 74, 142, 87, 44, 26, 90, 182, 226, 32, 67, 113, 1, 188

Deferred to Mark Rison: (5) (i.e., suggest be assigned to him)

269, 127, 29, 293, 178

R2:  reviewed during TGmc session.

# GEN Comments marked “review”

| CID | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- |

| 27 | 113.00 | 6.3.3.3.2 | "The set of MCS values that shall be supported by all HT STAs to join this BSS. The STA that is creating the BSS shall be able to receive andtransmit at each of the MCS values listed in the set."It is extremely odd to find normative behaviour in the description of a structure, particularly when it relates to the output of a scan operation, not an attempt to associate with a BSS. | Move any normative language relating to use of this variable into subclause 10.x. Change the cited text to informative, and avoid the problematical "must".Perhaps something like:"The set of MCS values that are supported by all HT STAs that are a member of this BSS. A STA that is creating a BSS is able to receive and transmit at each of the MCS values listed in the set. (See 10.x)" |
| --- | --- | --- | --- | --- |

Discussion:

802.11ac recently addressed this in their D3.  The language below mirrors their language.

Proposed resolution:

Revised. Make changes in <this document> under CID 27.

The changes in the description of the BSSBasicMCSSet are as follows:

The set of MCS values that are supported by all HT STAs that are members of the BSS. See 10.14a.

The changes for HTOperationalMCSSet are as follows:

The set of MCS values that the peer STA desires to use for communication within the BSS.

See 10.14a.

Question:  do we also need to add such a statement about Basic Rates? – comment fodder.

Changes start here

Create a 10.14a “HT BSS Operation” containing:

An HT STA that is creating or joining a BSS shall be able to receive and transmit at each of the MCS values listed in the BSSBasicMCSSet.

An HT STA that is creating or joining a BSS shall be able to receive at each of the MCS values listed in the STA’s HTOperationalMCSSet.

The STA’s HTOperationalMCSSet shall include all of the MCSs in the BSSBasicMCSSet.

At 113.10 reword description of BSSBasicMCSSet to the following:

“The set of MCS values that are supported by all HT STAs that are members of the BSS. See 10.14a.”

At 113.16 replace “The STA shall be able to receive at each of the data rates listed in the set.” in the description of the HTOperationalMCSSet with “See 10.14a.”

| 63 | 1442.01 | 14 | Delete frequency hopping | as in comment |
| --- | --- | --- | --- | --- |

Discussion:   There are three main aspects to this:

- Deleting the FH PHY clause

- Deleting the FH DSSS option

- Deleting MIB variables

- Deleting PICS entries

- Deleting references to MIB variables

- Deleting references to the deleted sections

- Delete other subclauses specific to FH operation

Proposed Resolution:

Revised.

Delete clause 14 and leave a temporary placeholder there to avoid renumbering the clauses.

Delete Annex I and leave a placeholder there to avoid renumbering the annexes.

Delete Annex K and leave a placeholder there to avoid renumbering the annexes.

Delete Clause 17.4.6.8 (Channel Agility)

Reserve CF3.

At 1823, reserve MD3 and MD4

At 1824, reserve MD7 and MD11

At 1793, reserve PC11.10

Delete B.4.5

Delete FH abbreviation from B.2.2.

Delete dot11PhyFHSSTable in its entirety.

Delete FH from dot11PHYType (both enumeration and description)

Delete the dot11PhyFHSSComplianceGroup and dot11PhyFHSSComplianceGroup2 compliance groups.

Delete references to the dot11PhyFHSSComplianceGroup2 compliance group in the MIB.

Delete FH, FHSS abbreviations

Delete reference to the FH Parameter Set element and 8.4.2.4 from p 148.

On p362 delete: “1/2 symbol period prior to the center of the symbol for FH,” (twice)

Delete FH Parameter Set element, FH Parameters element and FH Pattern Table element from management frame body tables and reorder to fill ordering gap in:

8.3.3.2 (Beacon),

8.3.3.10 (Probe Response)

Delete & mark reserved FH Parameter Set element, FH Parameters element and FH Pattern Table element from Table 8-54.

Delete 8.4.2.4 (FH Parameter Set element)

Delete 8.4.2.11 (Hopping Pattern Parameters element)

Delete 8.4.2.12 (Hopping Pattern Table element)

At 830 delete: “At STAs using an FH

PHY, when there is insufficient time before the next dwell boundary to transmit the subsequent fragment, the

STA initiating the frame exchange sequence may set the Duration/ID field in the last data or management

frame to be transmitted before the dwell boundary to the duration of one ACK time plus one SIFS time.”

At 838 delete: “In a STA having an FH PHY, control of the channel is lost at the dwell time boundary and the STA shall have

to contend for the channel after that dwell boundary. It is required that STAs having an FH PHY complete

transmission of the entire MPDU and associated acknowledgment (if required) before the dwell time boundary.

If, when transmitting or retransmitting an MPDU, there is not enough time remaining in the dwell to allow

transmission of the MPDU plus the acknowledgment (if required), the STA shall defer the transmission by

selecting a random backoff time, using the present CW (without advancing to the next value in the series). The

short retry counter and long retry counter for the MSDU are not affected.”

At 841.62 change as follows:

“Once the STA has contended for the

channel, that STA shall continue to send fragments until either all fragments of a single MSDU or MMPDU

have been sent or an acknowledgment is not received

. Should the sending of the fragments be interrupted

, when the next opportunity for transmission occurs the STA shall resume transmission.”

At 841.60 change as follows:

“The source STA has received an acknowledgment for a previous fragment and has more fragment(s) for

the same MSDU to transmit

.”

At 841.35 delete:

“If the source STA receives an acknowledgment but there is not enough time to transmit the next fragment and

receive an acknowledgment due to an impending dwell boundary, the source STA shall contend for the channel

at the beginning of the next dwell time.”

At 848 delete: “, such as dwell periods of an FH PHY”

At 848 delete: “In a STA having an FH PHY, control of the channel is lost at a dwell time boundary. It is required that the

current MPDU transmission and the accompanying acknowledgment of the MPDU be transmitted before the

dwell time boundary. After having been polled by the PC, if there is not enough time remaining in the dwell to

allow transmission of the MPDU plus the acknowledgment, the STA shall defer the transmission of the MPDU

and shall transmit a Null frame or CF-ACK frame. The short retry counter and long retry counter for the

MSDU shall not be affected.”

At 848/849 delete: “In a STA having an FH PHY, the PC shall not transmit a frame with any data subtype that includes CF-Poll to a STA if there is

insufficient time remaining before the dwell boundary for the STA to respond with a Null frame or CF-ACK

frame.”

At 850 delete: “For operation of the PCF in conjunction with an FH PHY,

dot11MediumOccupancyLimit shall be set equal to the dwell time. For operation in conjunction with other

PHY types,”.

At 852.65 delete:

“After a fragment is transmitted once, contents and length of that fragment

are not allowed to fluctuate to accommodate the dwell time boundaries.”

At 863,  table 9-4 delete modulation class 2 and renumber.

Delete 9.18.3.

At 873, delete “A TXOP shall not exceed dot11MaxDwellTime (if using an FH PHY).”

At 885, delete “dot11MaxDwellTime (if using an FH PHY),”.

At 892, delete “Neither EDCA TXOPs nor MCCA TXOPs shall exceed dot11MaxDwellTime (if using an FH PHY).”

At 981, delete “channel synchronization information (applicable only if the STA contains an FH PHY), and”

At 981, delete “If a Hopping Pattern Parameters element is present in the Beacon or Probe Response frame, and if

dot11MultiDomainCapabilityActivated is true, a STA that is joining an infrastructure BSS shall adopt the

pattern parameters in the element and calculate the hopping patterns using one of the algorithms defined in

8.4.2.11 or 8.4.2.12. Using the appropriate pattern, set, and index values from the FH Parameter Set element,

the STA shall adopt the values in use by the BSS when joining. The dot11RegDomainsImplementedValue

shall be set to Other when the STA is operating using Country element settings.”

At 981/982, delete “In addition to the table entries in 6.3.3.3.2, if a Hopping Pattern Parameters element is present in the Beacon or Probe Response frame, and if dot11MultiDomainCapabilityActivated is true, a STA that is joining an IBSS shall adopt the pattern parameters in the element and calculate the hopping patterns using one of the

algorithms defined in 8.4.2.11 or 8.4.2.12. Using the appropriate pattern, set, and index values from the FH

Parameter Set element, the STA shall adopt the values in use by the IBSS when joining. The

dot11RegDomainsImplementedValue shall be set to Other when the STA is operating using Country

element settings”

At 983 Delete 10.1.6 (Timing synchronization for FH PHYs)

At 1004, delete “g) Modification of the FH Parameter Set” and renumber list.

At 438, reserve the “channel agility” subfield.

At 440, delete “Bit 7 of the Capabilities Information field is used to indicate Channel Agility capability by the High Rate direct sequence spread spectrum (HR/DSSS) PHY or ERP. A STA sets the Channel Agility bit to 1 when

Channel Agility is in use and sets it to 0 otherwise.”

At 446, reserve status code 21.

At 1536, delete “An optional capability for Channel Agility is also provided. This option allows an implementation to overcome some inherent difficulty with static channel assignments (a tone jammer), without burdening all implementations with the added cost of this capability. This option can also be used to implement

IEEE 802.11-compliant systems that are interoperable with both FH and DS modulations. See Annex K for

more details.”

At 1537, remove “For the purposes of MAC and MAC management, when Channel Agility is

both present and enabled (see 17.3.2 and Annex J), the High Rate PHY shall be interpreted to be both a High

Rate and an FH PHY.”

At 1572, remove 17.4.6.8.

At 1575, remove 17.4.6.13

At 1793, reserve PC19

At 1819, reserve HRDS2

At 2189 remove “dot11ChannelAgility*” objects and any references.

Remove any dangling references caused by these removals and reword adjacent text as necessary.

Inform the ANA of any resources administered that should now enter the “reserved” state in the database.

| 64 | 1489.01 | 15 | delete IR | as in comment |
| --- | --- | --- | --- | --- |

Discussion:   There are three main aspects to this:

- Deleting the IR PHY clause

- Deleting MIB variables

- Deleting PICS entries

- Deleting references to MIB variables

- Deleting references to the deleted sections

- Delete other subclauses specific to FH operation

Proposed Resolution:

Revised.

Delete Clause 15 and leave a placeholder to preserve numbering (for now).

Delete dot11PhyIRTable and its contents.

Delete dot11PhyIRComplianceGroup and references to it from within the MIB.

Delete IR definition from B.2.2.

Delete & Reserve item CF5.

Delete B.4.7.

Delete IR from dot11PHYType (enum and description).

Delete IR and IrDA from abbreviations

At 362, delete “or 1/2 slot time prior to the center of the corresponding slot for infrared (IR).” (twice)

At 863 delete modulation class 1 and renumber.

Inform the ANA of any resources administered that should now enter the “reserved” state in the database.

| 73 |  |  | The IEEE 802.11 standard contains quite a few pages and many of them are in clauses marked obsolete. Maybe now would be the time to get rid of some of the obsoleted clauses that were marked as subject to be removed during the last (or the one before) maintenance rounds.. | Consider removing Clause 14 (FH PHY) (including 9.18.4, Annex I, Annex K), Clause 15 (IR PHY), and Annex J (Formal description). |
| --- | --- | --- | --- | --- |

Proposed Resolution.

Revised.  Delete Clause 14 (as shown in resolution for CID 63),  Clause 15 (as shown in the resolution for CID 64) and Annex J.

| 84 | 346.09 | 6.4.7.2.2 | Link-down could be failure to associate, or reassociate. | Change "Reassociate" to "(Re)Associate". Also, in the text at last line of 6.4.7.2.3, change "reassociate" to "(re)associate". |
| --- | --- | --- | --- | --- |

Discussion:

It is arguable that, when the STA has received a Disassociation or Deauthentication request,  it is no longer associated with that AP.  So a “Reassociation” is potentially misleading.

Agree that the STA could perform an Association.

Question:  is association the only valid option?  If so,  we should modify this text further?

Anyhow, accepting that the current reassociation is correct, agree with the proposed changes.

Proposed Resolution:

Accepted.

(note to editor,  also fix font size in “Reassociate with an alternate AP in the same ESS.”)

| 101 |  |  | It's time to let FHSS and IR die | Remove clauses 14 and 15 and associated paraphenalia (e.g. 8.4.2.4, 8.4.2.11, 8.4.2.12, 9.18.3, 10.1.6, B.4.5, B.4.7 and Annex K, and references to them). A global search for FH, FHSS and IR as whole words only would probably be wise |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Revised.   Delete the FH and IR PHYs as shown in the resolutions to CIDs 63 and 64.

| 102 | 1515.00 | 16.3.3 | The maximum MPDU length for the DS PHY is allowed to be just 4 octets. This is silly | In Table 16-2 delete the "4 <= x <= " in the Value for aMPDUMaxLength and delete the parentheses |
| --- | --- | --- | --- | --- |

Discussion:

This attribute is supposed to be a fixed upper bound.  As such showing a range makes no sense.

Proposed Resolution:

Accepted.

| 103 | 1553.00 | 17.3.3 | The maximum MPDU length for the HR PHY is allowed to be just 14 octets. This is silly | In Table 17-5 delete the "14 <= x <= " in the Value for aMP[D]UMaxLength and delete the parentheses |
| --- | --- | --- | --- | --- |

Discussion:

This attribute is supposed to be a fixed upper bound.  As such showing a range makes no sense.

Note CID 104 fixes the mis-spelling of the attribute.

Proposed Resolution:

Accepted.

| 106 | 1761.00 | 20.4.4 | HT does not have an aMPDUMaxLength | Add an aMPDUMaxLength and make it 4095 octets |
| --- | --- | --- | --- | --- |

Proposed Resolution: deferred for Mark Rison.

Rejected.   The limit on MPDU length in 802.11n is determined by the MAC, not the PHY.  It is related to MAC concepts such as A-MPDU aggregation structure and the maximum MSDU/MMPDU size.

Note that 802.11ac will introduce a framework for describing such constraints that will clarify this.

| 150 |  |  | No-one implements PCF | PCF should be pensioned off and everything except CF-End removed from the spec (after a suitable period of mourning) |
| --- | --- | --- | --- | --- |

Discussion:

Note that HCCA is built on PCF.   As and when we actually remove PCF,  we will need to change the architecture diagram.

Need to follow our usual 2-step process.  Note we have used both deprecated and obsolete.

The conventional warning is “The mechanisms described in this subclause are obsolete. Consequently, this subclause may be removed in a later revision of the standard.”

The other type of warning is:  “The use of the DSSS-OFDM option is deprecated, and this option may be removed in a later revision of the standard.”

Let’s stick to obsolete.  If we need to call out behaviour, we can use deprecated.

Proposed Resolution:

Revised.

At 480, the top of 8.4.2.6 (CF Parameter Set element) add:

“The PCF mechanism is obsolete. Consequently, this subclause may be removed in a later revision of the standard.”

At 818, the end of 9.1 (introduction) add.

“The PCF mechanism is obsolete. Consequently, the PCF mechanism may be removed in a later revision of the standard.”

At 819, the top of 9.2.3 (PCF);  and at 844, the top of 9.4 (PCF) add:

“The PCF mechanism is obsolete. Consequently, this subclause may be removed in a later revision of the standard.”

At 1791,  in the Protocol Capbility field for PC4 and PC5 add:

“The PCF mechanism is obsolete. Consequently, this option may be removed in a later revision of the standard.”

| 290 | 2321.01 | I | Remove obsolete annexes | Remove Annex I |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.  (See also resolution for CID 63)

| 291 | 2334.01 | J | Remove obsolete annexes | Remove Annex J |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted. Yea verily.   And the people said “Amen”.

| 292 | 2535.01 | K | Remove obsolete annexes | Remove Annex K |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.  (See also resolution for CID 63)

| 294 | 1442.01 | 14 | Remove obsolete clauses like the FH PHY | Remove the FH PHY clause and all references to the FH PHY. |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Revised.   Remove FH PHY as shown in the resolution to comment 63.

| 295 | 1489.01 | 15 | Remove obsolete clauses like the IR PHY | Remove the IR PHY clause and all references to the IR PHY. |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Revised.   Remove IR PHY as shown in the resolution to comment 64.

| 296 | 104.21 | 6.1 | Change Clause 6 introductory third paragraph to add an additional caution about operation. | At end of third paragraph, add a sentence "This model may not be compatible with operation in any Regulatory Domain or describe combinations of usable features in any Regulatory Domain." |
| --- | --- | --- | --- | --- |

Proposed Resolution:  (approved in Motion GEN-A)

Rejected.   Such a statement is not appropriate in the introduction to the layer management model,  which is about architecture,  not regulatory support.

| 300 | 1631.12 | 19.1.2 | The ERP-PBCC turbo mode and ERP-DSSS PHY modes are obsolete and should be removed. | Remove ERP-PBCC and ERP-DSSS options and all references to them. |
| --- | --- | --- | --- | --- |

Discussion:

I’m not sure whether the commenter intended to specify ERP-DSSS,  which is a mandatory mode of ERP,  or DSSS-OFDM,  which is an optional feature already marked as redundant in std-2012.

We can separately discuss ERP-DSSS, but it makes no sense to remove ERP-PBCC without also removing DSSS-OFDM,  as they are both obsolete options,  and the changes to remove these two options are closely interrelated.

Proposed Resolution.

Revised. It makes no sense to remove ERP-PBCC without also removing DSSS-OFDM, as they are both obsolete options,  and the changes to remove these two options are closely interrelated.  The editing instructions below remove both of these two options.

Remove ERP-PBCC and DSSS-OFDM options as follows:

Remove definition and abbreviation of ERP-PBCC and DSSS-OFDM in Clause 3.

Remove MODULATION_CODE_TYPE 1 and 2 at 365.40.

At 438, reserve the “DSSS-OFDM” field

At 440, delete “and 19.6”

At 440, remove “The use of the ERP-PBCC option is deprecated, and this option may be removed in a later revision of the standard.”

At 441, delete “An AP sets the DSSS-OFDM subfield to 1 in transmitted Beacon, Probe Response, Association Response,

and Reassociation Response MMPDUs to indicate that the use of direct sequence spread spectrum with

orthogonal frequency division multiplexing (DSSS-OFDM), as described in 19.7, is allowed within this

BSS. To indicate that the use of DSSS-OFDM is not allowed, the DSSS-OFDM subfield is set to 0 in

Beacon, Probe Response, Association Response, and Reassociation Response MMPDUs transmitted within

the BSS.

A STA sets the DSSS-OFDM subfield to 1 in transmitted Association Request, Reassociation Request, DLS

Request, and DLS Response MMPDUs when dot11DSSSOFDMOptionImplemented and

dot11DSSSOFDMOptionActivated are true. Otherwise, a STA sets the DSSS-OFDM subfield to 0 in

transmitted Association Request and Reassociation Request MMPDUs.

A STA in an IBSS sets the DSSS-OFDM subfield to 1 in transmitted Beacon and Probe Response MMPDUs

when dot11DSSSOFDMOptionImplemented and dot11DSSSOFDMOptionActivated are true. Otherwise, a

STA in an IBSS sets the DSSS-OFDM subfield to 0.

The use of the DSSS-OFDM option is deprecated, and this option may be removed in a later revision of the

standard.”

At 447, reserve Status Code 26.

At 863, Table 9-4 remove modulation classes 4 and 5 and renumber.

At 844, delete “DSSS-OFDM,” (twice)

At 918 remove “Note that when using the Clause 19 options, ERP-PBCC or DSSS-OFDM, there is no need to use protection mechanisms, as these frames start with a DSSS header.”

At 982 remove “If the DSSS-OFDM bit is 1 in the transmitted Capability Information field of an MMPDU, then any supported rates transmitted in that frame that include rates that are common to both DSSS-OFDM and ERPOFDM shall be interpreted by receiving and transmitting STA to indicate support for both DSSS-OFDM

and ERP-OFDM at the indicated rate. However, if any of those rates are indicated as basic (a rate in the

BSSBasicRateSet parameter), then the basic rate designation shall be interpreted by receiving and

transmitting STA to apply only for the ERP-OFDM modulation and rate.”

At 982 change last sentence as follows: “That is, if the rate is indicated as basic, the basic designation does not

apply to PBCC.”

At 1631 remove: “Two additional optional ERP-PBCC modulation modes with payload data rates of 22 and 33 Mb/s are defined. An ERP-PBCC STA may implement 22 Mb/s alone or 22 and 33 Mb/s. An optional modulation

mode known as DSSS-OFDM is also incorporated with payload data rates of 6, 9, 12, 18, 24, 36, 48, and

54 Mb/s.

The ERP-PBCC option is obsolete. Consequently, this option may be removed in a later revision of the

standard.

The use of the DSSS-OFDM option is deprecated, and this option may be removed in a later revision of the

standard.”

At 1632,  remove “c) ERP-PBCC (Optional)

1) This is a single carrier modulation scheme that encodes the payload using a 256-state packet

binary convolutional code. These are extensions to the PBCC modulation in Clause 17.

ERP-PBCC modes with payload data rates of 22 Mb/s and 33 Mb/s are defined in 19.6.

d) DSSS-OFDM (Optional)

1) This is a hybrid modulation combining a DSSS preamble and header with an OFDM payload

transmission. DSSS-OFDM modes with payload data rates of 6, 9, 12, 18, 24, 36, 48, and

54 Mb/s are defined in 19.7.

2) If the optional DSSS-OFDM mode is used, the supported rates in that mode are the same as the

ERP-OFDM supported rates.”

At 1632, change “The ERP modulation (ERP-OFDM) has been designed to coexist with existing Clause 16 and Clause 17 STAs.” as shown.

At 1633 and 1634, delete “ERP-PBCC: 5.5, 11, 22, and 33

DSSS-OFDM: 6, 9, 12, 18, 24, 36, 48, and 54”

At 1633 and 1635, delete “, ERP-PBCC, DSSS-OFDM” (twice)

At 1633 delete “or DSSS-OFDM”

At 1635 delete “; the optional DSSS-OFDM modulation at all OFDM rates; and the optional ERP-PBCC modulation at all ERPPBCC rates”

At 1635 delete “as well as DSSS-OFDM and ERPPBCC”

At 1636 delete “To support the optional modes included in the ERP, the long preamble PPDU only differs from 17.2.2.2 in the following:

a) The use of one bit in the SERVICE field to indicate when the optional ERP-PBCC mode is being

used.

b) The use of two additional bits in the SERVICE field to resolve the length ambiguity when the

optional ERP-PBCC-22 and ERP-PBCC-33 modes are being used.

c) Three additional optional rates given by the following SIGNAL field octets where the LSB is

transmitted first in time:

1) X'DC' (MSB to LSB) for 22 Mb/s ERP-PBCC

2) X'21' (MSB to LSB) for 33 Mb/s ERP-PBCC

3) X'1E' (MSB to LSB) for all DSSS-OFDM rates”

At 1636 delete “Three bits of the SERVICE field have been defined to support the optional modes of the ERP standard.”

At 1636 change the para “

Bit b2 is used to indicate that the transmit frequency and symbol clocks are

derived from the same oscillator. For all ERP systems, the Locked Clock Bit shall be set to 1, when

transmitting at a data rate described in Clause 17.

Bit b7 is also used to resolve data field length

ambiguities for the CCK 11 Mb/s mode and is defined in 17.2.3.6.

Other bits are reserved and shall be set to 0.” as shown.

At 1636 in Table 19-4 reserve fields for b3, b5, b6.

At 1636 delete 19.3.2.2.3.

At 1638 delete 19.3.2.5

At 1639 delete 19.3.2.6

At 1640 delete 19.3.2.7

At 1643 delete 19.3.3.4

At 1640 delete “Four optional ERP-PBCC modulation formats and data rates are specified for the ERP. They shall be basedon PBCC 5.5, 11, 22, and 33 Mb/s modulations. The rates of 5.5 Mb/s and 11 Mb/s are described in 17.4.6.7. No change in the spectral mask of 17.4.7.4 is required for these modes.”

At 1641, delete 19.3.3.2.

At 1644, delete “The transmit procedures for the optional DSSS-OFDM mode using the long or short PLCP preamble andheader are the same as those described in 17.2.5, and they do not change apart from the ability to transmit a higher rate PSDU using DSSS-OFDM.”

At 1644, delete “the ERP-PBCC modulation at rates of 5.5, 11, 22, and 33 Mb/s;” and “; and/or the DSSS-OFDM modulation rates of 6, 9, 12, 18, 24, 36, 48, and 54 Mb/s”

At 1645 change “ERP-DSSS or ERP-CCK,” a shown

At 1645 delete “A receiver that supports DSSS-OFDM is capable of receiving all rates specified in Clause 16 and all mandatory rates in Clause 18 and Clause 17. If the SIGNAL field indicates 3 Mb/s, the receiver shall

attempt to receive a DSSS-OFDM packet. The remaining receive procedures for a DSSS-OFDM-capable

receiver are the same as those described in 17.2.6, and they do not change apart from the ability to receive

DSSS-OFDM in the PSDU. If DSSS-OFDM is being received, the receiver shall handle the modulation

transition requirements as described in 19.7.3. The receiver shall then follow the receive procedure

described in 18.3.12.”

At 1646 delete “, ERP-PBCC, and DSSS-OFDM”

At 1647, delete 19.6.

At 1646 delete “X'DC' for 22 Mb/s

X'21' for 33 Mb/s”

At 1648 delete 19.7

At 1662, delete 19.8.3.3

At 1662 delete 19.8.3.4

At 1666 delete ERP-PBCC and DSS-OFDM from MODULATION parameter.

At 1671 delete ERP-PBCC and DSSS-OFDM from NON_HT_MODULATION parameter.

At 1672 delete “ERP-PBCC: 5.5, 11, 22, and 33”

At 1672 delete “DSSS-OFDM,” from L_DATARATE.

At 1672 delete “DSSS-OFDM” from SERVICE

At 1673 delete ERP-PBCC and DSS-OFDM from PREAMBLE_TYPE parameter.

At 1678 delete “ERP-PBCC, DSSS-OFDM,” (twice)

At 1683 delete “DSSS-OFDM,”  (twice)

At 1691 delete “DSSS-OFDM,”

At 1793 reserve PC18

At 1825 reserve ERP2 and delete ERP2.1.

At 1825 reserve ERP3.

At 1825 change features for ERP6 as shown: “Set SERVICE field bits for locked clocks, and  length extension (b0, b2, b3, b5, b6, and b7)”

At 1826 ERP9 change “bits b5, b6, and b7” to “bit b7”.

At 1826, reserve: ERP11, ERP12, ERP13, ERP14, ERP15

At 1827, reserve ERP40 and ERP41

At 2191, delete “dot11ERPPBCCOptionImplemented, dot11ERPBCCOptionActivated, dot11DSSSOFDMOptionImplemented and dot11DSSSOFDMOptionActivated ” objects and all references to them.

Remove any dangling references caused by these removals and reword adjacent text as necessary.

Inform the ANA of any resources administered that should now enter the “reserved” state in the database.

| 302 | 1536.25 | 17.1.1 | The PBCC option is obsolete and should be removed. | Remove PBCC option and all references to it |
| --- | --- | --- | --- | --- |

Discussion:

Proposed resolution:

Revised.

1536.30 indicates the PBCC option is obsolete.

Changes for CID 300 removed the ERP-PBCC option.  So the changes shown below remove the remaining PBCC features.

At 37,  remove abbreviations for HR/DSSS/PBCC* (two instances).

At 39, remove abbreviation for PBCC

At 438, Figure 8-38,  reserve the PBCC field

At 440, delete “An AP sets the PBCC subfield to 1 in transmitted Beacon, Probe Response, Association Response, and Reassociation Response management frames to indicate that the packet binary convolutional code (PBCC)

modulation option, as described in 17.4.6.7 and 19.6, is allowed within this BSS; a STA in an IBSS sets the

PBCC subfield to 1 in transmitted Beacon frames when dot11PBCCOptionImplemented is true. Otherwise

an AP or a STA in an IBSS sets the PBCC subfield to 0.

A non-AP STA sets the PBCC subfield to 1 in transmitted Probe Response, Association Request,

Reassociation Request, DLS Request, and DLS Response frames when dot11PBCCOptionImplemented is

true. Otherwise, a STA sets the PBCC subfield to 0.”

At 446, reserve status code 20.

At 982 delete: “If the PBCC bit is 1 in the

transmitted capability field of an MMPDU, then any supported rates transmitted in that frame that include

rates that are common to both PBCC and CCK shall be interpreted by receiving and transmitting STA to

indicate support for both PBCC and CCK at the indicated rate. However, if any of those rates are indicated

as basic, then the basic rate designation shall be interpreted by receiving and transmitting STA to apply only

for the CCK modulation and rate. That is, if the rate is indicated as basic, the basic designation does not

apply to DSSS-OFDM, PBCC, or ERP-PBCC.”

At 1536 delete “An optional mode replacing the CCK modulation with HR/DSSS/PBCC is provided.

The PBCC option is obsolete. Consequently, this option may be removed in a later revision of the standard.”

At 1536 delete “, or HR/DSSS/PBCC/short” and “, or HR/DSSS/PBCC”

AT 1538 delete “c) The use of a bit in the SERVICE field to indicate if the optional PBCC mode is being used;”

At 1540 change as follows: “

Bit 2 shall be used to

indicate that the transmit frequency and symbol clocks are derived from the same oscillator. … An IEEE

802.11-compliant device shall set the values of the bits b0, b1, b3, b4, b5, and b6 to 0.”

At 1540 delete “c) 5.5 Mb/s PBCC Length = (number of octets + 1) 8/5.5, rounded up to the next integer.

d) 11 Mb/s PBCC Length = (number of octets + 1) 8/11, rounded up to the next integer; the

service field (b7) bit shall indicate a 0 if the rounding took less than 8/11 or a 1 if

the rounding took more than or equal to 8/11.”

At 1540 Table 17-1 reserve b3.

At 1540 delete “c) 5.5 Mb/s PBCC Number of octets = (Length 5.5/8) –1, rounded down to the next integer.”

At 1541 delete “d) 11 Mb/s PBCC Number of octets = (Length 11/8)”

At 1541 change “(number of octets + P)” to “number of octets”

At 1541 change “Floor(((Length R) / 8) – P)” to “Floor((Length R) / 8)”

At 1541 delete as follows: “

” (twice)

At 1542 delete “Table 17-3 shows an example calculation for several packet lengths of PBCC at 11 Mb/s.

This example illustrates why normal rounding or truncation of the number does produce the right result. The

LENGTH field is defined in units of microseconds and corresponds to the actual length, and the number of

octets is exact.”  Delete Table 17-3.

At 1546 delete “and HR/DSSS/PBCC/short”

At 1548 delete “If the PHY implements the PBCC modulation

option, it shall detect either CCK or PBCC modulations, as indicated in the SIGNAL field, and shall report

the type of modulation used in the RXVECTOR.”

At 1553 change “(LENGTH+PBCC)” to “LENGTH” and delete the “where” definition for PBCC

At 1554 delete “and 1 means PBCC”.

At 1558 delete “, 5.5 PBCC, or 11 PBCC”

At 1566 delete “An optional PBCC mode is also provided

At 1569 delete 17.4.6.7 (DSSS/PBCC data modulation and modulation rate (optional))

At 1631 change as follows: “DSSS and CCK modulations.”

At 1635 delete “, PBCC”

At 1637 delete “11 Mb/s PBCC: see 17.2.3.6.”

At 1661 delete “PBCC” from this of modulation formats.

At 1666 delete “PBCC” from the MODULATION parameter enumeration

At 1793, reserve PC17

At 1822, reserve HRDS9 and delete HRDS9.1 and 9.2.

At 2189 delete the “dot11PBCCOptionImplemented” object and any references to it.

Remove any dangling references caused by these removals and reword adjacent text as necessary.

Inform the ANA of any resources administered that should now enter the “reserved” state in the database.

Note to EDITOR,  apply change for CID 300 first,  and then apply these edits.

# Other GEN comments

| 3 |  |  | There are 14 instances of the word "obsolete" in the standard. I think our shiny new standard should not include any obsolete material. | Take any obsolete material out of the standard, spindle, fold and mutilate it. Set fire to it. Expunge it completely. Then have a party. |
| --- | --- | --- | --- | --- |

Discussion

The term obsolete is applied to:

- The WDS term

- Hopping

- Infra-red

- PBCC

- ERP-PBCC

- The SDL Annex (J)

Proposed Resolution:

Revised.

Hopping, Infra-red, PBCC, ERP-PBCC and the SDL Annex are removed in response to other comments.   The use of WDS is resolved by comment 301.

| 125 |  |  | "X is present if Y" does not mean X is not present if not Y, but that often seems to be the intent | Go through all the "present if"s and similar, and change to "present if and only if" for those cases where the intent is indeed "X is present if Y and not present if not Y" |
| --- | --- | --- | --- | --- |

Discussion:

Assigned to Mark Rison

Submission required.

Note however in a previous revision we deliberately removed “if and only if”,  which carries connotations of equivalence that are generally not true as used in 802.11.

Recommend we stick to unambiguous language such as “present if y; otherwise not present”

This comment is related to CID28.

| 218 |  |  | What exactly does "mandatory" mean in the context of rates? If a rate is "mandatory", does that mean it has to be included in the basic rate set? Operational rate set? | Clarify |
| --- | --- | --- | --- | --- |

Discussion:

Without a reference to what prompted this, it is hard to know what the specific issue is.

Propose Resolution.  Assigned to Mark.

Rejected.  The commenter doesn’t indicate a specific issue to resolve or specific changes that would satisfy their comment.

In reply, generally we have three things:

- Manadatory rates

- Basic rates

- Operational rates

Operational rates necessarily include all the basic rates.

The STA starting a BSS has complete freedom in selection of the basic rates from the set of rates it supports.

The mandatory rates are the mandatory rates defined “for the attached PHY”, and are fixed by specification.

| 228 |  |  | How do PIFS recovery and EIFS interact? If the EIFS takes you beyond the end of the TXNAV, can you still do PIFS recovery? | Clarify |
| --- | --- | --- | --- | --- |

Proposed Resolution:  Assigned to Mark R

Rejected.  The comment does not indicate an issue to be resolved or specific changes that would satisfy the commenter.

In reply to the commenter, EIFS is used instead of DIFS when using either the DCF or the EDCAF to access the medium.  When PIFS is being used to access the medium,  it is done so without regard to whether a previous frame would have caused EIFS to be used,  if DCF/EDCAF were to be used for channel access.

| 229 |  |  | Why is there a +aSlotTime in the stuff related to the various timeouts anchored off PHY-TXEND? This seems wrong | Remove the + aSlotTimes |
| --- | --- | --- | --- | --- |

Discussion:

We hummed and harred over this in REVmb.  We changed it in response to a comment and then changed it back in response to another  comment that demonstrated the error of the change.

Suggest we don’t change anything unless we get a worked timing diagram that shows that the current spec is wrong.  “This seems wrong” is not adequate proof of wrongness.

Also, it would be kind of nice to know which of the many uses of +aSlotTime “seem wrong”.

Propose Resolution: Mark Rison

Rejected.   The group has carefully reviewed this material in an earlier revision ballot cycle and believes the current use to be correct.   The commenter is invited to identify which uses are incorrect and demonstrate why.

| 231 |  |  | Align EDCA stuff with the way the Other Place does it | Go on, you know you want to |
| --- | --- | --- | --- | --- |

Discussion.

This is an amusing,  but hopelessly inadequate comment.

Proposed resolution:

Rejected.  The commenter has not indicate a specfic issue to be resolved or specific changes that would satisfy the commenter.

| 183 |  |  | The rules for filtering based on the RA/SA/DA/TA are not clear | Clarify them |
| --- | --- | --- | --- | --- |

Discussion:

Filtering occurs in many places.   I’m not sure where this commenter is referring to,  and what is claimed to be unclear.

Proposed resolution: assigned to Mark Rison

Rejected.  The commenter has not indicated a specfic issue to be resolved or specific changes that would satisfy the commenter.

| 119 |  |  | The concepts "MMDU is bufferable" and "PM bit is reserved" need to be separated. It makes no sense to say that an Action MMDU sent by a non-AP STA is bufferable, for example, just because you want to be able to say that the PM is valid in the MPDUs used to send it | Bufferability should only pertain to MMDUs sent by APs or IBSS STAs, not non-AP STAs. Separately, the MPDUs in which the PM bit is valid should be enumerated (being clear that the PM bit in PS-Polls, for example, is not valid, and also that the PM bit in the ACK sent after a data frame sent in immediate response to a PS-Poll isn't not valid either) |
| --- | --- | --- | --- | --- |

Discussion:

It would be nice to know what exactly is the issue by citing the text where it occurs.

However,  I don’t see any issue.  Declaring that a particular action frame is bufferable is merely an act of classification.   It doesn’t affect the behaviour of a device (such as a non-AP STA) for which no behaviour is defined that depends on the bufferability of such MMPDUs.

Status: Assigned to Mark Rison

Note,  the commenter has a submission (11-12/1199) that may address this issue.  This submission should be considered first.

Proposed resolution:

Rejected.  The commenter has not indicated a specfic issue to be resolved or specific changes that would satisfy the commenter.

| 28 |  |  | The WG11 style described in 11-09/1034 describes a template for SAP and element presence ([optionally] present [only] if".There are a mixture of styles in STD 2012. These may come from a concern about ambiguity of the "present only if". Does this mean "present if x and not present otherwise", or does it mean "optionally present if x and not present otherwise"? | I believe it behooves us to be conservative and use the least ambiguous formulation, at the expense of extra words.Therefore I suggest that we replace any "present only if x" with "present if x and not present otherwise", and update the WG11 style guide to match. |
| --- | --- | --- | --- | --- |

It is awkward to describe the changes individually, so a .pdf file is attached showing the changes.

Note the changes at 117.22 and 159.47.

To find changes,  open the .pdf and do a “find all” on “(#28)” (93 instances).

Proposed Resolution:

Revised.  Make changes for (#28) as shown in attached .pdf file.

| 61 |  |  | Delete PLCP/PMD interface from all the PHYs. | as in comment |
| --- | --- | --- | --- | --- |

| 35 | 1504.00 | 16.1.2 | PLCP/PMD split is not a useful concept. | Merge PLCP/PMD into PHY in each PHY clause and update MAC to suit |
| --- | --- | --- | --- | --- |

| 275 |  |  | PMDs -- what are they and are they worth having? | Clarify and/or unite PLCPs and PMDs into one glorious United PHY of PLCP and PMD |
| --- | --- | --- | --- | --- |

Discussion:

Eldad made these changes in 802.11ac D4.0 as shown in 11-12/1009r3, which contains 14 pages of editing instructions.  The most technical changes were in the table of PHY characteristics,   and in the editing of the Tx and Rx procedure diagrams.

It is not possible to say “make changes like 11-12/1009”, although this does give a good example of what we need to see to resolve this comment.

Status: deferred

I have asked Eldad to prepare submissions showing the changes necessary.

| 111 |  |  | The HR PHY doesn't really have all that high a rate, by modern standards | Rename the High Rate PHY to something like the Slightly Higher Than Rock Bottom Low Rate PHY. A global case-sensitive search for HR would probably be wise |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.   While accepting that High Rate is now no longer an accurate description,   changing its name would create more confusion than it resolved.

| 114 |  |  | Reference is made to "the attached PHY" -- but what if there's more than one PHY (as is typically the case)? | See if changing this to "the attached PHY(s)" would work in all cases |
| --- | --- | --- | --- | --- |

Discussion:

We appear to be in two minds as to whether a newer shinier PHY incorporates an earlier PHY by reference,  or sits alongside an embodiment of the earlier PHY.

IMHO, we have only a single PHY – which implies lots of things like a single state machine.  However later PHYs incorporate the behaviour of earlier PHYs by reference.

Also 802.11ad complicates the picture somewhat.  But this is a comment on Std-2012,  so we can ignore 802.11ad for now.

The questions are:

- Do we agree with this model?

- If we agree, does it need clarifying

Proposed Resolution 0: 5 2

Rejected.   A STA has only one attached PHY.   In the case that a PHY provides backwards compatibility to the signals of earlier PHY(s), it is still a single PHY.   It incorporates the behaviour of the earlier PHY by reference.

Alternate Proposed Resolution 1: 2

Revised.

Add the following statement at 104.30, after “the specific manner in which these MAC and PHY LMEs are integrated into the overall MAC sublayer and PHY is not specified within this standard.” add:

“Note that a STA includes only one instance of a PHY entity.  When a STA supports modes of operation that are compatible with those of an earlier-standardized PHY entity, it does so by incorporating the behaviour of the earlier-standardized PHY entity by reference into the later-standardized PHY entity.”

Alternate Proposed Resolution 2: 7 0

Revised.

Add the following statement at 104.30, after “the specific manner in which these MAC and PHY LMEs are integrated into the overall MAC sublayer and PHY is not specified within this standard.” add:

“Note that a STA includes only one instance of a PHY entity.”

Alternate Proposed Resolution 3: 7 6

Revised.

Add the following statement at 104.30, after “the specific manner in which these MAC and PHY LMEs are integrated into the overall MAC sublayer and PHY is not specified within this standard.” add:

“A STA includes only one instance of a PHY entity.”

| 152 |  |  | Is a frame which was not "correctly received" a frame at all? | Look at the 22 instances of "correctly received" and determine whether the "correctly" is actually of any value |
| --- | --- | --- | --- | --- |

Discussion:

The MAC consists of a pipeline of processing steps,  each of which can discard data unit.  These include:

0. PHY RXEND INDICATE (error indications)

1. CRC calculation

2. Protocol Version filtering

3. RA filtering

3.5 Class-based filtering

4. Duplicate cache

5. Decryption MIC check

6. Decryption replay logic

There’s a definition for “correctly received” at 828.10.   It implies that the location where this is observed is between items 2 and 3 above.  I think there’s a definition somewhere (but couldn’t find it) of a received frame as one that gets past 3. above.   So “correctly” does, in theory, add value.  But I doubt whether the unrelated uses of this term are all intended to relate to this specific definition.   Also it makes no sense to have a more specific terms “correctly received” be lower numbered in the list above then the more general form of the term “received”.

However, having reviewed all the uses of the “correctly received” above, I note:

At 959.40, talks about correctly receiving PPDUs, which is an interesting concept.  I suspect it means correctly received at least one frame in a sounding PPDU.

I saw no benefit in most of them.

Proposed resolution: Status – discussed,  but not agreed.

Revised.

Replace all “correctly received” with “received”, except at:

- 826.03 “When a frame containing an acknowledgement is lost, the MAC that initiated the frame exchange does not receive a protocol indication of whether the initial frame was correctly received.”

- At 828 (twice) “slot boundary as defined in 9.3.7 after a correctly received frame, and its backoff time has expired. A correctly received frame is one where the PHY-RXEND.indication primitive does not indicate an error and the FCS indicates the frame is error free.” and “after a correctly received frame, and the backoff time for that AC has expired.”

- 878.12,  Replace “followed by a correctly received ACK frame transmitted by a STA (either a non-AP STA or an AP).” with “followed by a received ACK frame.”

- 959.55 is broken.   Leave it well alone.

- At 1805 (twice): “Hold CCA busy for packet duration of a correctly received PLCP but carrier lost during reception of MPDU”

- At 1812, 1822

| 129 |  |  | The relationship between PHYs is not clear (e.g. are 11 Mbps PPDUs ERP PPDUs?) | Clarify the relationship between PHYs, and whether the PPDUs sent by a PHY which another PHY depends on are considered PPDUs of that other PHY |
| --- | --- | --- | --- | --- |

Discussion:

The term “ERP PPDU” occurs once.

Generally we have three ways of describing PPDUs:

- By the identity of the STA that generates them

- By the modulation class

- By the values of TXVECTOR parameters that determine the format of a PPDU.

Clearly the only thing that matters is OTA formatting, not the identity of the STA that generates them.  So an HT STA can generate an ERP-OFDM (non-HT) PPDU.   The waters are slightly muddied when the OTA signal isn’t exactly the same when generated by the legacy device (e.g. an 802.11g STA) and a later device (802.11n) generating a legacy signal.

Such is the case for an 802.11n device transmitting a non-HT PPDU using more than one antenna – it is the same as the legacy signal, except for antenna-specific cyclic shifts, to avoid unintentional Beamforming.

In the case of an HT STA, it has only a single PHY – the Clause 20 PHY.  But it can generate OTA signals largely described by Clause 19, because this incorporated by reference into Clause 20.  This architectural model is explicit in Clause 19 and Clause 20, and implicit in Clause 17.

So we could define an ERP-OFDM PPDU as follows:

- A PPDU generated by a STA with a Clause 19 PHY,  and with MODULATION=ERP-OFDM

- A PPDU generated by an HT STA with FORMAT=NON_HT, NON_HT_MODULATION=ERP-OFDM

We have some definitions that do this,  for example:

non-high-throughput (non-HT) physical layer convergence procedure (PLCP) protocol data unit (PPDU): A Clause 20 physical layer (PHY) PPDU with the TXVECTOR FORMAT parameter equal to NON_HT.

But we also have a lot of “assumed definitions” such as the use of ERP PPDU.

The question is whether this is adequately clear or not, and if not, if we care enough to change anything.

Straw Polls:

Is the existing architectural model of a PHY with “backwards compatibility” clear enough?

Yes

No

If no, do we care enough to do anything about it?

Yes (note, those voting yes may be asked to write a submission providing recommended changes)

No.

| 134 |  |  | What's the difference between a "transmit power" and a "transmit power level"? | If it's that the former is in units of dB or mW while the latter is an index, then make sure this is clear and that the terms are used correctly in all cases |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.   Commenter does not indicate a problem to be resolved.

Commenter does not indicate specific changes that will resolve the comment.

| 146 |  |  | "multiple" means "two or more" but is sometimes actually used to mean "one or more" | Change all occurrences of "multiple" to "one or more" where appropriate. So, for example, fix the definitions of A-MPDU and A-MSDU in 3.1 so that they do not required an A-MPDU/A-MSDU to contain more than one MPDU/MSDU |
| --- | --- | --- | --- | --- |

Proposed Resoluion:

Revised.

At 6.15 Change “containing multiple MPDUs” to “containing one or more MPDUs”

At 6.20 change “containing multiple MSDUs” to “containing more or more MSDUs”

At 812.20 change “multiple” to “one or more”

| 116 |  |  | The spec often talks of doing things based on the "most recently received" MPDU or MMDU. However, this is not always correct. For instance, if a reassociation was attempted but failed, then the parameters remain those in the (Re)Association Response which led to the current BSS participation, not those in the Reassociation Response which signalled failure. Also, what if a response which was in some way invalid was "most recently received" -- does that count or not? | Check the use of each of the 75 instances of "recently" in the spec and correct those which aren't strictly correct |
| --- | --- | --- | --- | --- |

Discussion:

“recently received” should not apply to capabilities,  as these are static for the lifetime of an association.  Those “recently received” that apply to dynamic values are probably correct, although there are probably many more occurances of references to dynamic fields of frames where we don’t say this.  So it is arguably unnecessary.

854.60 “most recently received Association Response frame that was addressed to this STA.” is OTT.  Under what conditions will it keep state from former associations?

Propose Resolution:

Revised.

At 854.50 change “No HT Operation element is present in the most recently received Association Response frame that was addressed to this STA.” to “No HT Operation element is present in the Association Response frame received from the current AP.”

At 867.60 Change “most recently received HT Capabilities element from” to “HT Capabilities element received from”

At 868.03, 868.15, 868.30, 868.40, 1106.65 delete “most recently received”

At 943.40 delete “most recently transmitted”

At 1097.15 delete “most recently transmitted”

At 1098.32 delete “most recently”

| 320 |  | 10.23.6.3 | "The AP may transmit a group addressed BSS Transition Management Request frame to associated non-AP STAs if all associated non-AP STAs support the BSS Transition Management capability." I appreciate permission to transmit a group addressed frame when all the STA support BSS Transition Management. Now, what am I permitted to do if this is not the case? Since it is not explicitly prohibited I assume I can send a group addressed frame even if some STAs don't support BSS transition management. There is no harm in such a transmission and it may be of value. | Remove statement "The AP may transmit a group addressed BSS Transition Management Request frame to associated non-AP STAs if all associated non-AP STAs support the BSS Transition Management capability." |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Revised.  The intent of the original was to include a constraint.

At the end of the cited sentence add:  “; otherwise the AP shall not transmit a group

addressed BSS Transition Management Request frame.”

| 165 |  |  | What is a "QoS Data frame" or "QoS Data MPDU"? Does this refer to all frames of Type Data and Subtype 1000-1111, or all frames of Type Data and Subtype 1000-1011 (i.e. not QoS Null or immediate relatives), or all frames of Type Data and Subtype 1000 (i.e. not QoS Data + CF-anything)? | Clarify this and then make sure the term has been used correctly throughout the spec |
| --- | --- | --- | --- | --- |

Discussion:

As described in 381,  the “QoS + subtype-bit” terminology is used to select multiple subtypes,  and “(word)” terminology makes that word optional in the name of the subtype.

The convention described at p381 is rather subtle (and perhaps overly so), based on the absence of “data” in the name of the frame.

So “QoS +CF-Ack frame” “refers to the three QoS data subtypes that include “+CF-Ack” (p381.32),  but

“QoS Data+CF-Ack frame” refers to a single subtype.

p414 states: “Data frames with a value of 1 in the QoS subfield of the Subtype field are collectively referred to as QoS data frames. Each of these data subtypes contains QoS in their names, and this frame format is distinguished by the presence of a QoS Control field in the MAC header.”  We don’t have the “absence of the word data” trick played at p381 to indicate there is anything special about this phrase.

This is an unambiguous statement, but it is likely to mislead the reader because “a QoS Data frame” refers to all possible QoS subtypes, but “a QoS Data + CF-Ack frame” refers to a specific subtype.  There is no obvious signal that one has a special interpretation and the other does not.

One way of flagging that special interpretation would be to use the term “+QoS frame” to indicate a data frame with the QoS bit set.

The other issue of disambiguation we can consider is when the terms “QoS Data frame” is intended to mean one of the four QoS subtypes carrying data.   These could be replaced by the following term:  “+QoS +Data frame”.

Straw poll:

Finally, we have a small number of uses of “non-QoS data frame”.  What should we do?  we can:

1. Add a definition to p381,

2. Replace all such uses with a new terminology such as “-QoS frame”.  (i.e. a Data frame with the QoS bit equal to 0).

The most preservative change would be to replace all “QoS Data {frame|MPDU}” with “+QoS {frame|MPDU}”

and add definition at 381:

non-QoS Data frame – a frame of type Data with bit 7 of the Frame Control field equal to 0.

A +QoS frame – a frame of type Data with bit 7 of the Frame Control field equal to 1.

The following locations assume +Data:

93.40, 93.60, 93.63, 94.04, 94.06, 94.08, 391.40, 392.25, 392.64, 393.03, 393.40, …

There’s a lot of these locations to review.  If folks want this change,  the most efficient method would be for me to prepare a speculative draft showing the changes.

A more complete change is to change these specific locations to use the “+QoS +Data frame” terminology and add a definition to p381:

A +QoS +Data frame – a frame of type Data with bit 7 (QoS) of the Frame Control field equal to 1 and bit 6 (no data) of the Frame Control field equal to 0.

Proposed Resolution:

Revised.

The term “QoS data frame” is unambiguously intended to indicate all QoS subtypes, as indicated at 414.10.   However,  using the same term for a specific subtype and a general term creates ambiguity.  The changes below resolve that ambiguity.

Replace all “QoS Data {frame|MPDU}” with “+QoS {frame|MPDU}”

and add definition at 381:

non-QoS Data frame – a frame of type Data with bit 7 of the Frame Control field equal to 0.

A +QoS frame – a frame of type Data with bit 7 of the Frame Control field equal to 1.

Note to editor: another comment has uppercased “data frame -> Data frame”.  Those changes need to be made before these.

| 322 |  | 10.23.6.3 | "may send an unsolicited BSS Transition..." May here is giving permission. Reword as normative statement: "STA shall not send an unsolicited BSS... to a STA that does not support..." | As suggested |
| --- | --- | --- | --- | --- |

Discussion:

Proposed resolution:

Revised.

The existing statement adds value because it highlights the timing of any such transmission.  Better to leave that alone and add the prohibition.

Add to the end of the cited sentence “; otherwise the AP shall not send an unsolicited BSS Transition

Management Request frame to the STA.”

| 234 |  |  | Is b0 of the PVB required to be set if b0 of the bitmap control is and b0 of the PVB refers to AID 0? If AID 0 required to be shown in the PVB if b0 of bitmap control is set? | Clarify |
| --- | --- | --- | --- | --- |

Discussion:

Normative text in Clause 8 describes b0 of the bitmap control as being set to indicate group-addressed traffic.

481.20: “The Bitmap Control field is a single octet. Bit 0 of the field contains the Traffic Indicator bit associated with AID 0. This bit is set to 1 in TIM elements with a value of 0 in the DTIM Count field when one or more

group addressed MSDUs/MMPDUs are buffered at the AP or the mesh STA.”

And 992.55: “The bit for AID 0 (zero) in the Bit Map Control field of the TIM

element shall be set to 1 when group addressed traffic is buffered, according to 8.4.2.7.”

Figure O-2 (an example) shows B0 of the bitmap offset being set to 1 without B0 of the virtual bitmap being set.  So I guess we can view B0 of the virtual bitmap as reserved.

However at 985.63: “The TIM shall identify the STAs for which traffic is pending and buffered in the AP. This information is coded in a partial virtual bitmap, as described in 8.4.2.7. In addition, the TIM contains an indication whether group addressed traffic is pending. Every STA is assigned an AID by the AP as part of the association

process.  AID 0 (zero) is reserved to indicate the presence of buffered group addressed BUs.”

In the case of Multiple BSSID support,  bits 1..k of the partial virtual bitmap is used to indicate non-transmitted BSSID group addressed traffic, according the text at 481.55: “The bits 1 to k of the bitmap are used to indicate that one or more group addressed frames are

buffered for each AP corresponding to a nontransmitted BSSID. The AIDs from 1 to k are not

allocated to a STA. The AIDs from (k + 1) to (2n – 1) are reserved and set to 0. The remaining AIDs

are shared by the BSSs corresponding to the transmitted BSSID and all nontransmitted BSSIDs.”

So, the question is whether this is conflicting or ambiguous or not.   And if it’s ambiguous,  should we change it.

Proposed resolution (in the event that we decide not to change anything):

Rejected.   The comment does not indicate a specific location for the issue, or a specific issue to resolve or a specific change to be made.

| 319 |  | 8.5.14.10 | Contradictory statements need fixing.In 8.5.14.10:The BSS Transition Management Response frame uses the Action frame body format and is *optionally* transmitted by a STA in response to a BSS Transition Management Request frame.In 10.23.6.3:A non-AP STA that supports BSS transition management *shall* respond to an individually addressed BSS Transition Management Request frame with a BSS Transition Management Response frame. | Delete "optionally" from sentence in 8.5.14.10. |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.

The “optionally” in the cited text is correct.   Only a STA that supports this option need respond with transmission of the BSS Transition Management Response frame.

| 271 |  |  | What is the difference between the int/Int/Integer/Round-To-Integer and Floor functions? | Change to Floor everywhere (or possibly Floor (x+0.5) for Round-To-Integer) |
| --- | --- | --- | --- | --- |

Discussion:

There’s no benefit from having multiple equivalent functions – it merely  reflects the heterogenous sources of the standard.

Proposed Resolution.

Rejected.   The commenter has not indicated specific changes that would satisfy the comment.

| 269 |  |  | The PICS needs a good scrubbing | Scrub vigorously |
| --- | --- | --- | --- | --- |

| 127 | 1785.00 | B | The PICS is very messy (e.g. operator precedence is unclear, use of parentheses is random, use of "AND" v. "&" is random, whether to include "N/A", exactly what it means if there are multiple conditions, exactly what happens if none of the predicates are true, etc.) | Clean up the PICS |
| --- | --- | --- | --- | --- |
| 29 | 1785.00 | B.2.1 | In: "O. optional, but support of at least one of the group of options labeled by the same numeral isrequired"it is not clear what is the scope of the numbering. Are group numbers unique throughout the PICS, or only within a Table. | Update to indicate the scope of the numbering. |
| 293 | 1785.02 | B | Change Annex B PICS from normative to informative, because it informatively represents normative requirements, and provides a Support colum for information to be entered by Suppliers. "This annex may not be compatible with operation in any Regulatory Domain or describe combinations of usable features in any Regulatory Domain." | Change Annex B to informative |
| 178 | 1790.00 | B.4.4.1 | DCF (PC3) should not be mandatory for e.g. HT devices | Make DCF optional for all devices which support EDCA (perhaps with a note saying not supporting this would mean they could not join a non-QBSS) |

Status: deferred.

Commenter is working on a submission that may resolve this comment. If the commenter’s submission is not adopted,  we then need to consider the specific issues reported above.

Proposed Resolution: (works for all but 293 and 178).

Rejected.  The commenter does not indicate a specific issue to resolve or a specific change to be made.   However, the commenter is invited to make a submission showing the glowing conclusion of a well-scrubbed PICS.

| 268 |  |  | The filtering rules need to be clearer that the PM bit is ignored even in QoS (+)Null frames that are rejected as a duplicate. The problem is that SNs for QoS (+)Null frames may be set to any value, so if a STA keeps the same value and the initial transmission of a PM bit is missed, and there have been no intervening QoS Data transmissions, the receiver would otherwise not see the PM bit change | Clarify as suggested |
| --- | --- | --- | --- | --- |

Discussion:

What is the location for this change?

A QoS +Null frame should have no effect on the duplicate cache because it carries no payload.

“Sequence numbers for QoS (+)Null frames may be set to any value.”

The proposed change is to clarify that QoS Null frames never make it into the duplicate detection cache.

They then cannot create a SN collision with some other frame.

But how does that affect whether the PM subfield is ignored or not?   It depends on whether you believe that a frame rejected by the duplicate detection logic is “rejected for the purpose of checking the PM subfield” or not.  I could find nothing in the standard that describes when this checking takes place.

My mental picture is that this checking takes place on every received MPDU that generates an Ack or Block Ack.  So, it has passed through CRC and Address 1 filtering, but before duplicate detection.

Do you agree with my mental picture?   If so, it is necessary to state this anywhere?

Proposed Resolution.

Revised.

Insert the following new para at 835.32:

“A receiving STA discards QoS (+)Null frames and Null frames before they reach the duplicate detection logic.   Such frames have no effect on the state of the duplicate detection caches.”

| 267 |  |  | When the spec says "QoS Data" or "QoS data" (or Null equvalents), be clear what is intended -- does it include the +CF-foo subtypes? Similarly when it says "Data frame" or "data frame" -- does this include QoS frames? | Clarify |
| --- | --- | --- | --- | --- |

Discussion:

The resolution of comment 165 (above) clarifies the QoS Data frame position – i.e.,  this term applies to any +QoS subtype.

There is no non-QoS equivalent statement to that found on p411: “Data frames with a value of 1 in the QoS subfield of the Subtype field are collectively referred to as QoS data frames. Each of these data subtypes contains QoS in their names, and this frame format is distinguished by the presence of a QoS Control field in the MAC header.”

Having reviewed the use of “data frame”, the uses appear to be generic – i.e. mean “a frame of type=Data”.   However some of these uses are over-general, e.g. talking about transporting a TDLS management frame in a data frame, it must necessarily be a +Data Data frame.   However,  I am not suggesting we fix this in this resolution.

Proposed Resolution:

Revised.  Make changes as shown for comment 165 above.  These address the interpretation of “QoS Data frame”.

In addition,  at 381.10 add a new para:  “The term Data frame without any further qualification indicates a frame with the Type subfield equal to the value representing Data.”

| 266 |  |  | May all Action frames be sent before the 4WH in a RSNA, or must some be sent after (e.g. ADDTS)? | If some may only be sent after, say which |
| --- | --- | --- | --- | --- |

Discussion:

Commenter indicates this comment is addressed to ADDTS.

Commenter indicates the issue is: “that some AP vendors contend that allowing ADDTS before the 4WH in an RSNA opens the door to some form of DoS attack, and hence want to disallow an ADDTS in State 3”

Discussion:

As currently specified, the completion of the four-way handshake unblocks the 802.1X controlled port (see figure 10-6).  It has no effect on Class-based filtering.  All action frames may be sent in State 3.

An AP can choose to admit or deny an ADDTS request according to any local criteria.   One of these might be the RSNA 4-way handshake completion state of the STA.

You really want a STA attempting an ADDTS to respond to a new status code “addts failed because no RSNA exists” by waiting until completion of the RSN 4-way handshake before retrying the ADDTS.

An alternative is to add some “should” language related to the TSPEC setup,  probably in the ADDTS.request primitive description,  as it relates to behaviour in the SME.

Straw poll:

- Leave it as is,  reject comment

- Add a new status code

- Add “should” language to addts.request primitive

Proposed Resolution:

Rejected.  The commenter does not indicate an issue to be resolved or specific changes to be made.

In reply to the commenter, the completion of the four-way handshake unblocks the 802.1X controlled port (see figure 10-6).  It has no effect on Class-based filtering.  All action frames may be sent in State 3.

| 265 |  | 10.2.1.1 | "The STAs that currently have buffered BUs within the AP are identified in a TIM, which shall be included as an element within all Beacon frames generated by the AP. A STA shall determine that a BU is buffered for it by receiving and interpreting a TIM." -- not true e.g. if some but not all ACs are delivery-enabled | Fix |
| --- | --- | --- | --- | --- |

Discussion:

Cited text is 984.35.

The exclusion of delivery-enabled ACs is described at 990.15:

“At every beacon interval, the APSD-capable AP shall assemble the partial virtual bitmap

containing the buffer status of nondelivery-enabled ACs (if there exists at least one nondeliveryenabled

AC) per destination for STAs in PS mode and shall send this out in the TIM field of the

Beacon frame. When all ACs are delivery-enabled, the APSD-capable AP shall assemble the partial

virtual bitmap containing the buffer status for all ACs per destination.”

So the TIM bit is set according to the delivery-enabled status as described above.

Note, however, it also makes no sense to set it when a S-APSD schedule exists for an AC,  because that traffic will be delivered only during a SP.  We don’t want the STA to generate unnecessary PS-Polls.

On the assumption that nobody cared about S-APSD, we can just ignore this inconsistency too.

Straw poll:

- Fix the inconsistency related to S-APSD?

- Ignore the inconsistency?

- Mark S-APSD as obsolete?

Text at 481.30:

Each bit in the traffic-indication virtual bitmap corresponds to traffic buffered for a specific neighbor peer mesh STA within the MBSS that the mesh STA is prepared to deliver or STA within the BSS that the AP is prepared to deliver at the time the Beacon frame is transmitted.

This is arguably OK, because the AP is not “prepared to deliver” BUs for delivery-enabled ACs except when they are all enabled.

Proposed change (984.35):

The STAs that currently have buffered BUs (excluding those BUs for a STA associated with ACs that are U-APSD delivery-enabled when not all ACs are delivery-enabled by that STA) within the AP are identified in a TIM, which shall be included as an element within all Beacon frames generated by the AP. A STA shall determine that a BU is buffered for it by receiving and interpreting a TIM.

Alternatively,  we could use the “prepared to deliver” language here:

The STAs that currently have buffered BUs within the AP that the AP is prepared to deliver are identified in a TIM, which shall be included as an element within all Beacon frames generated by the AP. A STA shall determine that a BU is buffered for it by receiving and interpreting a TIM.

Proposed resolution:

Revised.

<select one of changes above and copy here>

| 262 |  | 10.2.1 | The introductory stuff does not account for U-APSD, e.g. if there are MSDUs on delivery-enabled ACs only but not all ACs are delivery-enabled, then the TIM will not signal their presence. | Change to cover all types and scenarios of power management, or delete sentences which duplicate information in other sections, and replace with general statement. |
| --- | --- | --- | --- | --- |

Discussion:

Having made the change for CID 265, do we need to do any more?

Proposed Resolution:

Revised. Make changes as shown for CID 265.  These resolve conflict between the U-APSD description and the general description.

| 261 |  |  | The word "frame" is used too loosely. Sometimes it refers to a MSDU or MMPDU, rather than an MPDU forming part of a fragmented MSDU or MMPDU. This affects, for example, whether the PM mode can change during a fragmented MSDU or MMPDU. | Make sure that "frame" is never used to refer to an MSDU or MMPDU. |
| --- | --- | --- | --- | --- |

Discussion:

Agree that a frame should not be used loosely.   It is synonymous with “MPDU” in our definitions.

There are 11,000 instances of frame in the standard.   I don’t know which of these are wrong, and I don’t propose to do the work to find out.

Commenter proposed by email:  “The first change would be to change all the “$management_frame frame”s to “$managament_frame MMPDU”s”

Proposed Resolution:

Rejected.  The commenter has not indicated a specific problem to be resolved or a specific change to be made.

| 255 |  |  | Can an AP send Notify Channel Width? Some parts of the spec suggest it can, some parts suggest it can't | Clarify |
| --- | --- | --- | --- | --- |

764.40 says: “This frame can be sent by both non-AP STA and AP. If an AP wishes to receive 20 MHz packets, it

broadcasts this Action frame to all STAs in the BSS. In addition, the AP indicates its current STA channel

width in the HT Operation element in the beacon.”

I reviewed all other uses of this frame, and found none that implied it was limited to non-AP STAs.

Proposed Resolution:

Rejected.  764.40 states unambiguously that this frame can be used by “both non-AP STA and AP”.  None of the other uses of “Notify Channel Width” conflict.

| 237 |  |  | Why is it that only in the case of PS-Poll with all ACs being DE priority is given to higher-priority ACs? | Make this principle more general |
| --- | --- | --- | --- | --- |

Discussion:

The cited situation only arises when an AP receives a PS-Poll from a STA that has set all its ACs to be delivery-enabled.

In the case, for example, that a non-AP STA sets one AC to be delivery enabled, the AP will buffer data for all ACs.   On receiving a PS-Poll, it will transmit an MSDU to the STA, but there is no indication of which AC that should come from.  The commenter proposes that we should indicate which.

There are two possible extensions:

1. Select from amongst non-delivery ACs when one or more ACs are delivery-enabled

2. Select from amongst non-delivery ACs regardless of whether any ACs are delivery enabled.

We also have the question of creating legacy interoperability issues,  so anything we do add needs to be no stronger than a “should”.

On balance,  the AP generally, except in the specific case of all ACs being delivery enabled, has the option of defining priority internally and selecting which of several eligible MSDUs to transmit,  and there seems little benefit from describing this.

The related text is: 99.60

A single buffered BU for a STA in the PS mode shall be forwarded to the STA after a PS-Poll has been received from that STA. For a STA using U-APSD, the AP transmits one BU destined for the STA from any AC that is not delivery-enabled in response to PS-Poll from the STA. When all ACs associated with the STA are delivery-enabled, AP transmits one BU from the highest priority AC. The AP can respond with either an immediate data or management frame or with an ACK, while delaying the responding data or management frame. For a STA in PS mode and not using U-APSD, the More Data field of the response data or management frame shall be set to indicate the presence of further buffered BUs for the polling STA. For a STA using U-APSD, the More Data field shall be set to indicate the presence of further buffered BUs that do not use delivery-enabled ACs. When all ACs associated with the STA are delivery-enabled, the More Data field shall be set to indicate the presence of further buffered BUs using delivery-enabled ACs. If there are buffered BUs to transmit to the STA, the AP may set the More Data bit in a QoS +CF-Ack frame to 1, in response to a QoS data frame to indicate that it has one or more pending BUs buffered for the PS STA identified by the RA in the QoS +CF-Ack frame. An AP may also set the More Data bit in an ACK frame to 1 in response to a QoS data frame to indicate that it has one or more pending BUs buffered for the PS STA identified by the RA in the ACK frame, if that PS STA has set the More Data Ack subfield in the QoS Capability element to 1. Further PS-Poll frames from the same STA shall be acknowledged and ignored until the BU has either been successfully delivered or presumed failed due to maximum retries being exceeded. This prevents a retried PS-Poll from being treated as a new request to deliver a buffered BU.

Proposed Resolution:

Rejected.   There is nothing to stop an AP internally prioritising one (non-delivery-enabled) AC over another in responding to a PS-Poll.  Adding mandatory statement to this effect might make legacy devices non-compliant.

| 235 |  |  | It might be useful if PS-Polls could trigger delivery-enabled traffic, if all ACs are delivery-enabled | Allow this |
| --- | --- | --- | --- | --- |

Commenter withdraws comment, 2012-10-24:  “Actually, the spec already allows this.  This comment is a duplicate of CID 213, which I withdrew a couple of weeks ago.  I withdraw this one too!”

Proposed Resolution:

Rejected.  Commenter has withdrawn the comment.

| 250 |  |  | Can BA/BAR be sent to start TXOP for HT-immediate BA? Annex G suggests it can, asπÇÇlong as it's not followed by anything:txop-sequence =[...] ¦[RTS CTS] (BlockAckReq BlockAck) ¦[...]Note no "+delayed-no-ack"! | Fix Annex G |
| --- | --- | --- | --- | --- |

Discussion:

Annex G indicates that a BlockAckReq BlockAck sequence can form the body of a txop-sequence, including at the start of the sequence.  So, yes, they can be sent (according to Annex G) to start a TXOP sequence.

In the case of delayed Block Ack,  this is covered by the term “txop-part-requiring-ack txop-part-providing-ack”.

It is hard to know exactly what is the issue,  because we have a question,  a note an an instruction to fix.

Commenter indicates by email:  “The issue is that a BA at the start of a TXOP should not be allowed for HT-immediate BA”

My analysis is that the commenter is incorrect.   A TXOP may start with a BAR/BA exchange, which may be followed by data.  See definition of txop-sequence at 2311.32.

Also the proposed change “BA at the start of a TXOP should not be allowed for HT-immediate BA” might make existing devices non-compliant.

Proposed Resolution:

Rejected.  The assertion that BAR/BA cannot be followed by data in a TXOP is incorrect. A TXOP can be filled with txop-sequence terms, the first of which can be a BAR/BA, and subsequent ones can contain data.  See 2311.32.

| 248 |  |  | Not all frames have rules for which AC they are assigned to for the purposes of admission control | Add rules to make sure all frames are covered |
| --- | --- | --- | --- | --- |

Discussion:

It would be nice to know a specific problem,  rather than an assertion that invites us to check all frame types.

Commenter indicates by email that CF-End and NDP are the PPDUs for which there are no AC rules.

Proposed resolution:

Rejected.   The commenter does not indicate a specific change to be made.

| 252 |  |  | "The X element is present" -- does this allow more than one X element to be present? | When it's just one, say "Exactly one". When it's any number, say "One or more" |
| --- | --- | --- | --- | --- |

Discussion:

I don’t believe saying “an X is present” or “the X is present” is ambiguous.   In my opinion,  this is precisely a single occurance,  or zero or one occurances when “optional” is included “an X element is optionally present”.

There multiple elements are present, we have explicit language calling that out.   e.g., “One or more Multiple BSSID elements are present if … ”

Further, we don’t want to raise the spectre of pretending that “an X element” or “the X element” is ambiguous, because then the logical follow-on would be to change every occurance of “a/an” or “the” to “a single” “the single”.  There are 76,000 “the’s” and 21,000 “a’s”.  I’m not volunteering to review these or change any substantial fraction of them.

Proposed Resolution:

Rejected.   “The X element is present…” refers to a single X element.  “The X element is optionally present” refers to zero or one X elements.  Where this is not the case other expressions are used,  such as at 421.40:  “One or more Multiple BSSID elements are present…”

| 256 |  |  | If a STA sends an operating mode notification saying it will henceforth only operate at 20 MHz, does this mean all subsequent broadcasts have to be 20 MHz? | Clarify |
| --- | --- | --- | --- | --- |

Discussion:

I feel the commenter is looking for free consultancy.

“Operating mode notification” is a TGac concept.

Proposed Resolution:

Rejected.

“Operating Mode notification” is not a concept in the 802.11REVmc draft, although we do note that it is a concept in a forthcoming amendment, not yet incorporated into REVmc.

But relating the question to “Notify Channel Width”, the text at 1098.01 makes it clear that the AP should not (no abosolute prohibition) transmit 40 MHz group addressed frames in the cited example.

| 257 |  |  | Reassociation to the same AP behaviour is not clearly defined (e.g. effect on TSes, whether failure leaves you unassociated, whether you need to re-do 4WH, meaning of PM bit in Reassociation Request, etc.) | Clarify |
| --- | --- | --- | --- | --- |

Discussion:

I believe that reassociation to the same AP should be exactly the same as an initial association,  except for:

1. What to do when the reassociation request fails, in which case we should leave things unchanged

2. The effect on the DS may be different, as the address map already exists and is current

We could attempt to clarify this by a statement in 10.3.

Proposed Resolution:

Revised.   Add the following to 10.3.5.5 at 1021.15,  after item i).

“When the ResultCode of the reassociation is SUCCESS, and the CurrentAPAddress in the MLME-REASSOCIATE.indication is the address of the AP (i.e., the non-AP STA is reassociating to its current AP), the SME treats this as a new association (i.e., discards any retained state related to the earlier association), except:

- MSDUs queued or buffered for that STA might be retained for subsequent transmission

- The DS might not be notified of any change of association state.”

| 251 |  |  | For PIFS recovery, you hang around, sample the medium just before the interval you successfully reserved expires and then if there's no-one thereat that particular instant you grab the medium again? Seems wasteful and unfair! | Say it ain't so |
| --- | --- | --- | --- | --- |

Discussion:

Commenter states by email: “That’s not what the text straddling the bottom of p. 877  says (“the TxPIFS slot boundary before the expiry of the TXNAV timer”)”

The TxPIFS slot boundary is related to transmit and CCA events,  not the TXNAV timer.

It is arguable whether the transmission cited at 877.65 is subject to the rules at 877.40: “the duration of transmission of that frame plus any expected acknowledgment for that frame is less than the remaining TXNAV timer value”.

We could clarify this in the following change:

After a valid response to the initial frame of a TXOP, if the Duration/ID field is set for multiple frame transmission and there is a subsequent transmission failure, the corresponding channel access function may transmit after the CS mechanism (see 9.3.2.1) indicates that the medium is idle at the TxPIFS slot boundary (defined in 9.3.7) provided that the duration of transmission of that frame plus the duration of any expected acknowledgment and applicable IFS is less than the remaining TXNAV timer value. At the expiry of the TXNAV timer, if the channel access function has not regained access to the medium, then the EDCAF shall invoke the backoff procedure that is described in 9.19.2.5. Transmission failure is defined in 9.19.2.5.

Proposed Resolution:

Revised.   At 878.01,  replace “before the expiry of the TXNAV timer” with “provided that the duration of transmission of that frame plus the duration of any expected acknowledgment and applicable IFS is less than the remaining TXNAV timer value”.

| 358 | 0.00 |  | The dot11RTSThreshold is described as related to the length of a frame, the length of a fragment and the length of a PSDU. These are inconsistent, given that a PSDU may contain an A-MPDU which contains multiple frames. This comes indirectly from a comment in LB188. | Make references consistent. |
| --- | --- | --- | --- | --- |

Discussion:

This parameter is particularly useless.

What it should really relate to is the relative duration of the RTS frame and the unit of transmission that is lost by a possible collision.   Given that PHY rates can vary a factor of 100,  relating RTS to a unit of octets is not very helpful,  regardless of which container holds those octets.

Futhermore, there are additional reasons to perform RTS/CTS, such as in establishing a TXOP,  or providing protection of later frames encoded using a new encoding.

However, having said how useless it is, we should at least resolve any contradictions in our spec,  which arise from adding:

1. Fragmentation (a really really long time ago)

2. A-MSDU

3. A-MPDU

I recommend that the best outcome is to relate it to PSDU length and being agnostic as to whether the PSDU holds a fragment, a whole MSDU,  and A-MSDU or an A-MPDU.

The changes to achieve this follow:

At 830.20:

Each fragment and ACK acts as a virtual RTS and CTS; therefore no further RTS/CTS frames need to be generated after the RTS/CTS that began the frame exchange sequence even though the PSDUs carrying subsequent fragments may be larger than dot11RTSThreshold.

At 837.50:

The SSRC shall be reset to 0 when a CTS frame is received in response to an RTS frame, when a BlockAck frame is received in response to a BlockAckReq frame, when an ACK frame is received in response to the transmission of a frame in a PSDU of length greater than dot11RTSThreshold containing all or part of an MSDU or MMPDU, or when a frame with a group address in the Address1 field is transmitted.

At 840.20:

After transmitting a frame that requires acknowledgment, the STA shall perform the ACK procedure, as defined in 9.3.2.8. The SRC for an MPDU of type Data or MMPDU and the SSRC shall be incremented every time transmission of a MAC frame in a PSDU of length less than or equal to dot11RTSThreshold fails for that MPDU of type Data or MMPDU. This SRC and the SSRC shall be reset when a MAC frame in a PSDU of length less than or equal to dot11RTSThreshold succeeds for that MPDU of type Data or MMPDU. The LRC for an MPDU of type Data or MMPDU and the SLRC shall be incremented every time transmission of a MAC frame in a PSDU of length greater than dot11RTSThreshold fails for that MPDU of type Data or MMPDU. This LRC and the SLRC shall be reset when a MAC frame in a PSDU of length greater than dot11RTSThreshold succeeds for that MPDU of type Data or MMPDU. All retransmission attempts for an MPDU of type Data or MMPDU that has failed the ACK procedure one or more times shall be made with the Retry field set to 1 in the Data or Management type frame.

At 879.40:

After transmitting a frame that requires an immediate acknowledgment, the STA shall perform either of the acknowledgment procedures, as appropriate, that are defined in 9.3.2.8 and 9.21.3. The short retry count for an MSDU or A-MSDU that is not part of a Block Ack agreement or for an MMPDU shall be incremented every time transmission of a frame in a PSDU of length less than or equal to dot11RTSThreshold fails for that MSDU, AMSDU, or MMPDU. QSRC[AC] shall be incremented every time transmission of an A-MPDU or frame in a PSDU of length less than or equal to dot11RTSThreshold fails. This short retry count and the QoS STA QSRC[AC] shall be reset when an A-MPDU or frame in a PSDU of length less than or equal to dot11RTSThreshold succeeds. The long retry count for an MSDU or A-MSDU that is not part of a Block Ack agreement or for an MMPDU shall be incremented every time transmission of a MAC frame in a PSDU of length greater than dot11RTSThreshold fails for that MSDU, A-MSDU, or MMPDU. QLRC[AC] shall be incremented every time transmission of an A-MPDU or frame in a PSDU of length greater than or equal to dot11RTSThreshold fails. This long retry count and the QLRC[AC] shall be reset when an A-MPDU or frame in a PSDU of length greater than dot11RTSThreshold succeeds. All retransmission attempts for an MPDU that is not sent under a Block Ack agreement and that has failed the acknowledgment procedure one or more times shall be made with the Retry field set to 1 in the data or management frame.

At 2134.05:

This attribute indicates the maximum number of transmission attempts of a frame, in a PSDU of length that is less than or equal to dot11RTSThreshold, that are made before a failure condition is indicated.

At 2134.20:

This attribute indicates the maximum number of transmission attempts of a frame, in a PSDU of length that is greater than dot11RTSThreshold, that are madebefore a failure condition is indicated."

Proposed Resolution:

Revised.  Relate the parameter to PSDU length as follows by making the changes as shown under CID 358 in 11-12/1229r<latest>.

| 39 | 1.00 | 1 | Commenting is onerous | Change the 802.11 template to add page numbers at both the top and bottom of every page. Saves 5 sec per CID |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.   The PDF reader will show somewhere on its user-interface the current “logical” page number.

The editor will endeavour to ensure that these page number match the printed page numbers.

| 74 | 26.00 |  | The set of bufferable management frames seem to be currently defined only in the definitions clause (3.2, page 26). This type of normative categorization of frame types would be nice to include somewhere in more formal way. | Consider adding a table of bufferable management frames in 10.2. |
| --- | --- | --- | --- | --- |

Discussion:

The term “bufferable management frame” is part of the ongoing confusion about MMPDUs and frames.

The thing that is buffered is actually an MMPDU.  Depending on how it is delivered, a single buffered MMPDU might be transmitted in multiple Management frames.

This confusion has given rise to misleading statements such as 1015.32:

“NOTE—as the Deauthentication frame is a bufferable MMPDU, the transmission of this frame might be

delayed by the operation of a power-saving protocol. The AID and the PTKSA are maintained (when

applicable) until the frame is acknowledged or attempts to transmit the frame are abandoned.”

However, I don’t see that we can attempt to fix this up all by itself, without addressing the bigger picture (See CID98).  So I don’t propose to resolve this terminological inexactitude at the moment.

Proposed Resolution:

Revised.  Make changes as shown under CID 74 in 11-12/1229r<latest> which achieve the commenter’s intent.

Changes:

At 26.10:

bufferable management frame: A Management frame that is buffered for delivery according to a power-saving protocol. See 10.2.0a.

Insert subclause 10.2.0a before 10.2.1:

10.2.0a Bufferable Management frames

Management frames are categorized as bufferable or nonbufferable, as shown in Table 10-x.

Bufferable Mangement frames are eligible to be queued for delivery using a power-saving mechanism.

Non-bufferable Management frames are delivered without reference to a power-saving mechanism.

Table 10-x – Bufferable/nonbufferable classification of Management frames

| Description | Classification |
| --- | --- |
| An Action, Disassociation, or Deauthentication frame | Bufferable |
| an individually addressed Probe Response frame that is sent in an IBSS in response to an individually addressed Probe Request frame | Bufferable |
| All other Management frames | Nonbufferable |

| 142 | 75.00 |  | 4.5.4.4 says "This standard does not provide data confidentiality for group addressed robust management frames" but 4.5.4.9 says "Management frame protection protocols in an MBSS apply to [...] group addressed frames indicated as "Group Addressed Privacy" in Table 8-38" | Amend 4.5.4.4 to give an exception for some group-addressed frames in an MBSS |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Revised.  At the end of the sentence at 75.15 “This standard does not provide data confidentiality for group addressed robust management frames.” add “, except for certain Action frames related to mesh operation,  as indicated in Table 8-38. “

| 87 | 94.01 | 10.4.4.3 | Figure 10-9 (the one added as 10-8a by 11aa) has SME doing the timer (is that correct, it seems odd?), and poor /missing dot notation on the service primitives. | (Page numbers are from 802.11aa-2012) Fix the dot notations. Consider the timer. (There is no text describing the timer, which would help undertand it.) |
| --- | --- | --- | --- | --- |

Discussion:

This is a comment on 802.11aa, and therefore not properly in scope of this review.  However, it does raise some points we can respond to.

In P802.11REVmc D0.3,   Figure 10-9 (p1140) has been modified to replace the dot notation with an MSC loop notation.   The TS Setup Timer is not strictly necessary, as it illustrates part of the protocol that is outside the scope of this standard.

Figure 10-8 includes an ADDTS timer as part of the MLME operation.  This is incorrect,  because the TIMOUT ResultCode was removed during REVmb.

Proposed Resolution:

Revised.   In D0.3 Figures 10-8 and 10-9,  remove the timers.

| 44 | 116.00 | 6.3.4.2.2 | "Operation" is used in multiple conflicting ways. BSSBasicMCSSet (clause 6) and HT Operation element are for the BSS but HTOperationalMCSSet (clause 6) and the HT Capabilities element are for the STA. Worse, the STA Channel Width field in the HT Operation element is for the BSS not the STA. (BTW, 11ac is using Operation Mode Notification in hte per STA sense) | Rename the Operational in xxOperational Rate/MCS Set to something else (Activated?). Rename STA Channel Width on p614 to BSS Channel Width. (Bonus: clarify that it is for both the AP and BSS) |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.   “Operational” can apply to multiple types of operation.   There is no compelling need to find different synonyms to apply to different contexts.

“STA Channel Width” is correct terminology.  It refers to the operating channel width of the STA transmitting the HT Operation element.   1091.15 constrains the values of this field in the case of transmission by an AP so that it is wholly dependent on the SCO field.   But such constraint does not exist in the case of an IBSS, in which case the STA Channel Width field tells us something about the STA’s capabilities as opposed to characteristics of the BSS.

| 26 | 116.00 | 6.3.4.2.2 | The JOIN request has very little purpose for the infrastructure case, as it doesn't cause any frame exchange sequence.In the IBSS case, it starts operation of the STA as a member of the BSS, analoguous to the START.request.Apart from those properties of the BSS that are adopted, the JOIN.request should, at least in the IBSS case, specify the same parameters of the START.request. Specifically this includes all capabilities of the STA. | Review JOIN.request against START.request, and add any parameters missing from the JOIN that are not inherited from the BSS (i.e. not in the BSSDescription). |
| --- | --- | --- | --- | --- |

Discussion:

The purpose of the JOIN.request is to specify all those parameters that are necessary for IBSS operation, those parameter that might go in a probe request frame,  those parameters that might go in an authentication frame and those parameters that might go in an association request frame that are no specified in the authentication.request and association.request primitives.

Note, this primitive is not relevant to Mesh, as a mesh STA always performs a START.request.

The commenter makes a logical error.   The BSSDescription describes the properties of a discovered BSS, not the properties of the STA performing the JOIN.request.  So an OperationalRateSet field in the BSS Description is the OperationalRateSet of the STA that transmitted the Beacon/ProbeResponse, not of the STA performing the JOIN.req.  The presence of a field in the BSS Description does not excuse the need to have the same field in the JOIN.request describing the STA’s capabilities, if its presence is necessary for “JOIN” operation (i.e., operation in an IBSS, transmitting an authentication frame, transmitting an association frame).

The START.request parameters are:

| START.request Parameter | Property of the BSS? | JOIN.request Parameter | In BSS Description? |
| --- | --- | --- | --- |
| SSID | Yes |  | Yes |
| SSIDEncoding (see notes below) | Yes |  | No |
| BSSType | Yes |  | Yes |
| BeaconPeriod | Yes |  | Yes |
| DTIMPeriod | Yes |  | Yes |
| CF parameter set | Yes |  | Yes |
| PHY parameter set | Yes |  | Yes |
| IBSS parameter set | Yes |  | Yes |
| ProbeDelay | No | ProbeDelay | No |
| CapabilityInformation | No | <missing> | Yes |
| BSSBasicRateSet | Yes |  | Yes |
| OperationalRateSet | No | OperationalRateSet | Yes |
| Country | Yes |  | Yes |
| IBSS DFS Recovery Interval | Yes |  | Yes |
| EDCAParameterSet | Yes |  | Yes |
| DSERegisteredLocation | Yes |  | Yes |
| HT Capabilities | No | <missing> | Yes |
| HT Operation | Yes |  | Yes |
| BSSMembershipSelectorSet | Yes |  | Yes |
| BSSBasicMCSSet | Yes |  | Yes |
| HTOperationalMCSSet | No | HTOperationalMCSSet | Yes |
| Extended Capabilities | No | <missing> | Yes |
| 20/40 BSS Coexistence | No | <missing> | Yes |
| Overlapping BSS Scan Parameters | Yes |  | Yes |
| MultipleBSSID | Yes |  | Yes |
| InterworkingInfo | No | <missing> | No |
| AdvertismentProtocolInfo | No | <missing> | No |
| RoamingConsortiumInfo | Yes |  | No |
| Mesh ID | Yes |  | Yes |
| Mesh Configuration | Mixture of BSS and STA |  | Yes |
|  |  | Join Failure Timeout |  |

Let’s discuss some possible anomalies here:

- SSIDEncoding describes the encoding of an accompanying SSID.  Affects the setting of a bit in the Extended Capabilities subfield in Beacon and Probe Response frames.  That subfield is used to affect the interpretation of the received SSID: 478.50 “the SSID is interpreted using UTF-8 encoding.”

  - Question is where is this “interpretation”?   In the MLME, or the SME?

  - But this parameter only affects the setting of a bit in the Extended Capabilities element,  and we also have an Extended Capabilities parameter.   So the presence of this as a separate parameter performs no purpose.

  - Recommended change:   Delete the SSIDEncoding parameter and fix up any references to it.

- CapabilityInformation, HT Capabilities and Extended capabilities should be added to the JOIN.request.

- 20/40 BSS Coexistence should be added.  Note,  see 113.30,  which clarifies that this element is not adopated.

- InterworkingInfo and AdvertisementProtocolInfo may be transmitted by non-AP STAs.  Should be added.

- Join Failure Timeout in the JOIN.request is an anomaly.   REVmb removed the TIMEOUT StatusCode (probably wrongly).

  - At 981.32: “If the JoinFailureTimeout timer expires prior to the receipt of a Beacon frame from the BSS, the MLME shall issue an MLME-JOIN.confirm primitive indicating the operation was unsuccessful.”

  - So arguably we’re OK.  In the interests of conservatism, will leave it there.

  - Add a TIMEOUT value to the ResultCode parameter.

Proposed Resolution:

Revised.

At 147.55, delete the “SSIDEncoding” parameter, and entry from parameter table.

At 151.33, delete “If an MLME receives an MLME-START.request primitive with the SSIDEncoding parameter value UTF8, the MLME shall set the UTF-8 SSID subfield of the Extended Capabilities element to one in Beacon and Probe Response frames.”

At 478.55, delete “NOTE—This is true for Beacon and Probe Response frames when the MLME-START.request primitive was issued with the SSIDEncoding parameter equal to UTF8.”

At 117.20,  add a TIMEOUT value to the ResultCode enumeration.

At 116.10,  add the following parameters,  copying to the parameter table the matching entry from the START.request:

- Capability Information

- HT Capabilities

- Extended Capabilities

- 20/40 BSS Coexistence

- InterworkingInfo

- AdvertisementProtocolInfo

| 90 | 180.23 | 6.3.26.6.2 | Get rid of all TIMEOUT and UNSPECIFIED_FAILURE ReasonCode values, unless there is discussion of them explicit in the protocol/behavior (and then give them a descriptive name) | Get rid of all TIMEOUT and UNSPECIFIED_FAILURE ReasonCode values, unless there is discussion of them explicit in the protocol/behavior (and then give them a descriptive name) |
| --- | --- | --- | --- | --- |

Discussion:

We agreed to do this in REVmb days.   Any remaining such ReasonCodes are either a failure in REVmb,  or were washed up on the tide of later amendments.

However, this work revealed that we have a related problem with ADDBA.request/confirm.

The ADDBA.request has a parameter that is a timeout value (BlockAckTimeout).   The 10.5 text related to timeouts refers to a MIB variable dot11ADDBAFailureTimeout that does not exist.   The same text refers to a TIMEOUT code.  To make matters worse,  there is normative text relating to this non-existent MIB variable.  The timeout is also implicitly referred to from Figure 10-12.

At 1032.50:

If the initiating STA is an HT STA, is a member of an IBSS, and has no other existing Block Ack agreement with the recipient STA, then the initiating STA shall transmit a Probe Request frame to the recipient STA and shall not transmit an ADDBA Request frame unless it receives a Probe Response frame from the recipient within dot11ADDBAFailureTimeout.

At 1034.40:

If there is no response from the recipient within dot11ADDBAFailureTimeout, the STA has not established a Block Ack mechanism with the recipient STA; and the MLME shall issue an MLMEADDBA. confirm primitive with a result code of TIMEOUT.

Straw poll: (yes/no votes for each)

- Keep the timeout parameter, fix references to wrong name.  Restore TIMEOUT result code.

- Delete the timeout parameter,  delete cited text below and fix Figure 10-12.

- We don’t care. Do nothing

There are no other instances of TIMEOUT that need to be addressed.

There is one instance of a locally-generated UNSPECIFIED_FAILURE on p159.

Proposed Resolution:

Revised.

At 159.08 and 20 delete the “ResultCode” parameter entirely.

At 159.30 delete “evaluates the result code and, if appropriate,”

| 182 | 406.00 |  | "STA contained in the AP" is occasionally used | Change such expressions to just "STA", since the general consensus seems to be that an AP is a STA rather than containing a STA |
| --- | --- | --- | --- | --- |

Discussion:

Disagree with the commenters assertion:  “general consensus seems to be that an AP is a STA rather than containing a STA”

Having discussed this with Mark Hamilton,  I am persuaded by his argument that an AP includes a STA,  and may include other architectural entities such as an interface to the DS.  Figure 4-6 illustrates this,  inasmuch as the AP box containing STA3 has STA3 as a part of the box,  rather than filling it entirely.

Proposed Resolution:

Rejected.  An AP contains a STA, and it may also contain other 802.11 architectural entities,  such as a portal.

| 226 | 1504.00 |  | The DSSS PHY and the HR/DSSS PHY are basically twins | Merge the DSSS PHY and the HR/DSSS PHY |
| --- | --- | --- | --- | --- |

Discussion:

This is a lot of work,  and to what benefit?

Proposed Resolution:

Rejected.  The commenter does not indicate specific changes that would satisfy the comment.

| 32 | 1504.00 | 16 | MAC protection for 11b-only devices is pure poison | Deprecate 1/2/5.5/11-only devices. WE can continue ot allow 1/2/5.5/11 as a PHY (for range/low PAPR) but only if coupled to an 11a/g PHY. |
| --- | --- | --- | --- | --- |

Discussion:

I understand the commenter’s concern.   Question is whether we need to do anything about it.

Proposed Resolution:

Rejected.  An AP can avoid the “pure poison” by excluding these rates from the Basic Rate Set and by avoiding the use of protection mechanisms for non-ERP receivers.

| 67 | 1902.00 | C.3 | dot11MitigationRequirement is defined here, but it is not referenced anywhere in the standard outside Annex C. This was the way this was added in 802.11h. I would assume this is related to the mitigation mechanism discussed in the context of TPC, but it is unclear whether dot11MitigtationRequirement is really needed or appropriate as a single value (wouldn't it be possible for there to be per-frequency rules that may differ between regulatory domains or even within one?). | Remove dot11MitigationRequirement. |
| --- | --- | --- | --- | --- |

Discussion:

Agree that there is no normative use of this variable.   There is text referring to “mitigation requirement”,  but this appears to be used as a general term,  not the value of  a MIB variable.

Further, mitigation requirements are a function of regulation.  They must be known and followed by the STAs whose behaviour determines compliance to a regulatory requirement, in order to achieve approval.

Proposed Resolution:

Accepted.

| 113 | 2134.00 | C.3 | aPSDUMaxLength is not relevant to fragmentation (because you errors in one MPDU in an A-MPDU do not to a first approximation cause problems for the other MPDUs in that A-MPDU (cf. A-MSDU)) | Delete the references to aPSDUMaxLength from the description of dot11FragmentationThreshold |
| --- | --- | --- | --- | --- |

Discussion:

There is no current PHY where an aPSDUMaxLength constraint has an effect.

The question is whether we are ever likely to create a PHY that could not transport the largest defined Data MPDU length (i.e,  aPSDUMaxLength < aMPDUMaxLength).  I think this very unlikely.

If we did invent such a PHY,  it would be the responsibility of that project to modify the language at the cited location to suit the new circumstances.

Proposed resolution:

Accepted.

| 1 | 2307.00 | G | Frame exchange sequences do not show Mesh operation | Add mesh frame exchange sequences where not already adequately covered. |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.  The commenter does not indicate specific changes that would satisfy the comment.

| 188 | 2307.00 | G | Annex G does not cover all HT sequences | Extend Annex G to cover all HT sequences. Otherwise deprecate it, but make sure it's not the only place where some rule or other is expressed |
| --- | --- | --- | --- | --- |

Discussion:

The 802.11n sequence are largely complete.   AFAIK, it is only the aspect of Beamforming training that is not fully covered.   And it may be that this is indeed covered, although I haven’t done the work to show this.

We have normative references to Annex G.  In REVmb days, Annex G was for a time informative.  Then we discovered the normative references, and so we made it normative.   Any change to deprecate Annex G would necessarily require that it be made informative and any normative references removed.

Proposed Resolution:

Rejected.  The commenter does not indicate specific changes that would satisfy the comment.