IEEE P802.11
Wireless LANs

| CID 857-Neighbor-Report-Normative-Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-10-22 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Santosh Pandey | Cisco Systems | 170 W Tasman Dr, San Jose, CA, 95134, USA | +1-408-8535990 | sanpande@cisco.com |
| Peter Ecclesine | Cisco Systems | 170 W Tasman Dr, San Jose, CA, 95134, USA |  | petere@cisco.com |
| Dave Stephenson | Cisco Systems | 170 W Tasman Dr, San Jose, CA, 95134, USA | +1 408-5277991 | daves@cisco.com |
| Andrew Myles | Cisco Systems | 201 Pacific Highway, St Leonards, NSW, Australia |  | amyles@cisco.com |
| Brian Hart | Cisco Systems | 170 W Tasman Dr, San Jose, CA, 95134, USA |  | brianh@cisco.com |

Abstract

Submission based on 11-12-1054-01-00ai-FILS-Reduced-Neighbor-Report.ppt and 11-12-1098-05 FILS-Reduced-Neighbor-Report-Normative-Text for 802.11 TGai.

Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGaf Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGaf Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGaf Editor: Editing instructions preceded by “TGaf Editor” are instructions to the TGaf editor to modify existing material in the TGaf draft.  As a result of adopting the changes, the TGaf editor will execute the instructions rather than copy them to the TGaf Draft.

LB 189 CID 857:

| 857 |  | AP discovery time depends on the number of bands that a STA supports as well the number of candidate bandwidths supported on these bands. If all the potential channels (50) and bandwidths (3) are supported, the UE may have to scan each of these in turn before associating to the AP- resulting scan time may be order of 10 seconds. This discovery time seems excessive. | Introduce an improved AP discovery mechanism. |
| --- | --- | --- | --- |

Discussion:

LB 178 CID 857 says that since a great many TV channels are in theory possible for white space use, an improved AP discovery mechanism is needed. Submission 11-12/1054r1 discusses the problem and one solution for the Fast Initial Link Setup situation that can be applied to Operation in TV White Spaces.

Propose Accepted for CID 857, per discussion in 11-12/1234r0. We adopt the Reduced Neighbor Report text previously included in TGai’s initial draft.

3. Definitions, acronyms, and abbreviations

3.1 Definitions

TGaf Editor: Insert the following definition retaining alphabetic order:

beacon channel: The channel on which the beacon of a BSS is transmitted.

8. Frame formats

  - Format of individual frame types

8.3.3 	Management frames

      - Beacon frame format

TGaf Editor: Insert the following entry text in Table 8-20: (11k)

Change the row of Order 63 in Table 8-20 Beacon frame body as follows:

| Table 8-20 -- Beacon frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| 63(11n) | Channel Switch Wrapper element | The Channel Switch Wrapper element is optionally present if dot11VHTOptionImplemented is true and at least one of a Channel Switch Announcement element or an Extended Channel Switch Announcement element is also present in the Beacon frame and the Channel Switch Wrapper element contains at least one subelement. The Channel Switch Wrapper element is optionally present if dot11TVHTOptionImplemented is true and at least one of a Channel Switch Announcement element or an Extended Channel Switch Announcement element is also present in the Beacon frame and the Channel Switch Wrapper element contains at least one subelement. |

TGaf Editor: insert the following entry text in Table 8-20: (11k)

| Table 8-20 -- Beacon frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| <ANA>(11n) | Reduced Neighbor Report | The Reduced Neighbor Report element is optionally present if dot11TVHTOptionImplemented (#1005)is(#1217) true(#1535). |

      - Probe Response frame format

TGaf Editor: Insert the following entry text in Table 8-27: (11k)

Change the row of Order 63 in Table 8-27 Probe Response frame body as follows:

| Table 8-27 – Probe Response frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| 63(11n) | Channel Switch Wrapper element | The Channel Switch Wrapper element is optionally present if dot11VHTOptionImplemented is true and at least one of a Channel Switch Announcement element or an Extended Channel Switch Announcement element is also present in the Beacon frame and the Channel Switch Wrapper element contains at least one subelement. The Channel Switch Wrapper element is optionally present if dot11TVHTOptionImplemented is true and at least one of a Channel Switch Announcement element or an Extended Channel Switch Announcement element is also present in the Beacon frame and the Channel Switch Wrapper element contains at least one subelement. |

TGaf Editor: insert the following entry text in Table 8-27: (11k)

| Table 8-27 - Probe Response frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| <ANA>(11n) | Reduced Neighbor Report | The Reduced Neighbor Report element is optionally present if dot11TVHTOptionImplemented (#1005)is(#1217) true(#1535). |

8.4.2 Information elements

8.4.2.1 General

TGaf Editor: insert the following entry text in Table 8-54: (11k) (#28)

| Table 8-54 - Element IDsinput.doc |  |  |  |
| --- | --- | --- | --- |
| (#1684)Element | Element ID | Length (in octets) | Extensible |
| Reduced Neighbor Report (see 8.4.2.af1) | <ANA> | 8 to 257 |  |

TGaf Editor: insert text as follows before 8.4.2.172 TVHT Operation element:

8.4.2.af1 Reduced Neighbor Report element(11n)

The Reduced Neighbor Report element contains channel and other information related to neighbor APs. The format of the Reduced Neighbor Report element is shown in Figure af1.

|  | Element ID | Length | Neighbor AP Information field #1 | Neighbor AP Information field #2 (optional) | … | Neighbor AP Information field #n (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | Variable | variable | variable | variable |
|  | Figure af1 - Reduced Neighbor Report element format |  |  |  |  |  |

The Element ID field is equal to the Reduced Neighbor Report value in Table 8-54.

The value of the Length field is between 5 and 255.

The format of Neighbor AP Information field is described in 8.4.2.af1.1.

8.4.2.af1.1 Neighbor AP Information field (11n)

The Neighbor AP Information field specifies TBTT and other information related to a group of neighbor APs on one channel. See Figure af2.

|  | TBTT Information Header | Operating Class |  | Channel Number | TBTT Information field #1 | TBTT Information field #2 (optional) | … | TBTT Information field #n (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 1 |  | 1 | Variable | variable |  | variable |
|  |  |  | Figure af2 - Neighbor AP Information field format |  |  |  |  |  |

The format of TBTT Information Header subfield is defined in Figure af3.

|  | TBTT Information Field Type | Reserved | TBTT Information Count |  | TBTT Information Length |
| --- | --- | --- | --- | --- | --- |
| Bits: | B0 B1 B2 B3 B7 |  |  | B8 B15 |  |
|  | Figure af3 - TBTT Information Header subfield |  |  |  |  |

The TBTT Information Field Type subfield is 2 bits in length and defines the structure of the TBTT Information field. Its value is 0. Values 1, 2, and 3 are reserved.

The TBTT Information Count subfield is 4 bits in length and contains the number of TBTT Information fields that are included in the Neighbor AP Information field, minus one. A value of 0 indicates one TBTT Information field is present.

The TBTT Information Length subfield is 1 octet in length and contains the length in octets of the TBTT Information field that is included in the Neighbor AP Information field.

Operating Class is 1 octet in length and indicates the band and bandwidth of the beacon channel of the APs in this Neighbor AP Information field. Valid values of Operating Class are shown in Table E-4.

Channel Number is 1 octet in length and indicates the last known beacon channel of the APs in this Neighbor AP Information field. Channel Number is defined within an Operating Class as shown in Table E-4.

The format of TBTT Information field is shown in Figure af4.

|  | TBTT Offset in TUs | Optional Subelements |
| --- | --- | --- |
| Octets: | 1 | Variable |
|  | Figure af4 - TBTT Information field |  |

The TBTT Offset in TUs subfield is one octet in length and indicates the offset in TUs, rounded down to nearest TU, to the next TBTT of an AP from the immediately prior TBTT of the AP that transmits this element. The value 254 is used to indicate an offset of 254 TUs or higher. The value 255 is used to indicate an unknown offset value.

The Optional Subelements field format contains zero or more subelements, each consisting of a 1-octet Subelement ID field, a 1-octet Length field, and a variable-length Data field, as shown in Figure 8-402. Any optional subelements are ordered by nondecreasing subelement ID.

10. MLME

10.42 Operation under the control of a geolocation database

TGaf Editor: insert text as follows before 10.42.8 White Space Map(WSM):

10.42.7.1 Reduced Neighbor Report

In Beacon and Probe Response frames, a reduced neighbor report is optionally sent by an AP with dot11TVHTOptionImplemented  true. A reduced neighbor report contains information on neighbor APs. A reduced neighbor report may not be exhaustive either by choice, or due to the fact that there may be neighbor APs not known to the AP.

The Reduced Neighbor Report element contains a list of operating classes and channels along with TBTT information for the reported neighbor APs on each operating class and channel. A Reduced Neighbor Report element only includes channels that are consistent with the Country element in the frame in which the Reduced Neighbor Report element appears. The reduced neighbor report contents may be derived from the NeighborListSet parameter of the MLME-NEIGHBORREPRESP.request primitive. The contents of the reduced neighbor report may also be configured or obtained by other means beyond the scope of this standard.

A serving AP shall include a value less than 255 in TBTT Offset in TUs if it is able to guarantee an accumulated error of 1.5 TU or better on the TBTT Offset in TUs subfield.

A STA receiving a Reduced Neighbor Report may use the report to schedule passive scanning for faster AP discovery. The scheduling process is beyond the scope of this standard. A STA receiving a Reduced Neighbor Report element with an unknown subelement identifier shall ignore the unknown subelement and continue to process the remaining subelements.