IEEE P802.11
Wireless LANs

| CIDs 46,47,48 Regarding Fine Timing Measurement |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-11-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Carlos Aldana | Qualcomm | 1700 Technology Drive, San Jose, CA, 95100 | +1(408)652-1251 | caldana@qca.qualcomm.com |
| Jouni Malinen | Qualcomm |  |  | jouni@qca.qualcomm.com |
| Youhan Kim | Qualcomm | 1700 Technology Drive, San Jose, CA 95110 | +1(408)830-5835 | youhank@qca.qualcomm.com |
| Adrian Stephens | Intel Corporation |  |  | adrian.p.stephens@intel.com |
| Brian Hart | Cisco Systems |  |  | brianh@cisco.com |
| Vinko Erceg | Broadcom |  |  | verceg@broadcom.com |

Abstract

Resolution of CIDs 46,47,48 regarding Fine Timing Measurement

The changes marked in this document are based on 802.11-2012 Specification.

Revision Notes:

Rev0 : Initial resolution proposal

Rev1 : Added motivation section, removed section numbers and replaced with variables instead, and modified Figure x-y.

Rev2 :  Added text to sections that have dot11MgmtOptionFineTimingXXX.  Removed unnecessary “” in Rev1. Added section 4.3.13.X

Proposed resolution: Agree in Principle

Motivation :

- The current resolution of 10ns for 11v is approriate for signals that have CH_BANDWIDTH < 100 MHz.  The signals of both 802.11ac (160 MHz) and 802.11ad (>1.8 GHz) have much greater requirements.  To be future proof, we recommend 0.1 ns.

- In order to accommodate the smaller resolution, we propose to increase the counter size from 4 bytes to 6 bytes so that they wrap around every 7.8 hours.  If the counter is left at 4 bytes, the wrap around value is 0.43 seconds, which may be too small for certain applications.

- In order to accommodate the smaller resolution, we propose to increase the Max ToA/ToD Error from 1 byte to 2 bytes.  If the counter is left at 1 byte, the max ToA/ToD error becomes only 25.5 ns.  Increasing it to 2 bytes allows for 6.5535us of error.  Previous max error value was 2.55us.

- Current Timing Measurement Frames and Timing Measurement Request Frames are class 3 frames that require both authentication and association.  This is unnecessary. Add new public action frames that have same functionality as exisiting 11v frames, and since they are public action they are class 1 frames.  Class 1 frames do not require either authentication or association.

- The current wording in section 8.5.15.3 (Timing Measurement Frame Format) has a multi-antenna ambiguity.  Remove this ambiguity in the new fine timing measurement frames and require that all transmissions occur from a single RF chain.

Note : All the text in this submission, except modifications in pages 14-17  in this submission is to be an insertion to the existing draft.   I have shown in black existing draft text from the matching .11v feature, and in red differences to the .11v mechanism.   This is to help the reader of this submission understand the scope of the changes.

Instruction to Editor: Please make the following changes:

4.3.13.X Fine Timing measurement

Fine Timing Measurement frames allow a receiving STA to accurately measure the Round Trip Time (RTT) relative to the sending STA. With the regular transfer of Fine Timing Measurement frames from one STA to another, it is possible for the recipient STA to track changes in the RTT with respect to the sending STA over time.

Add two new public action frames to Table 8-210 in section 8.5.8.1 called Fine Timing Measurement Request frame format and Fine Timing Measurement frame format.

Insert the following new section

6.3.XFine Timing measurement

6.3.X.1 General

The following set of primitives supports exchange of fine timing measurement information from one SME to another. The informative diagram in Fine Timing Measurement primitves and timestamps capture depicts various points in time that are of interest to the fine timing measurement procedure.  See figure x-y.

|  | Figure x-y -- Fine Timing Measurement primitives and timestamps capture |
| --- | --- |

NOTE 1—In Error: Reference source not found, t1 and t3 correspond to the point in time at which the start of the preamble for the transmitted frame appears at the transmit antenna port. An implementation may capture a timestamp during the transmit processing earlier or later than the point at which it actually occurs and offset the value to compensate for the time difference.

NOTE 2—In Error: Reference source not found, t2 and t4 correspond to the point in time at which the start of the preamble for the incoming frame arrives at the receive antenna port. Because time is needed to detect the frame and synchronize with its logical structure, an implementation determines when the start of the preamble for the incoming frame arrived at the receive antenna port by capturing a timestamp some time after it occurred and compensating for the delay by subtracting an offset from the captured value.

6.3.X.2 MLME-FINETIMINGMSMT.request

6.3.X.2.1 Function

This primitive requests the transmission of Fine Timing Measurement frame to a peer entity.

6.3.X.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-FINETIMINGMSMT.request(

Peer MAC Address,
Dialog Token,
Follow Up Dialog Token,
t1,
Max t1 Error,
t4,
Max t4 Error,
VendorSpecific
)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual addressed MAC Address | The address of the peer MAC entity to which the Fine Timing Measurement frame is sent. |
| Dialog Token | Integer | 0–255 | The dialog token to identify the Fine Timing Measurement transaction. A value of 0 indicates to stop transaction. |
| Follow Up Dialog Token | Integer | 0–255 | The dialog token of a Fine Timing Measurement frame which the current frame follows. See 10.2X (Fine Timing measurement procedure). |
| t1 | Integer | 0 – (248–1) | Set to the value of t1 t1 (see Error: Reference source not found) expressed in 0.1 ns units. |
| Max t1 Error | Integer | 0–65535 | Maximum error in the t1 value expressed in 0.1 ns units; see 8.5.8.Y (Fine Timing Measurement frame format). A value of 0 indicates that the upper bound on the error is unknown. A value of 65535 indicates that the upper bound on the error is greater than or equal to 6.5535 µs. |
| t4 | Integer | 0 – (248–1) | Set to the value of t4 (see Error: Reference source not found) expressed in 0.1 ns units. |
| Max t4 Error | Integer | 0–65535 | Maximum error in t4 value expressed in 0.1 ns units. A value of 0 indicates that the upper bound on the error is unknown. A value of 65535 indicates that the upper bound on the error is greater than or equal to 6.5535 µs. |
| VendorSpecific | A set of information elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.X.2.3 When generated

This primitive is generated by the SME to request that a Fine Timing Measurement frame be sent to a peer entity.

6.3.X.2.4 Effect of receipt

On receipt of this primitive, the MLME constructs a Fine Timing Measurement frame with the specified parameters. This frame is then scheduled for transmission.

      - MLME-FINETIMINGMSMT.confirm

6.3.X.3.1 Function

This primitive indicates that a Fine Timing Measurement frame has been successfully received by the peer STA to which it was sent.

6.3.X.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-FINETIMINGMSMT.confirm(

Peer MAC Address,

Dialog Token,

t1,

Max t1 Error,

t4,

Max t4 Error,

Vendor Specific

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual addressed MAC Address | The address of the peer MAC entity to which acknowledges the receipt of the Fine Timing Measurement frame. |
| Dialog Token | Integer | 0–255 | The dialog token to identify the Fine Timing Measurement transaction. A value of 0 indicates to stop transaction. |
| t1 | 48-bit unsigned Integer | 0 – (248–1) | Set to the value of t1 (see Error: Reference source not found) expressed in 0.1 ns units. |
| Max t1 Error | Integer | 0–65535 | Maximum error in the t1 value expressed in 0.1 ns units. A value of 0 indicates that the upper bound on the error is unknown. A value of 65535 indicates that the upper bound on the error is greater than or equal to 6.5535 µs. |
| t4 | 48-bit unsigned Integer | 0 – (248–1) | Set to the value of t4 (see Error: Reference source not found) expressed in 0.1 ns units. |
| Max t4 Error | Integer | 0–65535 | Maximum error in t4 value expressed in 0.1 ns units. A value of 0 indicates that the upper bound on the error is unknown. A value of 65535 indicates that the upper bound on the error is greater than or equal to 6.5535 µs. |
| VendorSpecific | A set of information elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.X.3.3 When generated

This primitive is generated by the MLME when an ACK corresponding to the Fine Timing Measurement frame is received from the peer STA.

6.3.X.3.4 Effect of receipt

On receipt of this primitive, the SME uses the information contained within the notification.

6.3.X.4 MLME-FINETIMINGMSMT.indication

6.3.X.4.1 Function

This primitive indicates that a Fine Timing Measurement frame has been received and the corresponding ACK has been transmitted.

6.3.X.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-FINETIMINGMSMT.indication(

Peer MAC Address,

Dialog Token,

Follow Up Dialog Token,

t1,

Max t1 Error,

t4,

Max t4 Error,

t2,

Max t2 Error,

t3,

Max t3 Error,

VendorSpecific

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual addressed MAC Address | The address of the peer MAC entity from which the Fine Timing Measurement frame was sent. |
| Dialog Token | Integer | 0–255 | The dialog token to identify the Fine Timing Measurement transaction. A value of 0 indicates to stop transaction. |
| Follow Up Dialog Token | Integer | 1–255 | The dialog token of a Fine Timing Measurement frame which the current frame follows. See 10.2X (Fine Timing measurement procedure). |
| t1 | 48-bit unsigned integer | 0 – (248–1) | Set to the value of t1 (see Error: Reference source not found) expressed in 0.1 ns units. |
| Max t1 Error | Integer | 0–65535 | Maximum error in the t1 value expressed in 0.1 ns units. A value of 0 indicates that the upper bound on the error is unknown. A value of 65535 indicates that the upper bound on the error is greater than or equal to 6.5535 µs. |
| t4 | 48-bit unsigned integer | 0 – (248–1) | Set to the value of t4 (see Error: Reference source not found) expressed in 0.1 ns units. |
| Max t4 Error | Integer | 0–65535 | Maximum error in t4 value expressed in 0.1 ns units. A value of 0 indicates that the upper bound on the error is unknown. A value of 65535 indicates that the upper bound on the error is greater than or equal to 6.5535 µs. |
| t2 | 48-bit unsigned Integer | 0 – (248–1) | Set to the value of t2 (see Error: Reference source not found) expressed in 0.1 ns units. |
| Max t2 Error | Integer | 0–65535 | Maximum error in t2 value expressed in 0.1 ns units. A value of 0 indicates that the upper bound on the error is unknown. A value of 65535 indicates that the upper bound on the error is greater than or equal to 6.5535 µs. |
| t3 | 48-bit unsigned integer | 0 – (248–1) | Set to the value of t3 (see Error: Reference source not found) expressed in 0.1 ns units. |
| Max t3 Error | Integer | 0–65535 | Maximum error in t3 value expressed in 0.1 ns units. A value of 0 indicates that the upper bound on the error is unknown. A value of 65535 indicates that the upper bound on the error is greater than or equal to 6.5535 µs. |
| VendorSpecific | A set of information elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.X.4.3 When generated

This primitive is generated by the MLME when a valid Fine Timing Measurement frame is received.

6.3.X.4.4 Effect of receipt

On receipt of this primitive, the SME uses the information contained within the notification.

Insert a new section called 6.3.Y

6.3.Y FineTiming Measurement Request

6.3.Y.1 General

The following set of primitives supports triggering a Fine Timing Measurement procedure or stopping an ongoing Fine Timing Measurement procedure.

6.3.Y.2 MLME-FINETIMINGMSMTRQ.request

6.3.Y.2.1 Function

This primitive requests the transmission of a Fine Timing Measurement Request frame to a peer entity.

6.3.Y.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-FINETIMINGMSMTRQ.request(

Peer MAC Address,
Trigger,

Vendor Specific
)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the Fine Timing Measurement Request frame is sent. |
| Trigger | Integer | 0–1 | The trigger to identify the action. |
| Vendor Specific | A set of information Elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.Y.2.3 When generated

This primitive is generated by the SME to request that a Fine Timing Measurement Request frame be sent to a peer entity.

6.3.Y.2.4 Effect of receipt

On receipt of this primitive, the MLME constructs a Fine Timing Measurement Request frame with the specified parameters. This frame is then scheduled for transmission.

6.3.Y.3 MLME-FINETIMINGMSMTRQ.indication

6.3.Y.3.1 Function

This primitive indicates that a Fine Timing Measurement Request frame has been received and the corresponding ACK has been transmitted.

6.3.Y.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-FINETIMINGMSMTRQ.indication(

Peer MAC Address,
Trigger,

Vendor Specific
)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the Fine Timing Measurement Request frame is sent. |
| Trigger | Integer | 0–1 | The trigger to identify the action. |
| Vendor Specific | A set of information Elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.Y.3.3 When generated

This primitive is generated by the MLME when a valid Fine Timing Measurement Request frame is received.

6.3.Y.3.4  Effect of receipt

On receipt of this primitive, the SME uses the information contained within the notification.

Add a new entry to Table 8-103 (Capabilities Field) in Section 8.4.2.29 (Extended Capabilities element) called Fine Timing Measurement and have a dot11MgmtOptionFineTimingMsmtActivated variable associated with it, as in below

| 4x | Fine Timing Measurement | The STA sets the Fine Timing Measurement field to 1 when dot11MgmtOptionFineTimingMsmtActivated is true, and sets it to 0 otherwise. See 10.2X (Fine Timing measurement procedure). |
| --- | --- | --- |

Insert a new section within 8.5.8

8.5.8.X Fine Timing Measurement Request frame format

The format of the Fine Timing Measurement Request frame is shown in Figure 8-520.

|  | Category |  | Action | Trigger |
| --- | --- | --- | --- | --- |
| Octets: | 1 |  | 1 | 1 |

The category field is set to the value for Public, specified in Table 8-38.

The Public Action field is set to indicate a Fine Timing Measurement Request frame, as defined in Table 8-210.

The Trigger field set to the value 1 indicates that the receiving STA requests a Fine Timing Measurement procedure at the sending STA as defined in 10.2X (Fine Timing measurement procedure).  The trigger field set to the value 0 indicates that the receiving STA requests that the sending STA stops sending Fine Timing Measurement frames. Trigger field values 2–255 are reserved.

8.5.8.Y Fine Timing Measurement frame format

The Fine Timing Measurement frame is used to support the timing measurement procedure described in 10.2X (Fine Timing measurement procedure). The format of the Fine Timing Measurement frame is shown in Fig 8-new123 Fine Timing Measurement frame format.

|  | Category | Action | Dialog Token | Follow Up Dialog Token | TOD |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 6 |
|  | TOA | Max TOD Error | Max TOA Error |  |  |
| Octets: | 6 | 2 | 2 |  |  |

Fig 8-new123 Fine Timing Measurement frame format

The category field is set to the value for Public, specified in Table 8-38.

The Public Action field is set to indicate a Fine Timing Measurement frame, as defined in Table 8-210.

The Dialog Token field is a nonzero value chosen by the sending STA to identify the Fine Timing Measurement frame as the first of a pair, with the second or follow-up Fine Timing Measurement frame to be sent later. The Dialog Token field is set to 0 to indicate that the Fine Timing Measurement frame will not be followed by a subsequent follow-up Fine Timing Measurement frame.

The Follow Up Dialog Token is the nonzero value of the Dialog Token field of the previously transmitted Fine Timing Measurement frame to indicate that it is the follow up Fine Timing Measurement frame and that the TOD, TOA, Max TOD Error and Max TOA Error fields contain the values of the timestamps captured with the first Fine Timing Measurement frame of the pair. The Follow Up Dialog Token is 0 to indicate that the Fine Timing Measurement frame is not a follow up to a previously transmitted Fine Timing Measurement frame. The value 0 in this field also indicates that the TOD, TOA, Max TOD Error, and Max TOA Error fields are reserved. See 10.2X  (Fine Timing measurement procedure).

The TOD, TOA, Max TOD Error, and Max TOA Error fields are expressed in units of 0.1 ns.

The TOD field contains a timestamp that represents the time at which the start of the preamble of the previously transmitted Fine Timing Measurement frame appeared at the transmit antenna port.

The TOA field contains a timestamp that represents the time at which the start of the preamble of the ACK to the previously transmitted Fine Timing Measurement frame arrived at the receive antenna port.

NOTE—The values specified in the TOD and TOA fields are described in 6.3.X (Fine Timing Measurement).

The Max TOD Error field contains an upper bound for the error in the value specified in the TOD field. For instance, a value of 2 in the Max TOD Error field indicates that the value in the TOD field has a maximum error of ± 0.2 ns.

The Max TOA Error field contains an upper bound for the error in the value specified in the TOA field. For instance, a value of 2 in the Max TOA Error field indicates that the value in the TOA field has a maximum error of ± 0.2 ns.

A value of 0 for the Max TOD Error or the Max TOA Error field indicates that the upper bound on the error in the corresponding TOD or TOA value is unknown. A value of 65535 indicates that the upper bound on the error is greater than or equal to 6.5535 us.

Insert the following new section:

10.2X Fine Timing measurement procedure

Implementation of Fine Timing Measurement is optional for a WNM STA. A STA that has a value of true for dot11MgmtOptionFineTimingMsmtImplemented is defined as a STA that supports fine timing measurement. A STA for which dot11MgmtOptionFineTimingMsmtImplemented is true shall set the Fine Timing Measurement field of the Extended Capabilities element to 1.

If dot11MgmtOptionFineTimingMsmtActivated is true, the Fine Timing Measurement field in the Extended Capabilities element shall be set to 1 and the STA supports the fine timing measurement procedure. If dot11MgmtOptionFineTimingMsmtActivated is false the STA shall set the Fine Timing Measurement field in the Extended Capabilities element to 0 and STA does not support the fine timing measurement procedure. The behavior of a STA that does not support the fine timing measurement procedure is defined in Section 9.24.4.

A STA that supports the fine timing measurement procedure may transmit a Fine Timing Measurement Request frame to a peer STA to request it to initiate or to stop an ongoing Fine Timing Measurement procedure, depending on the value of the Trigger field in the request frame.

STA that supports the fine timing measurement procedure may transmit Fine Timing Measurement frames addressed to a peer STA that also supports the fine timing measurement procedure.

Both the sending STA and receiving STA shall transmit out of a single RF chain.

A sending STA transmits Fine Timing Measurement frames in chained pairs. The first Fine Timing Measurement frame of a pair contains a nonzero Dialog Token. The follow up Fine Timing Measurement frame contains a Follow Up Dialog Token set to the value of the Dialog Token in the first frame of the pair. With the first Fine Timing Measurement frame, both STAs capture timestamps. The sending STA captures the time at which the Fine Timing Measurement frame is transmitted (t1). The receiving STA captures the time at which the Fine Timing Measurement frame arrives (t2) and the time at which the ACK response is transmitted (t3). The sending STA captures the time at which the ACK arrives (t4). See Figure x-y (Fine Timing measurement primitives and timestamps capture) in 6.3.x(Fine Timing measurement). In the follow up Fine Timing Measurement frame, the sending STA transfers the timestamp values it captured (t1 and t4) to the receiving STA.

The ACK frame should have the same CH_BANDWIDTH as the preceding action frame.

NOTE—A Fine Timing Measurement frame can contain nonzero values in both the Dialog Token and Follow Up Dialog Token fields, meaning that the Action frame contains follow up information from a previous measurement, and new Timestamp values are captured to be sent in a future follow up Fine Timing Measurement frame.

The round trip time (RTT) can be computed via

- [(t4 – t1) -(t3 – t2)]

If the ACK for a transmitted Fine Timing Measurement frame is not received, the sending STA may retransmit the frame. The sending STA shall capture a new set of timestamps for the retransmitted frame and its ACK.

There are 2 ways to stop this frame exchange:

- The Sending STA can send a Fine Timing Measurement Frame with Dialog Token set to 0.

- The Receiving STA can send a Fine Timing Measurement Request Frame with Trigger set to 0.

On receiving a Fine Timing Measurement frame with a Dialog Token for which timestamps have previously been captured, the receiving STA shall discard previously captured timestamps and capture a new set of timestamps.

In Section 6.5.4.2, make the following changes

The primitive provides the following parameters:

PLME-CHARACTERISTICS.confirm(

aSlotTime,

aSIFSTime,

aSignalExtension,

aCCATime,

aPHY-RX-START-Delay,

aRxTxTurnaroundTime,

aTxPLCPDelay,

aRxPLCPDelay,

aRxTxSwitchTime,

aTxRampOnTime,

aTxRampOffTime,

aTxRFDelay,

aRxRFDelay,

aAirPropagationTime,

aMACProcessingDelay,

aPreambleLength,

aRIFSTime,

aSymbolLength,

aSTFOneLength,

aSTFTwoLength,

aLTFOneLength,

aLTFTwoLength,

aPLCPHeaderLength,

aPLCPSigTwoLength,

aPLCPServiceLength,

aPLCPConvolutionalTailLength,

aMPDUDurationFactor,

aMPDUMaxLength,

aPSDUMaxLength,

aPPDUMaxTime,

aIUSTime,

aDTT2UTTTime,

aCWmin,

aCWmax,

aMaxCSIMatricesReportDelay

aMaxTODError,

aMaxTOAError,

aTxPmdTxStartRFDelay,

aTxPmdTxStartRMS,

aMaxTODFineError,

aMaxTOAFineError

)

| aMaxTODFineError | Integer | An estimate of the maximum error (in 0.1 ns units) in the TX_START_OF_FRAME_FINE_OFFSET value in the PHY-TXSTART.confirm(TXSTATUS) primitive. The estimated maximum error includes any error due to implementation component and environmental (including temperature) variability. |
| --- | --- | --- |
| aMaxTOAFineError | Integer | An estimate of the maximum error (in 0.1 ns units) in the RX_START_OF_FRAME_FINE_OFFSET value in the PHY-RXSTART.indicate(RXVECTOR) primitive. The estimated maximum error includes any error due to implementation component and environmental (including temperature) variability. |

Also add row TX_START_OF_FRAME_FINE_OFFSET to section 7.3.5.6.4 as shown below.

In Sections 16.2.6, 16.2.7,17.2.5, 17.2.6,18.3.11,18.3.12,20.3.22, 20.3.23, Replace “if dot11MgmtOptionTimingMsmtActivated” with “if either dot11MgmtOptionTimingMsmtActivated or dot11MgmtOptionFine TimingMsmtActivated”

In those same sections, replace “the value of TX_START_OF_FRAME_OFFSET” with “the values of both TX_START_OF_FRAME_OFFSET and TX_START_OF_FRAME_FINE_OFFSET”

In those same sections, replace “RX_START_OF_FRAME_OFFSET” with “both RX_START_OF_FRAME_OFFSET and RX_START_OF_FRAME_FINE_OFFSET”

In Table 17-6, add 2 new rows called TX_START_OF_FRAME_FINE_OFFSET and RX_START_OF_FRAME_FINE_OFFSET, as in

| TX_START_OF_FRAME_FINE_OFFSET | TXSTATUS | 0 to 248– 1. An estimate of the offset (in 0.1 ns units) from the point in time at which the start of the preamble corresponding to the frame was transmitted at the transmit antenna port to the point in time at which this primitive is issued to the MAC. |
| --- | --- | --- |
| RX_START_OF_FRAME_FINE_OFFSET | RXVECTOR | 0 to 248– 1. An estimate of the offset (in 0.1 ns units) from the point in time at which the start of the preamble corresponding to the incoming frame arrived at the receive antenna port to the point in time at which this primitive is issued to the MAC. |

In Tables 18-3, 19-2, 20-4, add a new row TX_START_OF_FRAME_FINE_OFFSET, as above.

In Tables 20-1, 19-2, 20-4, add a new row TX_START_OF_FRAME_FINE_OFFSET, as above.

In Annex B, add an entry to section B.4.21

| WNM2X | Fine Timing Measurement | 10.2X (Fine Timing measurement procedure) | CF19:O | Yes □ No □ N/A □ |
| --- | --- | --- | --- | --- |
| WNM2X.1 | Fine Timing Measurement Request | 8.5.8.X (Fine Timing Measurement Request frame format) | WNM2X:M | Yes □ No □ N/A □ |
| WNM2X.2 | Fine Timing Measurement | 8.5.8.Y (Fine Timing Measurement frame format) | WNM2X:M | Yes □ No □ N/A □ |

In Annex C, add both dot11MgmtOptionFineTimingMsmtImplemented and dot11MgmtOptionFineTimingMsmtActivated,

Dot11WirelessMgmtOptionsEntry ::=

SEQUENCE {

dot11MgmtOptionLocationActivated	TruthValue,

dot11MgmtOptionFMSImplemented	TruthValue,

dot11MgmtOptionFMSActivated	TruthValue,

dot11MgmtOptionEventsActivated	TruthValue,

dot11MgmtOptionDiagnosticsActivated	TruthValue,

dot11MgmtOptionMultiBSSIDImplemented	TruthValue,

dot11MgmtOptionMultiBSSIDActivated	TruthValue,

dot11MgmtOptionTFSImplemented	TruthValue,

dot11MgmtOptionTFSActivated	TruthValue,

dot11MgmtOptionWNMSleepModeImplemented	TruthValue,

dot11MgmtOptionWNMSleepModeActivated	TruthValue,

dot11MgmtOptionTIMBroadcastImplemented	TruthValue,

dot11MgmtOptionTIMBroadcastActivated	TruthValue,

dot11MgmtOptionProxyARPImplemented	TruthValue,

dot11MgmtOptionProxyARPActivated	TruthValue,

dot11MgmtOptionBSSTransitionImplemented	TruthValue,

dot11MgmtOptionBSSTransitionActivated	TruthValue,

dot11MgmtOptionQoSTrafficCapabilityImplemented	TruthValue,

dot11MgmtOptionQoSTrafficCapabilityActivated	TruthValue,

dot11MgmtOptionACStationCountImplemented	TruthValue,

dot11MgmtOptionACStationCountActivated	TruthValue,

dot11MgmtOptionCoLocIntfReportingImplemented	TruthValue,

dot11MgmtOptionCoLocIntfReportingActivated	TruthValue,

dot11MgmtOptionMotionDetectionImplemented	TruthValue,

dot11MgmtOptionMotionDetectionActivated	TruthValue,

dot11MgmtOptionTODImplemented	TruthValue,

dot11MgmtOptionTODActivated	TruthValue,

dot11MgmtOptionTimingMsmtImplemented	TruthValue,

dot11MgmtOptionTimingMsmtActivated	TruthValue,

dot11MgmtOptionChannelUsageImplemented	TruthValue,

dot11MgmtOptionChannelUsageActivated	TruthValue,

dot11MgmtOptionTriggerSTAStatisticsActivated	TruthValue,

dot11MgmtOptionSSIDListImplemented	TruthValue,

dot11MgmtOptionSSIDListActivated	TruthValue,

dot11MgmtOptionMulticastDiagnosticsActivated	TruthValue,

dot11MgmtOptionLocationTrackingImplemented	TruthValue,

dot11MgmtOptionLocationTrackingActivated	TruthValue,

dot11MgmtOptionDMSImplemented	TruthValue,

dot11MgmtOptionDMSActivated	TruthValue,

dot11MgmtOptionUAPSDCoexistenceImplemented	TruthValue,

dot11MgmtOptionUAPSDCoexistenceActivated 	TruthValue,

dot11MgmtOptionWNMNotificationImplemented	TruthValue,

dot11MgmtOptionWNMNotificationActivated	TruthValue,

dot11MgmtOptionUTCTSFOffsetImplemented	TruthValue,

dot11MgmtOptionUTCTSFOffsetActivated	TruthValue,

dot11MgmtOptionFineTimingMsmtImplemented	TruthValue,

dot11MgmtOptionFineTimingMsmtActivated	TruthValue,

}

dot11MgmtOptionFineTimingMsmtImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the station implementation is capable of supporting Fine Timing Measurement capability when the dot11WirelessManagementImplemented is set to true."

::= { dot11WirelessMgmtOptionsEntry XX }

dot11MgmtOptionFineTimingMsmtActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME.

Changes take effect as soon as practical in the implementation.

This attribute, when true, indicates that the station capability for Fine Timing Measurement is enabled. False indicates the station has no Fine Timing Measurement capability or that the capability is present but is disabled."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry YY }