IEEE P802.11
Wireless LANs

| Omission-of-Probe-Request-Normative-Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-11-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Giwon Park | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-1879 | giwon.park@lge.ccom |
| Kiseon Ryu | LG Electronics | 10225 Willow Creek Rd, San Diego, CA, 92131, USA | +1 (858)-635-5209 | Kiseon.ryu@lge.com |
| HanGyu Cho | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-7902 | Hangyu.cho@lge.com |

Abstract

The submission provides normative text for ‘Omission of Probe Request’ as identified in subclause 6.4 of the SFD (11-12-0151-13-00ai-proposed-specification-framework-for-tgai) as:

6.2.9 Omission of Probe Request (11-12/0655r5)

STA may omit a probe request due to reception of, for example,

- Broadcast probe requests

- Broadcast probe responses

- Beacon

Current Probe Request omission procedure which was described in P802.11ai D01 was incomplete (The reason about incomplete description was mentioned in the “11-12-1267-00-00ai-Omission of Probe Request”) and needs to add the further explanation. Thus, we clarify the Probe Request omission operation which was described in P802.11ai D01.

This amendment’s baseline is P802.11ai D01

      - Active scanning

        - Introduction

        - Active scanning procedur of the scanning STA

        - Sending a probe request

Instructions to Editor: Append the following underlined text in this Clause as follows:

When an MLME receives an MLME-SCAN.request primitive with ScanType indicating an active scan, a STA may not transmit a Probe Request frame to a channel at which the STA has received:

- A broadcast addressed Probe Request frame to which the clause 10.1.4.3.5(Criteria to respond to probe request) allows at least the same responses as the information indicated in the received MLME-SCAN.request primitive.

- A broadcast addressed Probe Response or a Beacon frame containing at least the same information as indicated in the received MLME-SCAN.request primitive.

When a STA omitted its Probe Request transmission, STA performs the following procedure:

- Set a ProbeTimer to 0 and start the ProbeTimer. If PHY-CCA.indication (busy) primitive has not been detected before the ProbeTimer reaches MinChannelTime, then send its own Probe Request frame to a channel which the STA has received a broadcast addressed Probe Request frame, else process the received probe responses or Beacon.

- If neither Probe Response nor Beacon has been received until ProbeTimer reaches MaxChannelTime, the STA sends its own Probe Request frame.

Motion: To authorize the Editor to incorporate the text changes proposed in contribution 11-12-xxxx-00-00ai-Omission-of-Probe-request to the draft TGai Specification Document.

Yes: ____________; No: _________________; Abstain: _____________________

[Result of Motion]