IEEE P802.11
Wireless LANs

| Omission-of-Probe-Request-Normative-Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-11-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Giwon Park | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-1879 | giwon.park@lge.ccom |
| Kiseon Ryu | LG Electronics | 10225 Willow Creek Rd, San Diego, CA, 92131, USA | +1 (858)-635-5209 | kiseon.ryu@lge.com |
| HanGyu Cho | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-7902 | hangyu.cho@lge.com |

Abstract

The submission provides normative text for ‘Omission of Probe Request’ as identified in subclause 6.4 of the SFD (11-12-0151-13-00ai-proposed-specification-framework-for-tgai) as:

6.2.9 Omission of Probe Request (11-12/0655r5)

STA may omit a probe request due to reception of, for example,

- Broadcast probe requests

- Broadcast probe responses

- Beacon

Current Probe Request omission procedure which was described in P802.11ai D01 was incomplete (The reason about incomplete description was mentioned in the “11-12-1267-00-00ai-Omission of Probe Request”) and needs to add the further explanation. Thus, we clarify the Probe Request omission operation which was described in P802.11ai D01.

This amendment’s baseline is P802.11ai D01

      - Active scanning

        - Introduction

        - Active scanning procedur of the scanning STA

        - Sending a probe request

Instructions to Editor: Append the new paragraph to the end of the following existing text with the following text:

When an MLME receives an MLME-SCAN.request primitive with ScanType indicating an active scan, a STA may not transmit a Probe Request frame to a channel at which the STA has received:

- A broadcast addressed Probe Request frame to which the clause 10.1.4.3.5(Criteria to respond to probe request) allows at least the same responses as the information indicated in the received MLME-SCAN.request primitive.

- A broadcast addressed Probe Response or a Beacon frame containing at least the same information as indicated in the received MLME-SCAN.request primitive.

When a broadcast addressed Probe Request frame that allows at least the same responses as the information indicated in the received MLME-SCAN.request primitive is received, the STA set a ProbeTimer to 0 and start the ProbeTimer, performs the following procedure:

- If PHY-CCA.indication (busy) primitive has not been detected before the ProbeTimer reaches MinChannelTime, then send its own Probe Request frame.

- If neither Probe Response nor Beacon frame has been received from an AP perceived to be adequent for the STA, until the ProbeTimer reaches MaxChannelTime, the STA sends its own Probe Request frame.

Motion: To authorize the Editor to incorporate the text changes proposed in contribution “11-12-1269-03-00ai-tgai-spec-text-proposal-for-omission-of-probe-request” to the draft TGai Specification Document.

Yes: ____________; No: _________________; Abstain: _____________________

[Result of Motion]