IEEE P802.11
Wireless LANs

| AP-Redirect-Normative-Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-11-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Giwon Park | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-1879 | giwon.park@lge.ccom |
| Kiseon Ryu | LG Electronics | 10225 Willow Creek Rd, San Diego, CA, 92131, USA | +1 (858)-635-5209 | Kiseon.ryu@lge.com |
| HanGyu Cho | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-7902 | Hangyu.cho@lge.com |

Abstract

The submission provides normative text for band adjustment as identified in subclause 6.4 of the SFD (11-12-0151-13-00ai-proposed-specification-framework-for-tgai) as:

6.4	Band adjustmenet (11-12/1042r3)

If the AP’s BSS load on the operating channel is not enough to accommodate STAs, the AP may redirect the STAs to other channels by including the neighbor AP information in the Probe Response or Beacon.

This amendment’s baseline is P802.11ai D01

8. Frame formats

  - Format of individual frame types

8.3.3 	Management frames

      - Probe Response frame format

Instructions to Editor: Append the Clause 8.3.3.10 with the following text:

| Table 8-27 - Probe Response frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| <ANA>(11n) | AP Redirect Information | The AP Redirect Information element is optionally present if dot11FILSActivated (#1005)is(#1217) true(#1535). |

8.4.2 Information elements

8.4.2.1 General(#28)

Instructions to Editor: Append the following text in the Table 8-54:

| Table 8-54 - Element IDsinput.doc |  |  |  |
| --- | --- | --- | --- |
| (#1684)Element | Element ID | Length (in octets) | Extensible |
| AP Redirect Information(see 8.4.2.ai6) | <ANA> | 8 to 257 |  |

8.4.2.ai6 AP Redirect Information element(11n)

Instructions to Editor: Append the new Clause 8.4.2.ai6 with the following text:

The AP Redirect Information element contains channel and other information related to a neighbor AP. The format of the AP Redirect Information element is shown in Figure ai6.

|  | Element ID | Length | Redirect AP Information |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | Variable |

Figure ai6 – AP Redirect Information element format

The Element ID field is equal to the AP Redirect Information value in Table 8-54.

The value of the Length field is between 5 and 255.

The format of Redirect AP Information field is described in 8.4.2.ai6.1.

8.4.2.ai6.1 Redirect AP Information field (11n)

The Redirect AP Information field specifies BSSID and other information related to an APs on one channel. See Figure ai6.1.

|  | Redirect Type | Operating Class | Channel Number | BSSID (Optional) | TBTT Offset in TUs (Optional) | Optional Subelements (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | variable |
|  | Figure ai6.1 - Redirect AP Information |  |  |  |  |  |

The Redirect Type contains a 1-octet field indicating the type of redirection, and is one of the values in Table ?.

- Channel Redirection: In this type, AP may redirect the STAs from current channel to other channel by indicating the BSSID of a neighbor AP of other channel in AP Redirect Information element.

- Band Redirection:  In this type, AP may redirect the STA from current band to other band by including the other band information in AP Redirect Information element.

Table 8-ai4 – Redirect Type

| Redirect Type | Value |
| --- | --- |
| Channel Redirection | 0 |
| Band Redirection | 1 |
| Reserved | 2 - 255 |

Operating Class is 1 octet in length and indicates the band and bandwidth of the beacon channel of the AP in this Redirect AP Information field. Valid values of Operating Class are shown in Table E-4.

Channel Number is 1 octet in length and indicates the last known beacon channel of the AP in this Redirect AP Information field. Channel Number is defined within an Operating Class as shown in Table E-4.

The BSSID is the BSSID of the BSS being reported. Only when the Redirect Type is Channel Redirection, BSSID is included in this field.

The TBTT Offset in TUs subfield is one octet in length and indicates the offset in TUs, rounded down to nearest TU, to the next TBTT of an AP from the immediately prior TBTT of the AP that transmits this element. The value 254 is used to indicate an offset of 254 TUs or higher. The value 255 is used to indicate an unknown offset value.

The Optional Subelements field format contains zero or more subelements, each consisting of a 1-octet Subelement ID field, a 1-octet Length field, and a variable-length Data field, as shown in Figure 8-402. Any optional subelements are ordered by nondecreasing subelement ID.

10. MLME

10.25 Fast Initial Link Setup (FILS) Procedures

10.25.1 General

10.25.2 FILS Discovery Frame Generation and usage

Instructions to Editor: Append the new Clause 10.25.3 with the following text:

10.25.3 AP Redirection

Channel Redirection is the act of an AP redirecting STAs from the current channel to the other channel in order to increase throughput by reducing contention for the busy current channel. If an AP is physically collocated with dual band (e.g., 2.4GHz and 5GHz band), the AP may redirect the STA from the channel of the current band to the channel of other band.

If the BSS load on the operating channel is not enough to accommodate STAs, the AP may redirect the STAs to other channel by including the AP Redirect Information (see 8.4.2.ai6) in the unicast addressd Probe Response frame. Based on the AP Redirect Information element, a scanning STA is recommended to associate with the AP indicated in AP Redirect Information element.

10.26 Management frame Fast Initial Link Setup procedures

10.26.1 Reduced Neighbor Report

Instructions to Editor: Append the new Clause 10.26.2 with the following text:

10.26.2 AP Redirect Information

In the unicast addressed Probe Response frame, AP Redirect Information (see 8.4.2.ai6) is optionally sent by an AP with dot11FILSActivated set to true. AP Redirect Information contains neighbor AP information. The AP Redirect Information element may contain the BSSID and the TBTT offset of the AP in addition to the operating class and the channel number. The AP Redirect Information contents may be derived from the NeighborListSet parameter of the MLME-NEIGHBORREPRESP.request primitive.

Motion: To authorize the Editor to incorporate the text changes proposed in contribution 11-12-1270-01-00ai-tgai-spec-text-proposal-for-ap-redirection to the draft TGai Specification Document.

Yes: ____________; No: _________________; Abstain: _____________________

[Result of Motion]