IEEE P802.11
Wireless LANs

| AP-Redirect-Normative-Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-11-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Giwon Park | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-1879 | giwon.park@lge.ccom |
| Kiseon Ryu | LG Electronics | 10225 Willow Creek Rd, San Diego, CA, 92131, USA | +1 (858)-635-5209 | Kiseon.ryu@lge.com |
| HanGyu Cho | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-7902 | Hangyu.cho@lge.com |
| Jarkko Kneckt | Nokia | Otaniementie 19b, FIN-02210 Espoo, Finland |  | Jarkko.Kneckt@nokia.com |

Abstract

The submission provides normative text for band adjustment as identified in subclause 6.4 of the SFD (11-12-0151-13-00ai-proposed-specification-framework-for-tgai) as:

6.4	Band adjustmenet (11-12/1042r3)

If the AP’s BSS load on the operating channel is not enough to accommodate STAs, the AP may redirect the STAs to other channels by including the neighbor AP information in the Probe Response or Beacon.

In 802.11ai D0.1 the Neighbor Report element indicates the candidate APs and their operating channels. The Neighbor Report element is designed to report Neighbor Measurement results and it is not optimal for candidate AP indication:

- It is not possible to recommend scanning a specific channel. The Neighbor Report includes always AP information.

- It is not possible to redirect, i.e. indicate the best candidate AP

- The length of the element is not optimised

The AP Redirect Information element may be included to Beacon, Probe Response.

This amendment’s baseline is P802.11ai D0.1

8.3.3.2 Beacon frame format

Instructions to Editor: Append the Clause 8.3.3.2 with the following text:

Table 8-20 – Beacon frame bodyinput.doc

| Order | Information | Notes |
| --- | --- | --- |
| <ANA>(11n) | AP Redirect Information | The AP Redirect Information element is optionally present if dot11FILSActivated (#1005)is(#1217) true(#1535). |

      - Probe Response frame format

Instructions to Editor: Change the Clause 8.3.3.10 as shown:

| Table 8-27 - Probe Response frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| <ANA>(11n) | AP Redirect Information | The AP Redirect Information element is optionally present if dot11FILSActivated (#1005)is(#1217) true(#1535). |

8.4.2 Information elements

8.4.2.1 General(#28)

Instructions to Editor: Append the following text in the Table 8-54:

| Table 8-54 - Element IDsinput.doc |  |  |  |
| --- | --- | --- | --- |
| (#1684)Element | Element ID | Length (in octets) | Extensible |
| AP Redirect Information(see 8.4.2.ai6) | <ANA> | 5 to 257 |  |

8.4.2.ai6 AP Redirect Information element(11n)

Instructions to Editor: Append the new Clause 8.4.2.ai6 with the following text:

The AP Redirect Information element contains channel and other information related to a neighbor AP. The format of the AP Redirect Information element is shown in Figure ai6.

|  | Element ID | Length | Redirect Neighbor AP Information |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | Variable |

Figure ai6 – AP Redirect Information element format

The Element ID field is equal to the AP Redirect Information value in Table 8-54.

The value of the Length field is between 3 and 255.

The format of Redirect Neighbor AP Information field is described in 8.4.2.ai6.1.

8.4.2.ai6.1 Redirect Neighbor AP Information field (11n)

The Redirect Neighbor AP Information field specifies Redirect type, operating channel and AP related information to guide a STA in AP discovery and selection. . See Figure ai6.1.

|  | Redirect Type |  | Operating Class and Channel Number Presence Indicator | BSSID Presence Indicator | TBTT Offset Presence Indicator | Reserved | Operating Class | Channel Number | BSSID | TBTT Offset in TUs | Optional Subelements (optional) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits: | 1 |  | 1 | 1 | 1 | 4 | 0 or 8 | 0 or 8 | 0 or 48 | 0 or 8 | Variable |  |
|  |  | Figure ai6.1 - Redirect Neighbor AP Information field |  |  |  |  |  |  |  |  |  |  |

The Redirect Type is a 1-bits subfield indicating the type of redirect, and is one of the values in Table 8-ai4.

- AP Redirect: In this type, AP redirects the STAs to other BSS by indicating the information of the BSS of a neighbor AP and the channel in AP Redirect Information element.

- Band Redirect:  In this type, AP redirects the STA from current band to other band by including the other band information in AP Redirect Information element.

Table 8-ai4 – Redirect Type

| Redirect Type | Value |
| --- | --- |
| AP Redirect | 0 |
| Band Redirect | 1 |

The Operating Class and Channel Number Presence Indicator subfield indicates the presence of the Operating Class and Channel Number fields. A value 1 indicates that the Operating Class and Channel Number fields are present.

The BSSID Presence Indicator subfield indicates the presence of the BSSID field. A value 1 indicates that the BSSID field is present.

The TBTT Offset Presence Indicator subfield indicates the presence of the TBTT Offset in TUs field. A value 1 indicates that the TBTT Offset in TUs field is present.

Operating Class is 1 octet in length and indicates the band and bandwidth of the beacon channel of the AP. Valid values of Operating Class are shown in Table E-4. This subfield is present only if the Operating Class and Channel Number Presence Indicator is set to 1.

Channel Number is 1 octet in length and indicates the last known beacon channel of the AP. Channel Number is defined within an Operating Class as shown in Table E-4. This subfield is present only if the Operating Class and Channel Number Presence Indicator is set to 1.

The BSSID is the BSSID of the BSS being reported. This subfield is present only if the BSSID and TBTT Offset Presence Indicator is set to 1.

The TBTT Offset in TUs subfield is one octet in length and indicates the offset in TUs, rounded down to nearest TU, to the next TBTT of an AP from the immediately prior TBTT of the AP that transmits this element. The value 254 is used to indicate an offset of 254 TUs or higher. The value 255 is used to indicate an unknown offset value. This subfield is present only if the BSSID and TBTT Offset Presence Indicator is set to 1.

The Optional Subelements field contains zero or more subelements, each consisting of a 1-octet Subelement ID field, a 1-octet Length field, and a variable-length Data field, as shown in Figure 8-402. Any optional subelements are ordered by nondecreasing subelement ID.

10. MLME

10.1.4.3.7 Sending a response to probe request

Instructions to Editor: Add the the following text:

If the AP considers its operating channel too congested to accommodate STAs, the AP should include one AP Redirect Information element with Redirect Type indicating value 0 or 1 to Beacon or Probe Response frame. The decision when AP considers its operating channel too congested is implementation specific.

Motion: To authorize the Editor to incorporate the text changes proposed in contribution 11-12-1270-04-00ai-tgai-spec-text-proposal-for-ap-redirect to the draft TGai Specification Document.

Mover:

Seconder:

Results:

Yes: ____________;

No: _________________;

Abstain: _____________________

[Result of Motion]